Flags

There are special instructions designed to test and manipulate the flags. A flag or bit is set when it equals 1; it is cleared (or reset) when it equals 0. The CPU sets flags by turning on individual bits in the Flags register. There are two basic types of flags: control flags and status flags.

Control Flags

Individual bits can be set in the Flags register by the programmer to control CPU operation. These are the Direction, Interrupt, and Trap flags, abbreviated as DF, IF, and TF, respectively:

Flag Name

Typical Usage

Direction flag (DF)

affects block data transfer instructions such as MOVS, CMPS, and SCAS. The flag values are 1 = up and 0 = down. They can also be manipulated by the STD and CLD instructions.

Interrupt flag (IF)

specifies whether or not system interrupts can occur. Hardware devices such as the keyboard, disk drives, and the system clock timer signal interrupts. A program will sometimes briefly disable interrupts when performing a critical

operation that cannot be interrupted. The flag values are 1 = enabled, and 0 =

disabled; they are manipulated by the CLI and STI instructions.

Trap flag (TF)

determines whether or not the CPU is halted after each instruction. When this flag is set, a debugging program can let a programmer single step (trace) a program one instruction at a time. The flag can be set by the INT 3 instruction.

Status Flags

The Status flags reflect the outcomes of arithmetic and logical operations performed by the CPU. They are the Overflow, Sign, Zero, Auxiliary Carry, Parity, and Carry flags.

Flag Name

Typical Usage

Carry flag (CF)

set when the result of an unsigned arithmetic operation is too large to fit into the destination. For example, if the sum of 200 and 58 were stored in the 8-bit register AL, the result would overflow the register and the Carry flag would equal 1 (be set). The flag values are 1 = carry, 0 = no carry.

Overflow flag (OF)

set when the result of a signed arithmetic operation is too wide (too many bits) to fit into the destination. If the sum of -128 and -2 is added in an 8-bit AL register, the Overflow flag would be set. The Overflow flag values are 1 = overflow and 0 = no overflow.

Sign flag (SF)

set when the result of an arithmetic or logical operation generates a negative result. Because a negative number always has a 1 in the highest bit position, the Sign flag is always a copy of the destination's sign bit. The flag values are 1 = negative, 0 = positive

Zero flag (ZF)

set when the result of an arithmetic or logical operation generates a result of zero. The flag is used primarily by jump and loop instructions, to allow branching to a new location in a program based on the comparison of two values. The flag values are 1 = zero, 0 = not zero.

Auxiliary Carry (AC)

set when an operation causes a carry from bit 3 to bit 4 (or a borrow from bit 4 to bit 3) of an operand. Along with the Parity flag, it is less often used than other status flags. The flag values are 1 = carry, and 0 = no carry.

Parity flag (PF)

reflects the number of 1 bits in the result of an operation. If there is an even number of bits, the Parity is even. If there are an odd number of bits, the Parity is odd. This flag is used by the operating system to verify memory integrity and by communications software to verify the correct transmission of data.

Finding the Right Instruction

After you get the structure of a program worked out and written down, the next step is to

determine the instruction statements required to do each part of the program. Since the

examples in this book are based on the 8088 family of microprocessors, now is a good time to give you an overview of the instructions the 8086 has for you to use. First, however, is a hint about how to approach these instructions. You do not usually learn a new language by memorizing an entire dictionary of the language. A better way is to learn a few useful words and practice putting these words together in simple sentences. You can then learn more words as you need them to express more complex thoughts. Likewise, you should not try to memorize all the instructions for a microprocessor at once. As an introduction, however, the few pages here contain a list of all the 8086 instructions with a short explanation of each. Skim through the list and pick out a dozen or so instructions that seem useful and understandable. As a start, look for move, input, output, logical, and arithmetic instructions. Then look through the list again to see if you can write a simple program. You can use the instruction reference as you write programs. Here we simply list the 8086 instructions in functional groups with single-sentence descriptions so that you can see the types of instructions that are available to you. As you read through this section, do not expect to understand all the instructions. When you start writing programs, you will probably use this section to determine the type of instruction and the instruction reference to get the instruction details you need them. After you have written a few programs, you will remember most of the basic instruction types and will be able to simply look up an instruction in the instruction reference to get any additional details you need.

I. Data transfer instructions

PUSH              Copy specified word to top of stack.

POP                Copy word from top of stack to specific location.

PUSHA           (80186/80188 only) Copy all registers stack.

POPA             (80186/80l88 only) Copy words f stack to all registers.

XCHG             Exchange bytes or exchange words.

XLAT         Translate a byte in AL using a table in memory.

Simple input and output port transfer instructions:

IN                Copy a byte or word from specified port to accumulator.

OUT            Copy a byte or word from accumulator specified port.

Special address transfer instructions:

LEA            Load effective address of operand in specified register.

LDS            Load DS register and other specified register from memory.

LES            Load ES register and other specified register from memory.

Flag transfer instructions:

LAHF          Load (copy to) AH with the low byte the flag register.

SAHF         Store (copy) AH register to low byte of register.

PUSHF       Copy flag register to top of stack.

POPF         Copy word at top of stack to flag register

II. Arithmetic instructions

ADD          Add specified byte to byte or specified word to word.

ADC          Add byte + byte + carry flag or word + word + carry flag.

INC            Increment specified byte or specified by 1.

AAA          ASCII adjust after addition.

DAA          Decimal (BCD) adjust after addition.

SUB           Subtract byte from byte or word from word.

SBB           Subtract byte and carry flag from byte word and carry flag from word.

DEC           Decrement specified byte or specified word by l.

NEG           Negate – invert each bit of a specified byte or word and add 1 (form 2’s

complement).

CMP          Compare two specified bytes or two specified word

AAS          ASCII adjust after subtraction.

DAS          Decimal (BCD) adjust after subtraction.

Multiplication Instructions

MUL       Multiply unsigned byte by byte or unsigned word by word

IMUL      Multiply signed byte by byte or signed word by word

AAM       ASCII adjust after multiplication

Division Instructions

DIV         Divide unsigned word by byte or unsigned double word by word

IDIV        Divide signed word by byte or signed double word by word

AAD        ASCII adjust before division

CBW       Fill upper byte of word with copies of sign bit of lower byte

CWD       Fill upper word of double word with copies of sign bit of lower word

III. Logic and Bit manipulation instructions (AND, OR, XOR)

Logic instructions:

NOT       Invert each bit in a byte or word

AND       AND the content of a byte or a word with another byte or word

OR         OR the content of a byte or a word with another byte or word

XOR       Exclusive OR the content of a byte or a word with another byte or word

Shift instructions:

SHL/SAL     Shift bits of word or byte left, put zero(s) in LSB(s)

SHR            Shift bits of word or byte right, put zero(s) in MSB(s)

SAR            Shift bits of word or byte right, copy old MSB into new MSB

Rotate instructions:

ROL          Rotate bits of byte or word left, MSB to LSB and to CF

ROR         Rotate bits of byte or word right, LSB to MSB and to CF

RCL          Rotate bits of byte or word left, MSB to CF and CF to LSB

RCR         Rotate bits of byte or word right, LSB to CF and CF to MSB

IV. String instructions

A string is a series of bytes or a series of words in sequential memory locations. A string often consists of ASCII character codes. In the list, a ”/” is used to separate different mnemonics for the same instruction. Use the mnemonic which most clearly describes the function of the instruction in a specific application. A ”B” In a mnemonic is used to specifically indicate that a string of bytes is to be acted upon. A ”W” In the mnemonic is used to indicate that a string of words Is to be acted upon.

REP                                           An instruction prefix. Repeat following instruction until

CX=0

REPE/REPZ                              An instruction prefix. Repeat instruction until CX = 0 or

zero Flag ZF!=1

REPNE/REPNZ                         An instruction prefix. Repeat until CX = 0 or ZF = 1

MOVS/MOVSB/MOVSW         Move byte or word from one string to another

COMPS/COMPSB/COMPSW   Compare two string bytes or two string words

INS/INSB/INSW                        (80186/80188) Input string byte or word from port

OUTS/OUTSB/OUTSW            (80186/80l88) output string byte or word to port

SCAS/SCASB/SCASW             Scan a string. Compare a string byte with a byte in AL or

A string word with a word in AX

LODS/LODSB/LODSW             load string byte into AL or string word into AX

STOS/STOSB/STOSW            Store byte from AL or word from AX into string

V. Program execution transfer instructions

Instructions are used to tell the 8086 to start fetching instructions from some new address,

rather than continuing in sequence. Unconditional transfer instructions:

CALL     Call a procedure (subprogram), save return address on stack

RET        Return from procedure to calling program

JMP        Go to specified address to get next instruction

Conditional transfer instructions:

A ”/”is used to separate two mnemonics which represent the same instruction. Use the

mnemonic which most dearly describes the decision condition in a specific program. These instructions are often used after a compare instruction. The terms below and above refer to unsigned binary numbers. Above means larger in magnitude. The terms greater than or less than refer to signed binary numbers. Greater than means more positive.

JA/JNBE       Jump if above/Jump if not below or equal

JAE/JNB       Jump if above or equal/Jump if not below

JB/JNAE       Jump if below/Jump if not above or equal

JBE/JNA       Jump if below or equal/Jump if not above

JC                  Jump if carry flag CF 1

JE/JZ             Jump if equal/Jump if zero flag ZF = 1

JG/JNLE       Jump if greater/Jump if not less than or equal

JGE/JNL       Jump if greater than or equal Jump if not less than

JL/JNGE       Jump if less than/Jump if not greater than or equal

JLE/JNG       Jump if less than or equal/Jump if not greater than

JNC               Jump if no carry (CF = 0)

JNE/JNZ       Jump if not equal/lump if not ’ zero (ZF = 0)

JNO              Jump if no overflow (overflow flag OF = 0)

JNP/JPO       Jump if not parity/Jump if parity odd (PF = 0)

JNS               Jump if not sign (sign flag SF=0)

JO                 Jump if overflow flag OF=1

JP/JPE          Jump if parity/Jump if parity even (PF =1)

JS                 Jump if sign (SF = 1)

Iteration control instructions:

These instructions can be used to execute a series of instructions some number of times. Here mnemonics separated by a ”/” represent the same instruction. Use the one that best fits the specific application.

LOOP                      Loop through a sequence of instructions until CX= 0

LOOPE/LOOPZ     Loop through a sequence instructions while ZF= l and CX != 0

LOOPNE/LOOPNZ      Loop through a sequence instructions while ZF=0 and CX != 0

JCXZ                             Jump to specified address if CX=0

Interrupt instructions:

INT       Interrupt program execution call service procedure

INTO    Interrupt program execution OF =1

IRET    Return from interrupt se procedure to main program

High-level language interface instructions:

ENTER     (80l86/80188 only) Enter procedure

LEAVE     (80l86/80188 only) Leave procedure

BOUND    (80l86/80188 only) Check effective address within specified array bounds

VI. Processor control instructions

Flag set/clear instructions:

STC     Set carry flag CF to 1

CLC    Clear carry flag CF to 0

CMC   Complement the state of the carry flag CF

STD     Set direction flag DF to l (decrement string pointers)

CLD    Clear direction flag DF to 0

STI      Set interrupt enable flag to 1 (enable INTR input)

CLI      Clear interrupt enable flag to 0 (disable INTR input)

Execution control instructions:

HLT        Halt (do nothing) until interrupt or reset

WAIT     Wait (do nothing) until signal on the test pin is low

ESC        Escape to external coprocessor such as 8087 or 8089

LOCK     An instruction prefix. Prevents another processor from taking the bus while the

adjacent instruction executes

NOP        No action except fetch and decode

The assembly instruction format includes Label: which defines the program line, Opcode or mnemonic which describe the required operation, the operand which includes the data and the comment a typical instruction is shown down

Assembly instruction format