ADDRESSING MODES

Introduction

- Efficient software development for the microprocessor requires a complete familiarity with the addressing modes employed by each instruction.

Objectives

• Explain the operation of each data-addressing mode.

• Use the data-addressing modes to form assembly language statements.

• Explain the operation of each program memory-addressing mode.

• Use the program memory-addressing modes to form assembly and machine language statements

• Select the appropriate addressing mode to accomplish a given task.

• Detail the difference between addressing memory data using real mode and pro-tected mode operation.

• Describe sequence of events that place data onto the stack or remove data from the stack.

• Explain how a data structure is placed in memory and used with software.

3–1 DATA ADDRESSING MODES

• MOV instruction is a common and flexible instruction.

– provides a basis for explanation of data-addressing modes

• Figure 3–1 illustrates the MOV instruction and defines the direction of data flow.

• Source is to the right and destination the left, next to the opcode MOV.

– an opcode, or operation code, tells the microprocessor which operation to perform.

Register Addressing

• The most common form of data addressing.

– Once register names learned, easiest to apply.

• The microprocessor contains these 8-bit register names used with register addressing: AH, AL, BH, BL, CH, CL, DH, and DL.

• 16-bit register names: AX, BX, CX, DX, SP, BP, SI, and DI.

• In 80386 & above, extended 32-bit register names are: EAX, EBX, ECX, EDX, ESP, EBP, EDI, and ESI.

• 64-bit mode register names are: RAX, RBX, RCX, RDX, RSP, RBP, RDI, RSI, and R8 through R15.

• Important for instructions to use registers that are the same size.

– Never mix an 8-bit \with a 16-bit register, an 8-or a 16-bit register with a 32-bit register

– This is not allowed by the microprocessor and results in an error when assembled

• Figure 3–3 shows the operation of the MOV BX, CX instruction.

• The source register’s contents do not change.

– the destination register’s contents do change

• The contents of the destination register or destination memory location change for all instructions except the CMP and TEST instructions.

• The MOV BX, CX instruction does not affect  the leftmost 16 bits of register EBX.

Immediate Addressing

• Term immediate implies that data immediately follow the hexadecimal opcodein the memory.

– immediate data are constant data

– data transferred from a register or memory location are variable data

• Immediate addressing operates upon a byte or word of data.

• Figure 3–4 shows the operation of a MOV EAX, 13456H instruction.

• As with the MOV instruction illustrated in Figure 3–3, the source data overwrites the destination data.

• In symbolic assembly language, the symbol # precedes immediate data in some assemblers.

– MOV AX,#3456H instruction is an example • Most assemblers do not use the # symbol, but represent immediate data as in the MOV AX,3456H instruction.

– an older assembler used with some Hewlett-Packard logic development does, as may others – in this text, the # is not used for immediate data

• The symbolic assembler portrays immediate data in many ways.

• The letter H appends hexadecimal data.

• If hexadecimal data begin with a letter, the assembler requires the data start with a 0.

– to represent a hexadecimal F2, 0F2H is used in assembly language

• Decimal data are represented as is and require no special codes or adjustments.

– an example is the 100 decimal in the MOV AL,100 instruction

• An ASCII-coded character or characters may be depicted in the immediate form if the ASCII data are enclosed in apostrophes.

– be careful to use the apostrophe (‘) for ASCII data and not the single quotation mark (‘)

• Binary data are represented if the binary number is followed by the letter B.

– in some assemblers, the letter Y

• Each statement in an assembly language program consists of four parts or fields.

• The leftmost field is called the label.

– used to store a symbolic name for the memory location it represents

• All labels must begin with a letter or one of the following special characters: @, $, -, or ?.

– a label may any length from 1 to 35 characters

• The label appears in a program to identify the name of a memory location for storing data and for other purposes.

• The next field to the right is the opcode field.

– designed to hold the instruction, or opcode

– the MOV part of the move data instruction is an example of an opcode

• Right of the opcode field is the operand field.

– contains information used by the opcode

– the MOV AL,BL instruction has the opcode MOV and operands AL and BL

• The comment field, the final field, contains a comment about the instruction(s).

– comments always begin with a semicolon (;)