What Are Interrupts?

Interrupts are very important to the operation of any computer. Interrupts give the processor the ability to respond quickly to its peripherals (such as the keyboard and the hard disk) and to the outside world in general. Without interrupts, a processor would be unable to service more than one task efficiently and reliably. The importance of the interrupt is illustrated when comparing an interrupt to a doorbell. If your door did not have a doorbell, you would have to periodically go to the door to see if anyone happened to be there at that time. Of course, that would be very inefficient. With a doorbell, you only need to go to the door when the doorbell rings, and you are then confident that someone is there waiting. Likewise, it is not efficient for the processor to continually check whether any of its peripherals require attention at a given time. An interrupt is a doorbell to the processor to signal that some device needs service.

During normal execution of a program, instructions are read from memory and executed sequentially by the processor. The processor uses a special register called the instruction pointer to keep track of the next instruction to be executed. A set of general-purpose registers are used for manipulation and temporary storage of any data used by the program.

An interrupt is a special input to the processor. When the processor is signalled that an interrupt has occurred, the processor finishes the instruction currently being executed and saves the instruction pointer and a status word to the stack. The stack is a special block of memory used to keep track of information during function calls and interrupts. The processor uses a special register called the stack pointer to keep track of the location of the top of the stack, which is where the last item was added to the stack.

After the processor saves this information to the stack, the processor branches to a special routine called an interrupt service routine or interrupt handler. If needed, this routine can determine the source of the interrupt and then service the device that requested the interrupt. This process may involve reading a character from the keyboard and updating the monitor, or moving data from an adapter board into computer memory. After handling the interrupt, the last instruction executed in the interrupt service routine is an interrupt return instruction. This instruction causes the processor to restore the instruction pointer and status register with the values that were saved to the stack at the time of the interrupt and to resume execution.