Lec. (5)

Microprocessors

2nd  stage

2014

The Intel x86 Architecture

22. The Intel x86 Architecture

22.1. Introduction

In 1978 Intel had a 16-bit successor to the 8080, the 8086/8088. The 8088 was internally identical to the 8086 with the exception of an 8-bit external data bus, rather than 16-bit. Certainly this would limit the performance of the device. The growth of the PC industry essentially tracked Intel’s continued development and ongoing refinement of the x86 architecture.

Intel introduced microprocessors way back in 1969. Their first 4-bit microprocessor was the 4004. This was followed by the 8080 and 8085. The work on these early microprocessors led to the development of the Intel architecture (IA). The first processor in the IA family was the 8086 processor. It has a 20-bit address bus and a 16-bit data bus. Thus, it could address up to 1 MB of memory.

The 8088 is a less expensive version of the 8086 processor. The cost reduction is obtained by using an 8-bit data bus. Except for this difference, the 8088 is identical to the 8086 processor. Intel introduced segmentation with these processors. These processors can address up to four segments of 64 KB each. This IA segmentation is referred to as the real mode segmentation.

The 80186 is a faster version of the 8086. It also has a 20-bit address bus and 16-bit data bus, but has an improved instruction set. The 80186 was never widely used in computer systems. The real successor to the 8086 is the 80286, which was introduced in 1982. It has a 24-bit address bus, which implies 16 MB of memory address space. The data bus is still 16 bits wide, but the 80286 has some memory protection capabilities.

Intel introduced its first 32-bit CPU—the 80386—in 1985. It has a 32-bit data bus and 32-bit address bus. The memory address space has grown substantially (from 16 MB address space to 4 GB). This processor introduced paging into the IA architecture. It also allowed definition segments as large as 4 GB. Like the 80286, it can run all the programs written for 8086 and 8088 processors.

The Intel 80486 was introduced in 1989. This is an improved version of the 80386. While maintaining the same address and data buses, it combined the coprocessor functions for performing floating-point arithmetic. The 80486 processor has added more parallel execution capability to instruction decode and execution units to achieve a scalar execution rate of one instruction per clock. It has an 8 KB onchip L1 cache. Furthermore, support for the L2 cache and multiprocessing has been added.

The latest in the family is the Pentium series. It is not named 80586 because Intel found belatedly that numbers couldn’t be trademarked! The first Pentium was introduced in 1993. The Pentium is similar to the 80486 but uses a 64-bit wide data bus. Internally, it has 128- and 256-bit wide datapaths to speed internal data transfers. However, the Pentium instruction set supports 32-bit operands like the 80486. The Pentium has added a second execution pipeline to achieve superscalar performance by having the capability to execute two instructions per clock. It has also doubled the onchip L1 cache, with 8 KB for data and another 8 KB for the instructions.

The Pentium Pro processor has a three-way superscalar architecture. That is, it can execute three instructions per CPU clock. The address bus has been expanded to 36 bits, which gives it an address space of 64 GB. In addition to the L1 caches provided by the Pentium, the Pentium Pro has a 256 KB L2 cache in the same package as the CPU.

The Pentium II processor has added multimedia (MMX) instructions to the Pentium Pro architecture. It has expanded the L1 data and instruction caches to 16 KB each. It has also added more comprehensive power management features including Sleep and Deep Sleep modes to conserve power during idle times. Table 7.1 summarizes the key characteristics of the IA family of processors.

Intel’s 64-bit Itanium processor is targeted for server applications. For these applications, the Pentium’s memory address space is not adequate. The Itanium uses a 64-bit address bus to provide substantially large address space. Its data bus is 128 bits wide. In a major departure, Intel has moved from the CISC designs of Pentium processors to RISC orientation for their Itanium processors.

Table 7.1 Key characteristics of the IA family of processors (“Year” refers to the year of introduction; “Frequency” refers to the frequency at introduction)

22.2 The Architecture of the 8086 CPU

Figure 24: Simplified block diagram of the Intel 8086 CPU

Figure 24 is a simplified block diagram of the 8086 CPU. The 8086/8088 microprocessor consists of two internal units: the execution unit (EU), which executes the instructions, and the bus interface unit (BIU), which fetches instructions, reads operands and writes results.

The 8086 is strictly organized as two separate and autonomous functional blocks. The execution unit, or EU, handles the arithmetic and logical operations on the data and has a 6 byte first-in, first-out (FIFO) instruction queue (4 bytes on the 8088). The segment registers of the BIU are responsible for access instructions and operands from memory. The main linkage between the two functional blocks is the instruction queue, with the BIU looking ahead of the current instruction being executed in order to keep the queue filled with instructions for the EU to decode and operate on.

The symbol on the BIU side that looks like a carpenter’s saw horse is called a multiplexer, or MUX, and its function is to combine the address and data information into a single, 20-bit external bus. The multiplexed (or shared) bus allows the 8086 CPU to have only 40 pins on the package, while the 68000 has 64 pins, primarily due to the extra pins required for the 23 address and 16 data pins. The multiplexed bus requires that systems using the 8086 must have external logic on the board to latch the address into holding registers during the first part of the bus cycle in order to have a stable address to present to memory during the second half of the cycle.

The 20-bit wide address bus gives the 8086 1 MByte address range. The BIU manages the extra bus cycle required to fetch both bytes of the word. The programmer’s model of the 8086 is shown in Figure 25.

Figure 25: Programmer’s model of the 8086 register set.

It is manufactured using high-performance metal-oxide-semiconductor (HPMOS) technology and it contains approximately 29,000 transistors which enclosed in a 40-pin package (fig. 26)

GND

AD14

AD13

AD12

AD11

AD10

AD9

AD8

AD7

AD6

AD5

AD4

AD3

AD2

AD1

AD0

NM1

INTR

CLK

GND

VCC

AD15

A16/S3

A17/S4

A18/S5

A19/S6

BHE/S7

MN/MX

RD

HOLD

HLDA

WR

M/IO

DT/R

DEN

ALE

INTA

TEST

REDY

RESET

(RQ/GT0)

(RQ/GT1)

(LOCK)

(S2)

(S1)

(S0)

(OS0)

(OS1)

Figure 26: 8086 pins package

AD15-AD0 Multiplexed address(ALE=1)/data bus(ALE=0).

A19/S6-A16/S3 (multiplexed) High order 4 bits of the 20-bit address OR status bits S6-S3.

M/IO Indicates if address is a Memory or IO address.

RD When 0, data bus is driven by memory or an I/O device.

WR Microprocessor is driving data bus to memory or an I/O device.

When 0, data bus contains valid data.

ALE (Address latch enable) When 1, address data bus contains a memory or I/O address.

DT/R (Data Transmit/Receive) Data bus is transmitting/receiving data.

DEN (Data bus Enable) Activates external data bus buffers.

S7, S6, S5, S4, S3, S2, S1, S0

S7: Logic 1, S6: Logic 0.

S5: Indicates condition of IF flag bits.

S4-S3: Indicate which segment is accessed during current bus cycle:

S2, S1, S0 : Indicate function of current bus

INTR When 1 and IF=1, microprocessor prepares to service interrupt. INTA becomes active after current instruction completes.

INTA Interrupt Acknowledge generated by the microprocessor in response to INTR. Causes the interrupt vector to be put onto the data bus.

NMI Non-maskable interrupt. Similar to INTR except IF flag bit is not consulted and interrupt is vector 2.

CLK Clock input must have a duty cycle

VCC/GND Power supply (5V) and GND (0V).

MN/ MX Select minimum (5V) or maximum mode (0V) of operation.

BHE Bus High Enable. Enables the most significant data bus bits (D 15 -D 8 ) during a read or write operation.

READY Used to insert wait states (controlled by memory and IO for reads/writes) into the microprocessor.

RESET Microprocessor resets if this pin is held high for 4 clock periods.

TEST An input that is tested by the WAIT instruction.

HOLD Requests a direct memory access (DMA). When 1, microprocessor stops and places address, data and control bus in high-impedance state.

HLDA (Hold Acknowledge) Indicates that the microprocessor has entered the hold state.

RO/GT1 and RO/GT0 Request/grant pins request/grant direct memory accesses (DMA) during maximum mode operation.

LOCK Lock output is used to lock peripherals off the system.

QS1 and QS0 The queue status bits show status of internal instruction queue.

22.2.1  Minimum & Maximum Mode Systems

In the minimum mode, all the control signals for the memory and I/O are generated by the microprocessor. This reduces cost but provides low functionality. In the maximum mode, a separate IC called the 8288 Bus Controller is used to provide control signals for memory and I/O operations. The S0-S2 pins are decoded by the 8288 to provide control signals as ALE, DEN, DT/-R, -IOWC (I/O Write Command), -IORC (I/O Read Command),          -MWTC (Memory Write Control), -MRDC (Memory Read Control), -INTA etc.

The minimum mode signals can be divided into the following basic groups:

address\data bus

status

control

interrupt

DMA