Lec 8

Microprocessors

2nd  stage

25-12-2014

The Bus

### 12 The Bus

The CPU communicates with the other components via a bus. A bus is a set of wires that acts as a shared but common data path to connect multiple subsystems within the system. It consists of multiple lines, allowing the parallel movement of bits. Buses are low cost but very versatile, and they make it easy to connect new devices to each other and to the system. At any one time, only one device (be it a register, the ALU, memory, or some other component) may use the bus. However, this sharing often results in a communications bottleneck. The speed of the bus is affected by its length as well as by the number of devices sharing it. Quite often, devices are divided into master and slave categories, where a master device is one that initiates actions and a slave is one that responds to requests by a master.

Figure 13 shows a typical bus consisting of data lines, address lines, control lines, and power lines. Often the lines of a bus dedicated to moving data are called the data bus. These data lines contain the actual information that must be moved from one location to another. Control lines indicate which device has permission to use the bus and for what purpose (reading or writing from memory or from an I/O device, for example). Control lines also transfer acknowledgments for bus requests, interrupts, and clock synchronization signals. Address lines indicate the location n (in memory, for example) that the data should be either read from or written to. The power lines provide the electrical power necessary. Typical bus transactions include sending an address (for a read or write), transferring data from memory to a register (a memory read), and transferring data to the memory from a register (a memory write). In addition, buses are used for I/O reads and writes from peripheral devices. Each type of transfer occurs within a bus cycle, the time between two ticks of the bus clock.

Figure 13: The Components of a Typical Bus

Due to the different types of information buses transport and the various devices that use the buses, buses themselves have been divided into different types. Processor-memory buses are short, high-speed buses that are closely matched to the memory system on the machine to maximize the bandwidth (transfer of data) and are usually very design specific. I/O buses are typically longer than processor-memory buses and allow for many types of devices with varying bandwidths. These buses are compatible with many different architectures. A backplane bus (Figure 14) is actually built into the chassis of the machine and connects the processor, the I/O devices, and the memory (so all devices share one bus). Many computers have a hierarchy of buses, so it is not uncommon to have two buses (for example a processor-memory bus and an I/O bus) or more in the same system. High-performance systems often use all three types of buses.

Figure 14: A Backplane Bus

Personal computers have their own terminology when it comes to buses. PCs have an internal bus (called the system bus) that connects the CPU, memory, and all other internal components. External buses (sometimes referred to as expansion buses) connect external devices, peripherals, expansion slots, and I/O ports to the rest of the computer. Most PCs also have local buses, data buses that connect a peripheral device directly to the CPU. These are very high-speed buses and can be used to connect only a limited number of similar devices. Expansion buses are slower but allow for more generic connectivity.

Buses are physically little more than bunches of wires, but they have specific standards for connectors, timing, and signaling specifications and exact protocols for usage. Synchronous buses are clocked, and things happen only at the clock ticks (a sequence of events is controlled by the clock). Every device is synchronized by the rate at which the clock ticks, or the clock rate. The bus cycle time mentioned earlier is the reciprocal of the bus clock rate. For example, if the bus clock rate is 133MHz, then the length of the bus cycle is 1/133,000,000 or 7.52ns. Because the clock controls the transactions, any clock skew (drift in the clock) has the potential to cause problems, implying that the bus must be kept as short as possible so the clock drift cannot get overly large. In addition, the bus cycle time must not be shorter than the length of time it takes information to traverse the bus. The length of the bus, therefore, imposes restrictions on both the bus clock rate and the bus cycle time.

With asynchronous buses, control lines coordinate the operations and a complex handshaking protocol must be used to enforce timing. To read a word of data from memory, for example, the protocol would require steps similar to the following:

- ReqREAD: This bus control line is activated and the data memory address is put on the appropriate bus lines at the same time.

- ReadyDATA: This control line is asserted when the memory system has put the required data on the data lines for the bus.

- ACK: This control line is used to indicate that the ReqREAD or the ReadyDATA has been acknowledged.

Using a protocol instead of the clock to coordinate transactions means that asynchronous buses scale better with technology and can support a wider variety of devices.

To use a bus, a device must reserve it, because only one device can use the bus at a time. As mentioned previously, bus masters are devices that are allowed to initiate transfer of information (control bus) whereas bus slaves are modules that are activated by a master and respond to requests to read and write data (so only masters can reserve the bus). Both follow a communications protocol to use the bus, working within very specific timing requirements. In a very simple system the processor is the only device allowed to become a bus master. This is good in terms of avoiding chaos, but bad because the processor now is involved in every transaction that uses the bus.