Turbulence

Turbulent flow occurs because, in general, rapid fluid motion is unstable.

Suppose fluid is initially flowing through a pipe in the orderly manner, illustrated

by the pathlines . Eventually, something will disturb

the motion, e.g. an obstruction in or a knock on the pipe. Consequently,

elemental volumes, described here as ‘elements’, will alter course and, if

they are moving rapidly enough, fluid friction will not be able to restore

them to their original paths. Moreover the disturbed elements disturb other

elements of fluid from their original path, and soon the entire flow is in the

semi-chaotic state called turbulence, illustrated in Figure 2.5(b).

It is the ratio of fluid momentum (arising from ‘inertia forces’) to viscous

friction which determines whether the flow becomes smooth (i.e. laminar)

or turbulent. This ratio is usually characterised by the non-dimensional

Reynolds number

R_ = uX

Here u is the mean speed of the flow, X is a nominated characteristic length

of the system (in this case the diameter of the pipe) and  (/ is the kinematic viscosity of the fluid. The value of _is important for

characterising types of fluid flow, e.g. turbulence. For instance in pipes, flow

will be usually turbulent if _ is larger than about 2300 ___2300_. Criteria

for laminar or turbulent flow in heat transfer are discussed in Chapter 3.

In turbulent flow, random local fluctuations of velocity in three dimensions

are imposed on the mean flow. Thus small elements of fluid moving

along the pipe also move rapidly inwards and outwards across the pipe,

. Since fluid does not slip at the pipe surface

, the mean speed near the surface is smaller than the average

and the mean speed near the centre of the pipe is correspondingly larger.

Therefore the effect of the sideways motions of the fluid elements is to carry

fluid of larger velocity outwards, and fluid of smaller velocity inwards. This

transfer of momentum by elements of fluid is much larger than the corresponding

transfer by molecular motions because

an element of fluid may move significantly across the pipe in a single jump.

In this case with water as the fluid, the mean free path of a molecule in the

liquid is of the order of nanometres.

This transfer of momentum from the fluid to the walls constitutes a

sizeable friction force opposing the motion of the fluid. Thus the presence of

turbulence increases friction as compared with laminar flow; it is important

to appreciate this characteristic of turbulent flow as, for instance,