.

To understand such systems, we must start with the basic laws of mechanics

as they apply to fluids, notably the laws of conservation of mass, energy

and momentum. The term fluid includes both liquids and gases, which,

unlike solids, do not remain in equilibrium when subjected to shearing

forces. The hydrodynamic distinction between liquids and gases is that

gases are easily compressed, whereas liquids have volumes varying only

slightly with temperature and pressure. Gaseous volumes vary directly with

temperature and inversely with pressure, approximately as the perfect-gas

law _pV = nRT_. Nevertheless, for air, flowing at speeds <100ms−1 and

not subject to large imposed variations in pressure or temperature, density

change is negligible; this is the situation for the renewable energy systems

analysed quantitatively in this book. It does not apply to the analysis

of gas turbines, for which specialist texts should be consulted. Therefore,

throughout this text, moving air is considered to have the fluid dynamics

of an incompressible fluid. This considerably simplifies the analysis of most

renewable energy systems.

Many important fluid flows are also steady, i.e. the particular type of flow

pattern at a location does not vary with time. So it is useful to picture a set

of lines, called streamlines, parallel with the velocity vectors at each point.

A further distinction is between laminar and turbulent flow

For example, watch the smoke rising from a smouldering taper in still air.

Near the taper, the smoke rises in an orderly, laminar, stream, with the

paths of neighbouring smoke particles parallel. Further from the taper, theflow becomes chaotic, turbulent, with individual smoke particles intermingling

in three dimensions. Turbulent flow approximates to a steady mean

flow, subject to internal friction caused by the velocity fluctuations. However,

even in turbulence, the airflow remains within well-defined (though

imaginary) streamtubes, as bounded by streamlines.