| Microprocessors | lecture 7 : Programming with 8086 Microprocessor |
| --- | --- |

Calling procedure/subroutine

Procname PROC FAR

……………………………..

……………………………..

Procname ENDP

Here the code segment consists only one procedure. The FAR operand in this case informs the assembler and linker that the defined procedure name is the entry point for program execution, whereas the ENDP directive defines the end of the procedure. A code segment however, may contain any number of procedures, each distinguished by its own PROC and ENDP directives.

A called procedure is a section of code that performs a clearly defined task known as subroutine which provides following benefits.

- Reduces the amount of code because a common procedure can be called from any number of places in the code segment.

- Encourage better program organization.

- Facilitates debugging of a program because defects can be more clearly isolated.

- Helps in the ongoing maintenance of programs because procedures are readily identified for modification.

A CALL to a procedure within the same code segment is NEAR CALL<. A FAR CALL calls a procedure labeled FAR, possibly in another code segment.

Microprocessors	lecture 7 : Programming with 8086 Microprocessor

DISPLAY PROC NEAR

MOV AH, 09H

MOV DX, OFFSET STR

INT 21H

RET

DISPLAY ENDP

- To display number contained in [BX] DISPLAY PROC NEAR

MOV Dl, [BX] ADD Dl, 30 MOV AH, 02H INT 21H

RET DISPLAY ENDP

INT 10H Video service:

<Video –modes>

| Text mode | Row column |  | Color | No.of | Resolution |  | colors |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pages |  |  |  |
| 00 | 25 | 40 | Color | 8 | 360 | 400 | 16 colors |
| 01 | 25 | 40 | Color | 8 | 360 | 400 | 16 colors |
| 02 | 25 | 80 | Color | 4 | 720 | 400 | 16 colors |
| 03(by | 25 | 80 | color | 4 | 720 | 400 | 16 colors |
| default) |  |  |  |  |  |  |  |
| 07 | 25 | 80 | Mono-hrome | 0 | 720 | 400 | 16 colors |

Microprocessors	lecture 7 : Programming with 8086 Microprocessor

| Graphic mode | Color | Pages | Resolution |  | No of colors |
| --- | --- | --- | --- | --- | --- |
| 04 | Color | 8 | 320 | 200 | 4 |
| 05 | Color | 8 | 320 | 200 | 4 |
| 06 | Color | 8 | 640 | 200 | 2 |
| 0D | Color | 8 | 320 | 200 | 16 |
| 0E | Color | 4 | 640 | 200 | 16 |
| 0F | Mono chrome | 2 | 640 | 350 | 1 |
| 10 | Color | 2 | 640 | 350 | 16 |
| 11 | Color | 1 | 640 | 480 | 2 |
| 12 | Color | 1 | 640 | 480 | 16 |
| 13 | Color | 1 | 320 | 200 | 256 |

| Attribute |  |  |
| --- | --- | --- |
|  | Background | Foreground |
| Attribute: | BL R G B | I R G B |
| Bit number: | 7 6 5 4 | 3 2 1 0 |

I – Intensity, BL - Blink

| Color | Hex |
| --- | --- |
|  | Value |
| Black | 0 |
| Blue | 1 |
| Green | 2 |
| Cyan | 3 |
| Red | 4 |
| Magnet | 5 |
| Brown | 6 |
| White | 7 |
| Gray | 8 |
| Light Blue | 9 |
| Light Green | A |
| Light cyan | B |
| Light red | C |
| Light magenta | D |
| Yellow | E |
| Bright white | F |

Microprocessors	lecture 7 : Programming with 8086 Microprocessor

TITLE sorting the numbers – descending order

DOSSEG

.MODEL SMALL

.STACK 100H

.CODE MAIN PROC FAR

|  | MOV AX, @ DATA |
| --- | --- |
|  | MOV DS, AX |
|  | MOV DX, 4H |
| DOPASS: | MOV CX, 4H |
|  | MOV SI, 00H |
| CHECK: | MOV AL, ARR [SI] |
|  | CMP ARP [SI+1] , AL |
|  | JC NOSWAP |
|  | MOV BL, ARR [SI + 1] |
|  | MOV ARR[SI +1] , AL |
|  | MOV ARR [SI], BL |
| NOSWAP: | INC SI |
|  | LOOP CHECK |
|  | DEC DX |
|  | JNZ DOPASS |
|  | MOV AX, 4C00H |
|  | INT 21H |
|  | MAIN ENDP |

.DATA

ARR DB 8,2,9,4,7 END MAIN

Note: Display if numbers are with 1 digit

MOV CX, 05H

MOV SI, 00H

- MOV DL, ARR[SI] ADD DL, 30H MOV AH, 02H INT 21H

MOV DL,’ ’

MOV AH, 02H INT 21H

INC SI LOOP L

MOV AX, 4C00H

INT 21H

MAIN ENDP

Microprocessors	lecture 7 : Programming with 8086 Microprocessor

TITLE addition of 100 natural even numbers

.MODEL SMALL

.STACK 100H

.DATA

|  | TEN DW 10 |  |
| --- | --- | --- |
| .CODE |  |  |
| MAIN | PROC FAR |  |
|  | MOV AX, @ DATA |  |
|  | MOV DS, AX |  |
|  | MOV CX, 63H |  |
|  | MOV AX, 02H |  |
|  | MOV DX, 04H |  |
| L1: | ADD AX, DX |  |
|  | ADD DX, 02H |  |
|  | LOOP L1 |  |
| L2: | MOV DX, 0000H |  |
|  | DIV TEN | ; DX: AX /10 |
|  | INC CX |  |
|  | ADD DX, 30H | ; remainder |
|  | PUSH DX |  |
|  | CMP AX, 00H | ; quotient |
|  | JE L3 |  |
|  | JMP L2 |  |
| L3: | POP DX |  |
|  | MOV AH, 02H |  |
|  | INT 21H |  |

LOOP L3

MOV AX, 4C00H

INT 21H

MAIN ENDP

END MAIN

TITLE to display string at (10,40) with green background and red foreground dosseg

.Model small

.Stack 100H

.Code MAIN PROC FAR

MOV AX, @ DATA MOV DX, AX

MOV SI, OFFSET VAR1

L2:	MOV AH, 02H ; Set cursor position MOV DH, ROW

MOV DL, COL

| Microprocessors | lecture 7: Programming with 8086 Microprocessor |
| --- | --- |
| INT 10H |  |
| MOV AL, [SI] |  |
| CMP AL, ‘$’ |  |
| JE L1 |  |
| MOV AH, 09H |  |
| MOV DH, ROW |  |
| MOV DL, COL |  |
| MOV BL, 24H | ;background & foreground |
| MOV BH, 00h | ; page |
| MOV CX, 01H | ; no. of repeated characters |
| INT 10H |  |
| INC SI |  |

INC COL

JMP L2

L1:	MOV AX, 4C00H

INT 21H MAIN ENDP

.DATA ROW DB 10 COL DB 40

VAR1 DB “video model”, ‘$’

END MAIN

TITLE TO GENERATE MULTIPLICATION TABLE

.MODEL SMALL

.STACK 32

.DATA

NUM1 DB 5

NUM2 DB 1

TAB DB 10 DUP (?)

.CODE MAIN PROC FAR

MOV AX, @ DATA MOV DS, AX MOV BX, 0

MOV CX, 10

L1:	MOV AL, NUM1

MUL NUM2 MOV TAB [BX], AL INC BX

INC NUM2

LOOP L1

MOV AX, 4C00H

Microprocessors	lecture 7: Programming with 8086 Microprocessor

INT 21H

MAIN ENDP

END MAIN

TITLE to add 10 sixteen bit Numbers in memory table

| .MODEL SMALL |  |  |  |
| --- | --- | --- | --- |
| .STACK 32 |  |  |  |
| .DATA |  |  |  |
| NUM DW DUP (2) |  |  |  |
| NUM DW DUP (3) |  |  |  |
| SUMH DW 0 |  |  |  |
| SUML DW 0 |  |  |  |
|  |  | Note: To access the data of the |  |
| .CODE |  |  |  |
|  |  | memory i.e. table. |  |
|  | MAIN PROC FAR |  |  |
|  |  | We use e.g. NUM[BX] |  |
|  | MOV AX, @ DATA |  |  |
|  |  | Increasing the BX register by 2 |  |
|  | MOV DS, AX |  |  |
|  | MOV CX, 10 |  |  |
|  | MOV AX, 0 |  |  |
|  | MOV BX, 0 |  |  |
| L1: | ADD AX, NUM [BX] |  |  |
|  | MOV SUML, AX |  |  |
|  | JNC L2 |  |  |
|  | INC SUMH |  |  |
| L2: | ADD BX, 2 |  |  |
|  | LOOP L1 |  |  |
|  | MOV AX, 4C00H |  |  |
|  | INT 21H |  |  |
| MAIN ENDP |  |  |  |
| END MAIN |  |  |  |
| SUBROUTINE TO CLEAR THE SCREEN |  |  |  |
| SCR_CLEAR PROC NEAR |  |  |  |
| MOV AX, 0600H | ; Request scroll |  |  |
| MOV BH, 61H ; blue on brown for attribute on pixel(generally (07H) white on black |  |  |  |
| MOV CX, 0000 | ; Full screen |  |  |
| MOV DX, 184FH |  |  |  |

INT 10H

RET

SCR_CLEAR ENDP

AH-06h: Scroll upward of lines in a specified area of the screen.

AL- 00H caused entire screen to scroll up, effectively clearing it. Setting a nonzero value in AL causes the number of lines to scroll up.