The Computer Level Hierarchy

If a machine is to be capable of solving a wide range of problems, it must be able to execute programs written in different languages, from FORTRAN and C to Lisp and Prolog.

Programming experience teaches us that when a problem is large, we should break it down and use a "divide and conquer" approach. In programming, we divide a problem into modules and then design each module separately. Each module performs a specific task and modules need only know how to interface with other modules to make use of them.

Computer system organization can be approached in a similar manner. Through the principle of abstraction, we can imagine the machine to be built from a hierarchy of levels, in which each level has a specific function and exists as a distinct hypothetical machine. We call the hypothetical computer at each level a virtual machine. Each level's virtual machine executes its own particular set of instructions, calling upon machines at lower levels to carry out the tasks when necessary. By studying computer organization, you will see the rationale behind the hierarchy's partitioning, as well as how these layers are implemented and interface with each other. Figure 2 shows the commonly accepted layers representing the abstract virtual machines.

Figure 2:   The Abstract Levels of Modern Computing Systems

Level 6, the User Level, is composed of applications and is the level with which everyone is most familiar. At this level, we run programs such as word processors, graphics packages, or games. The lower levels are nearly invisible from the User Level.

Level 5, the High-Level Language Level, consists of languages such as C, C++, FORTRAN, Lisp, Pascal, and Prolog. These languages must be translated (using either a compiler or an interpreter) to a language the machine can understand. Compiled languages are translated into assembly language and then assembled into machine code. (They are translated to the next lower level.) The user at this level sees very little of the lower levels. Even though a programmer must know about data types and the instructions available for those types, she need not know about how those types are actually implemented.

Level 4, the Assembly Language Level, encompasses some type of assembly language. As previously mentioned, compiled higher-level languages are first translated to assembly, which is then directly translated to machine language. This is a one-to-one translation, meaning that one assembly language instruction is translated to exactly one machine language instruction. By having separate levels, we reduce the semantic gap between a high-level language, such as C++, and the actual machine language (which consists of 0s and 1s).

Level 3, the System Software Level, deals with operating system instructions. This level is responsible for multiprogramming, protecting memory, synchronizing processes, and various other important functions. Often, instructions translated from assembly language to machine language are passed through this level unmodified.

Level 2, the Instruction Set Architecture (ISA), or Machine Level, consists of the machine language recognized by the particular architecture of the computer system. Programs written in a computer's true machine language on a hardwired computer can be executed directly by the electronic circuits without any interpreters, translators, or compilers.

Level 1, the Control Level, is where a control unit makes sure that instructions are decoded and executed properly and that data is moved where and when it should be. The control unit interprets the machine instructions passed to it, one at a time, from the level above, causing the required actions to take place.

Control units can be designed in one of two ways: They can be hardwired or they can be microprogrammed. In hardwired control units, control signals emanate from blocks of digital logic components. These signals direct all of the data and instruction traffic to appropriate parts of the system. Hardwired control units are typically very fast because they are actually physical components. However, once implemented, they are very difficult to modify for the same reason.

The other option for control is to implement instructions using a microprogram. A microprogram is a program written in a low-level language that is implemented directly by the hardware. Machine instructions produced in Level 2 are fed into this microprogram, which then interprets the instructions by activating hardware suited to execute the original instruction. One machine-level instruction is often translated into several microcode instructions. This is not the one-to-one correlation that exists between assembly language and machine language. Microprograms are popular because they can be modified relatively easily. The disadvantage of microprogramming is, of course, that the additional layer of translation typically results in slower instruction execution.

Level 0, the Digital Logic Level, is where we find the physical components of the computer system: the gates and wires. These are the fundamental building blocks, the implementations of the mathematical logic that are common to all computer systems.

x86 Architecture Details

In this section, we focus on the basic architectural features of the x86 processor family, which includes both Intel IA-32 and 32-bit AMD processors.

Modes of Operation

x86 processors have three primary modes of operation: protected mode, real-address mode, and system management mode. A sub-mode, named virtual-8086, is a special case of protected mode. Here are short descriptions of each:

Protected Mode

Protected mode is the native state of the processor, in which all instructions and features are available. Programs are given separate memory areas named segments, and the processor prevents programs from referencing memory outside their assigned segments.

Virtual-8086 Mode While in protected mode, the processor can directly execute real-address mode software such as MS-DOS programs in a safe multitasking environment. In other words, if an MS-DOS program crashes or attempts to write data into the system memory area, it will not affect other programs running at the same time. Windows XP can execute multiple separate virtual-8086 sessions at the same time.

Real-Address Mode

Real-address mode implements the programming environment of the Intel 8086 processor with a few extra features, such as the ability to switch into other modes.

This mode is available in Windows 98, and can be used to run an MS-DOS program that requires direct access to system memory and hardware devices. Programs running in real-address mode can cause the operating system to crash (stop responding to commands).

System Management Mode

System Management mode (SMM) provides an operating system with a mechanism for implementing functions such as power management and system security. These functions are usually implemented by computer manufacturers who customize the processor for a particular system setup.

Address Space

In 32-bit protected mode, a task or program can address a linear address space of up to 4 GBytes. Beginning with the P6 processor, a technique called Extended Physical Addressing allows a total of 64 GBytes of physical memory to be addressed. Real-address mode programs, on the other hand, can only address a range of 1 MByte. If the processor is in protected mode and running multiple programs in virtual-8086 mode, each program has its own 1-MByte memory area.

Basic Program Execution Registers

Registers are high-speed storage locations directly inside the CPU, designed to be accessed at much higher speed than conventional memory. When a processing loop is optimized for speed, for example, loop counters are held in registers rather than variables. Figure 3 shows the basic program execution registers. There are eight general-purpose registers, six segment registers, a processor status flags register (EFLAGS), and an instruction pointer (EIP).

Figure 3 : The basic program execution registers.

General-Purpose Registers

The general-purpose registers are primarily used for arithmetic and data movement. As shown in Figure 4, the lower 16 bits of the EAX register can be referenced by the name AX.

Figure 4: general-purpose registers.

Portions of some registers can be addressed as 8-bit values. For example, the AX register, has an8-bit upper half named AH and an 8-bit lower half named AL. The same overlapping relationship exists for the EAX, EBX, ECX, and EDX registers:

The remaining general-purpose registers can only be accessed using 32-bit or 16-bit names, as shown in the following table:

Specialized Uses Some general-purpose registers have specialized uses:

•EAX is automatically used by multiplication and division instructions. It is often called the extended accumulator register.

•The CPU automatically uses ECX as a loop counter.

•ESP addresses data on the stack (a system memory structure). It is rarely used for ordinary arithmetic or data transfer. It is often called the extended stack pointer register.

•ESI and EDI are used by high-speed memory transfer instructions. They are sometimes called the extended source index and extended destination index registers.

•EBP is used by high-level languages to reference function parameters and local variables on the stack. It should not be used for ordinary arithmetic or data transfer except at an advanced level of programming. It is often called the extended frame pointer register.

Segment Registers

In real-address mode, 16-bit segment registers indicate base addresses of preassigned memory areas named segments. In protected mode, segment registers hold pointers to segment descriptor tables. Some segments hold program instructions (code), others hold variables (data), and another segment named the stack segment holds local function variables and function parameters.

Instruction Pointer

The EIP, or instruction pointer, register contains the address of the next instruction to be executed. Certain machine instructions manipulate EIP, causing the program to branch to a new location.

EFLAGS Register

The EFLAGS (or just Flags) register consists of individual binary bits that control the operation of the CPU or reflect the outcome of some CPU operation. Some instructions test and manipulate individual processor flags.

Control Flags

Control flags control the CPU’s operation. For example, they can cause the CPU to break after every instruction executes, interrupt when arithmetic overflow is detected, enter virtual-8086 mode, and enter protected mode.

Programs can set individual bits in the EFLAGS register to control the CPU’s operation. Examples are the Direction and Interrupt flags.

Status Flags

The Status flags reflect the outcomes of arithmetic and logical operations performed by the CPU. They are the Overflow, Sign, Zero, Auxiliary Carry, Parity, and Carry flags.

Their abbreviations are shown immediately after their names:

•The Carry flag (CF) is set when the result of an unsigned arithmetic operation is too large to fit into the destination.

•The Overflow flag (OF) is set when the result of a signed arithmetic operation is too large or too small to fit into the destination.

•The Sign flag (SF) is set when the result of an arithmetic or logical operation generates a negative result.

•The Zero flag (ZF) is set when the result of an arithmetic or logical operation generates a result of zero.

•The Auxiliary Carry flag (AC) is set when an arithmetic operation causes a carry from bit 3 to bit 4 in an 8-bit operand.

•The Parity flag (PF) is set if the least-significant byte in the result contains an even number of 1 bits. Otherwise, PF is clear. In general, it is used for error checking when there is a possibility that data might be altered or corrupted.

MMX Registers

MMX technology was added onto the Pentium processor by Intel to improve the performance of advanced multimedia and communications applications. The eight 64-bit MMX registers support special instructions called SIMD (Single-Instruction, Multiple-Data). As the name implies, MMX instructions operate in parallel on the data values contained in MMX registers. Although they appear to be separate registers, the MMX register names are in fact aliases to the same registers used by the floating-point unit.

XMM Registers

The x86 architecture also contains eight 128-bit registers called XMM registers. They are used by streaming SIMD extensions to the instruction set.