Instruction Execution Cycle

The execution of a single machine instruction can be divided into a sequence of individual operations called the instruction execution cycle. Before executing, a program is loaded into memory. The instruction pointer contains the address of the next instruction. The instruction queue holds a group of instructions about to be executed. Executing a machine instruction requires three basic steps: fetch, decode, and execute. Two more steps are required when the instruction uses a memory operand: fetch operand and store output operand.

Each of the steps is described as follows:

- Fetch: The control unit fetches the next instruction from the instruction queue and increments the instruction pointer (IP). The IP is also known as the program counter.

- Decode: The control unit decodes the instruction’s function to determine what the instruction will do. The instruction’s input operands are passed to the ALU, and signals are sent to the ALU indicating the operation to be performed.

- Fetch operands: If the instruction uses an input operand located in memory, the control unit uses a read operation to retrieve the operand and copy it into internal registers. Internal registers are not visible to user programs.

- Execute: The ALU executes the instruction using the named registers and internal registers as operands and sends the output to named registers and/or memory. The ALU updates status flags providing information about the processor state.

- Store output operand: If the output operand is in memory, the control unit uses a write operation to store the data.

The sequence of steps can be expressed neatly in pseudocode:

loop

fetch next instruction

advance the instruction pointer (IP)

decode the instruction

if memory operand needed, read value from memory

execute the instruction

if result is memory operand, write result to memory

continue loop

A block diagram showing data flow within a typical CPU is shown in Fig 4–1.

The diagram helps to show relationships between components that interact during the instruction execution cycle. In order to read program instructions from memory, an address is placed on the address bus. Next, the memory controller places the requested code on the data bus, making the code available inside the code cache. The instruction pointer’s value determines which instruction will be executed next. The instruction is analyzed by the instruction decoder, causing the appropriate digital signals to be sent to the control unit, which coordinates the ALU and floating-point unit.

Although the control bus is not shown in this figure, it carries signals that use the system clock to coordinate the transfer of data between the different CPU components.

Instructions Address mode

Any  instruction  issued  by  the  processor  must  carry  at  least  two types of information. These are the operation to be performed, encoded in what  is  called  the  op-code  field,  and  the  address  information  of  the operand  on  which  the  operation  is to be  performed, encoded in  what  is called  the  address  field.  Instructions can be classified based on the number of operands as:

- Three - address instruction: takes the form operation add-1, add-2, add-3, each of add-1, add-2, and add-3 refers to a register or to a memory location. Add R1,R2,R3  It also indicates that the values to be  added  are  those  stored  in  registers  R1  and  R2  that  the  results should be stored in register R3.

- Two - address instruction:  takes the form operation add-1, add-2.  In  this  form,  each  of  add-1  and  add-2  refers  to  a  register  or  to  a memory location  , ADD R1,R2, adds the contents of register R1 to the contents of register R2 and stores the results in register R2. The original contents of register R2 are lost due to this operation while the original contents of register R1 remain intact.

- One - address instruction:  takes the form operation add1.  Add1 refers to a register or to a memory location, ADD R1 In this case the instruction implicitly refers to a register, called the Accumulator  Racc,  such  that  the  contents  of  the  accumulator  is added to the contents of the register R1 and the results are stored back into the accumulator Racc.

- One and half - address instruction:  take the form operation add1, add2, one refers to a register and other refers to memory and vice versa.  The instruction ADD B, R1. In this case, the instruction adds the contents of register R1 to the contents of memory location B and stores the result in register R1.

- Zero - address instruction: instructions that use stack operation.

The different ways in which operands can be addressed are called the addressing modes.  Addressing modes (described in table 1) differ in the way the address information of operands is specified which include:

- immediate  mode:  the  value  of  the  operand  is  (immediately) available  in  the  instruction  itself,  it  is  customary  to  prefix  the operand  by  the  special  character  (#).The  use  of  immediate addressing leads to poor programming practice because a change in the value of an operand requires a change in every instruction that uses the immediate value of such an operand.

- Direct mode: the operand of instruction represent address of the memory location that holds the operand.

- Indirect  mode:  The operand of instruction ( a  register or a memory location)  holds  the  (effective)  address  of  the  operand .  It is customary  to  include  the  name  of  the  register  or  the  memory location in parentheses.

- Index  mode:  the  address  of  the  operand  is  obtained  by  adding  a constant to the content of a register, called the index register.

- Relative mode:  is the same as indexed addressing except that the program counter (PC) replaces the index register.

- Autoincrement mode:  uses a register to hold the address of the operand.  The content of the autoincrement register is first is used as the effective address of the operand and then increment the content of the autoincrement register.

- Autodecrement mode:  uses a register to hold the address of the operand.  The content of the autodecrement register is first decremented and the new content is used as the effective address of the operand.

Table 1: summary of address mode