- Introduction to Stone,Rocks,Applications

There are many definitions of rocks, which the earth is made off. A more acceptable scientific definition of rocks is that; a rock is a natural occurring solid cohesive aggregate of one or more mineral or mineral materials.

Generally rocks can be divided into three major types based on the process of their formation. These are; Igneous rocks, sedimentary rocks and metamorphic rocks.

Igneous rocks are formed by the solidification and cooling of magma in volcanic areas, while sedimentary rocks are formed by low temperature accumulation of sediments in tectonic basins and topographical sinks. Metamorphic rocks on the other hand are formed by application of temperature and pressure on pre-existing rocks. Metamorphic rocks therefore form at great depths, but exposed on the surface due to erosion and epirogenic movements. Each of these three rock types can be further classified in terms of; chemistry, how the form and environment of formation. The distributions of these major rock types are critical in regional mapping of natural resources.

- Stone vs Brick

- Similarities:

Both stacked-

Mortar Joints-

- Differences:

- Shape: Brick molded - Stone Cut and Carve

Physical Properties: Brick made/controlled – Stone provided by nature

RockTo the geologist any mass of mineral matter, whether consolidated or not, which forms part of the Earth's crust ... (2) The civil engineer regards rock as something hard, consolidated, and/or load bearing, which, where necessary, has to be removed by blasting. This concept also accords with the popular idea of the meaning of the word.

Stone In geology the word 'stone' is admissible only in combinations such as limestone, sandstone, etc., or where it is used as the name for extracted material - building stone, stone road. It should not be used as a synonym for rock or pebble.

1.Types of Rocks used in Stone Masonry

Stone Masonry: Building stones obtained by quarrying from the rocky strata of earth and reducing it to the required shapes and sizes for construction

- Types of rock:

Rocks normally consist of several minerals, some essential, some accessory. A rock may be thought of as a "mineral environment." Each rock type was formed under certain specific conditions, resulting in the formation of a fairly predictable group of minerals. Rocks fall into three classes according to their origin: Igneous - Sedimentary – Metamorphic

Igneous - Formed as a result of cooling of the molten rock to solid state - It is nonporous, hard, strong and durable .

Granite: An igneous-plutonic rock, medium to coarse-grained that is high in silica, potassium, sodium and quartz but low in calcium, iron and

may contain fossils. It can be a component of bricks and cement.

Consists mainly of quartz, feldspar, mica, and other colored minerals; colors include black, gray, red, pink, brown, buff, and green. Non-porous, hard, strong, durable, Color Range, Surface Textures, Sources, Primary uses

Pumice: An igneous-volcanic rock, it is a porous, brittle variety of rhyolite and is light enough to float. It is formed when magma of granite composition erupts at the earth’s surface or intrudes the crust at shallow depths. It is used as an abrasive material in hand soaps, emery boards, etc.

Gabbro: An igneous-plutonic rock, generally massive, but may exhibit a layered structure produced by successive layers of different mineral composition. It is widely used as crushed stone for concrete aggregate, road metal, railroad ballast, etc. Smaller quantities are cut and polished for dimension stone (called black granite).

Basalt: An igneous volcanic rock, dark gray to black, it is the volcanic equivalent of plutonic gabbro and is rich in ferromagnesian minerals. Basalt can be used in aggregate.

magnesium. It is widely used for architectural construction, ornamental stone and monuments.

- Sedimentary:

Conglomerate: A sedimentary rock with a variable hardness, consisted of rounded or angular rock or mineral fragments cemented by silica, lime, iron oxide, etc. Usually found in mostly thick, crudely stratified layers. Used in the construction industry.

Sandstone: A sedimentary rock more or less rounded. Generally thick-bedded, varicolored, rough feel due to uneven surface produced by breaking around the grains. Used principally for construction, it is easy to work, the red-brown sandstone of Triassic age, better known as "brownstone," has been used in many eastern cities.

Metamorphic

Gneiss: A metamorphic uneven granular medium to coarse grained crystalline with more or less parallel mineral orientation. Colors are too variable to be of diagnostic value. Due to physical and chemical similarity between many gneisses and plutonic igneous rocks some are used as building stones and other structural purposes.

Quartzite: A metamorphic or sedimentary rock with crystalline texture, consists of rounded quartz grains cemented by crystalline quartz, generally white, light gray or yellow to brown. Same uses as sandstone. It is a variety of and stone composed of mainly granular quartz cemented by silica, color varies from brown, buff, tan, ivory, red through gray.

Schist: A metamorphic uneven-granular, medium to coarse grained, crystalline with prominent parallel mineral orientation. Goes from silvery white to all shades of gray with yellow to brown tones depending on the mineral concentration. Some schists have graphite and some are used as building stones. Made of silica with smaller amounts of iron oxide and magnesium oxide - Color varies from blue, green, brown, gold, white, gray, and red

Slate: Consists mainly of clays and shales - Major ingredients are silicon dioxide, iron oxide, potassium oxide, magnesium oxide, and sometimes titanium, calcium and sulfur - Slate found in parallel layers, which enables it to be cut into thin sheets

Marble: A metamorphic even-granular grain to medium grained and may be uneven granular and coarse grained in calc-silicate rock. The normal color is white but accessory minerals act as coloring agents and may produce a variety of colors. Depending upon its purity, texture, color and marbled pattern it is quarried for use as dimension stone for statuary, architectural and ornamental purposes. Dolomite rich marble may be a source for magnesium and is used as an ingredient in the manufacture of refracting materials.Discussion:Discuss the different types of rocks and their uses.

- Quarrying and producing bulding stones

Irregular-sized stone is produced by blasting the rock, the larger pieces are cut into smaller units for use as an exterior finish, rest is crushed and sorted into various sizes as aggregates - Most of the dimensional stones used in building construction are produced by cutting large blocks in the quarry - Cut with diamond  belt saws (12ft wide, 2 to 5 ft thick, and 50 ft long); rubber air bags inflated in the saw cut to break it away and then the separated rock is lowered onto prepared stone chips cushion - Thereafter it is cut into smaller sizes and transported by front-end loaders to the mill for further processing

- Types of Stone

Fieldstone -

Rubble Stone -

Dimension Stone -

- Stone Masonry Patterns

- Rubble (Unsquare pieces)

- Ashlar (Square Pieces)

- Coursed or Random

- Orientation

- Types of stone masonry walls and their consruction

- 1. Types of Stone Masonry Walls:

(a) Solid masonry wall made by laying stone masonry over a prepared bed of mortar, and proceeding in a similar manner to increase the height;

(b) Composite wall made of an outer wall of large stone slabs, attached to a backing of structural frame or brick/concrete masonry wall; and

(c) Cavity wall made by two different types of masonry wall separated by a cavity, which is either insulated or empty and connected together by metal ties.

5. Precast concrete masonry and construction of walls

Precast Concrete Masonry Blocks: Manufactured by vibrating a stiff concrete mixture into metal molds, immediately turning it out wet onto a rack (so that the mold can be reused immediately) at a rate of 1000 or more units per hour - Racks are cured at an accelerated  rate by subjecting them to steam, either at the atmospheric pressure or for faster curing at higher pressure. After the units are steam cured, the units are dried to a specific moisture content, and bundled in wooded crates for shipping to the construction site

Made in varying sizes and shapes: Channel bond beam, Low-web beam, Solid unit, Capping unit, A-block, H block, Header unit, Control joint unit, Single Bullnose, etc.

Main ingredient is serpentine; color ranges from olive green to greenish black, is fine grained and dense - Basalt: Color ranges from gray to black; used mainly for paving stones and retaining walls.