CHAPTER FOUR

CIRCUIT THEOREMS

4.1 INTRODUCTION

The growth in areas of application of electric circuits has led to an evolution from simple to complex circuits. To handle the complexity, engineers over the years have developed some theorems to simplify circuit analysis. Such theorems include Thevenin’s and Norton’s theorems. In addition to circuit theorems, we discuss the concepts of superposition, maximum power transfer, Millman’s theorem, in this chapter.

Practice problem:

For the circuit in Figure (4.1), find vo when is = 15 and is = 30 A.

Figure 4.1

4.2 SUPERPOSITION

The idea of superposition rests on the linearity property.

The superposition principle states that the voltage across (or current through) an element in a linear circuit is the algebraic sum of the voltages across (or currents through) that element due to each independent source acting alone.

However, to apply the superposition principle, we must keep two things in mind:

1. We consider one independent source at a time while all other independent sources are turned off. This implies that we replace every voltage source by 0 V (or a short circuit), and every current source by 0 A (or an open circuit).

2. Dependent sources are left intact because they are controlled by circuit variables. With these in mind, we apply the superposition principle in three steps:

Steps to Apply Super position Principle:

1.  Turn off all independent sources except one source. Find the output (voltage or current) due to that active source using nodal or mesh analysis.

2.   Repeat step 1 for each of the other independent sources.

3. Find the total contribution by adding algebraically all the contributions due to the independent sources.

Analyzing a circuit using superposition has one major disadvantage: it may very likely involve more work.  Keep in mind that superposition is based on linearity.

Example 4.1: Use the superposition theorem to find v in the circuit in Fig. 4.2.

Solution / Since there are two sources, let

v = v1 + v2

Figure 4.2 For Example 4.2.

where v1 and v2 are the contributions due to the 6-V voltage source and the 3-A current source, respectively. To obtain v1, we set the current source to zero, as shown in Fig. 4.3(a). Applying KVL to the loop in Fig. 4.3(a) gives

12i1 − 6 = 0 ⇒ i1 = 0.5 A

Thus,

v1 = 4i1 = 2 V

We may also use voltage division to get v1 by writing

v1 =44 + 8(6) = 2 V

Figure 4.3 For Example 4.2:

(a) calculating v1, (b) calculating v2.

To get v2, we set the voltage source to zero, as in Fig. 4.3(b). Using current division,

i3 =84 + 8(3) = 2 A

Hence,         v2 = 4i3 = 8 V

And we find          v = v1 + v2 = 2 + 8 = 10 V

Practice problems:

1-Using the superposition theorem, find vo in the circuit in Figure below.

Answer: 12 V.

2- Use superposition to obtain vx in the circuit of Figure below.

Answer: 0.75 A.

4.3 SOURCE TRANSFORMATION

We have noticed that series-parallel combination and wye-delta transformation help simplify circuits. Source transformation is another tool for simplifying circuits. We can substitute a voltage source in series with a resistor for a current source in parallel with a resistor, or vice versa, as shown in Fig. 4.4. Either substitution is known as a source transformation.

Key Point: A source transformation is the process of replacing a voltage source vs in series with a resistor R by a current source is in parallel with a resistor R, or vice versa.

Figure 4.4 Transformation of independent sources.

We need to find the relationship between vs and is that guarantees the two configurations in Fig. 4.4 are equivalent with respect to nodes a, b.

Suppose RL, is connected between nodes a, b in Fig. 4.4(a). Using Ohms law, the current in RL is.

iL=vsR+RL                  R and RL in series                                                            (4.1)

If it is to be replaced by a current source then load current must be VR+RL

Now suppose the same resistor RL, is connected between nodes a, b in Fig. 4.4 (b). Using current division, the current in RL, is

iL=isRR+RL                                                                                                        (4.2)

If the two circuits in Fig. 4.4 are equivalent, these resistor currents must be the same. Equating the right-hand sides of Eqs.4.1 and 4.2 and simplifying

is=vsR  or    vs= is R                                                                                        (4.3)

However, we should keep the following points in mind when dealing with source transformation.

1. Note from Fig. 4.4 that the arrow of the current source is directed toward the positive terminal of the voltage source.

2. Note from Eq. (4.3) that source transformation is not possible when R = 0, which is the case with an ideal voltage source. However, for a practical, nonideal voltage source, R ≠ 0. Similarly, an ideal current source with R =∞cannot be replaced by a finite voltage source.

Example 4.2: Use source transformation to find vo in the circuit in Fig. 4.5.

Solution:

We first transform the current and voltage sources to obtain the circuit in Fig. 4.6(a). Combining the 4-Ω and 2-Ω resistors in series and transforming the 12-V voltage source gives us Fig. 4.6(b). We now combine the 3-Ω and 6-Ω resistors in parallel to get 2-Ω. We also combine the 2-A and 4-A current sources to get a 2-A source. Thus, by repeatedly applying source transformations, we obtain the circuit in Fig. 4.6 (c).

Figure 4.5

Figure 4.6

Alternatively, since the 8-Ω and 2-Ω resistors in Fig. 4.6(c) are in parallel, they have the same voltage vo across them. Hence,

vo = (8||2)(2 A) = 8×210 (2) = 3.2 V

Practice problems:

1-Find io in the circuit shown below using source transformation.

Answer: 1.78 A.

4.4 THEVENIN’S THEOREM

It often occurs in practice that a particular element in a circuit is variable (usually called the load) while other elements are fixed. As a typical example, a household outlet terminal may be connected to different appliances constituting a variable load. Each time the variable element is changed, the entire circuit has to be analyzed all over again. To avoid this problem, Thevenin’s theorem provides a technique by which the fixed part of the circuit is replaced by an equivalent circuit.

According to Thevenin’s theorem, the linear circuit in Fig. 4.7(a) can be replaced by that in Fig. 4.7(b) is known as the Thevenin equivalent circuit; it was developed in 1883 by M. Leon Thevenin (1857–1926), a French telegraph engineer.

Figure 4.7 Replacing a linear two-terminal

circuit by its Thevenin equivalent: (a) original circuit, (b) the Thevenin equivalent circuit.

Thevenin’s theorem states that a linear two-terminal circuit can be replaced by an equivalent circuit consisting of a voltage source VTh in series with a resistor RTh, where VTh is the open-circuit voltage at the terminals and RTh is the input or equivalent resistance at the terminals when the independent sources are turned off.

To find the Thevenin equivalent voltage VTh and resistance RTh, suppose the two circuits in Fig. 4.7 are equivalent. the open-circuit voltage across the terminals a-b in Fig. 4.7(a) must be equal to the voltage source VTh in Fig. 4.7(b), since the two circuits are equivalent. Thus VTh is the open-circuit voltage across the terminals as shown in Fig. 4.8(a); that is,

VTh = voc                                                                                                                (4.4)

Figure 4.8 Finding VTh and RTh.

RTh is the input resistance at the terminals when the independent sources are turned off, as shown in Fig. 4.8(b); that is,

RTh = Rin                                                                                                                 (4.5)

To apply this idea in finding the Thevenin resistance RTh, we need to consider two cases.

CASE 1: we turn off all independent sources. RTh is the input resistance of the network looking between terminals a and b, as shown in Fig. 4.8(b).

Figure 4.9 Finding RTh when circuit has

dependent sources.

CASE 2: If the network has dependent sources, we turn off all independent sources. As with superposition, dependent sources are not to be turned off because they are controlled by circuit variables. We apply a voltage source vo at terminals a and b and determine the resulting current io. Then RTh = vo/io, as shown in Fig. 4.9(a). Alternatively, we may insert a current source io at terminals a-b as shown in Fig. 4.9(b) and find the terminal voltage vo. Again RTh = vo/io. Either of the two approaches will give the same result. In either approach we may assume any value of vo and io. For example, we may use vo = 1 V or io = 1 A, or even use unspecified values of vo or io. It often occurs that RTh takes a negative value. In this case, the negative resistance (v = −iR) implies that the circuit is supplying power. This is possible in a circuit with dependent sources. The current IL through the load and the voltage VL across the load are easily determined once the Thevenin equivalent of the circuit at the load’s terminals is obtained, as shown in Fig. 4.10(b). From Fig. 4.10(b), we obtain

IL =VThRTh + RL                                                                                                    (4.6a)

VL = RLIL =RLRTh + RLVTh                                                                             (4.6b)

Note from Fig. 4.10(b) that the Thevenin equivalent is a simple voltage divider, yielding VL by mere inspection.

Figure 4.10 A circuit with a load :(a) original circuit, (b) Thevenin equivalent.

Example 4.3: Find the Thevenin equivalent circuit of the circuit shown in Fig. 4.11, to the left of the terminals a-b. Then find the current through RL = 6, 16, and 36 Ω.

Figure 4.11 For Example 4.3.

Solution:

We find RTh by turning off the 32-V voltage source (replacing it with a short circuit) and the 2-A current source (replacing it with an open circuit). The circuit becomes what is shown in Fig. 4.12(a). Thus,

RTh = 4||12 + 1 =4 × 1216 + 1 = 4 Ω

Figure 4.12 For Example 4.3: (a) finding RTh, (b) finding VTh.

To find VTh, consider the circuit in Fig. 4.12(b). Applying mesh analysis to the two loops, we obtain

−32 + 4i1 + 12(i1 − i2) = 0,        i2 = −2 A

Solving for i1, we get i1 = 0.5 A. Thus,

VTh = 12(i1 − i2) = 12(0.5 + 2.0) = 30 V

The Thevenin equivalent circuit is shown in Fig. 4.13. The current through RL is

IL =VThRTh + RL =304 + RL

When RL = 6,       IL  =3010=3 A

Figure 4.13 The Thevenin equivalent circuit

When RL = 16,     IL  =3020=1.5 A

When RL = 36,     IL  =3040=0.75 A

Practice problem: Using Thevenin’s theorem, find the equivalent circuit to the left of the terminals in the circuit in Figure below. Then find i.

Answer: VTh = 6 V, RTh = 3 Ω, i = 1.5 A.

4.5 NORTON’S THEOREM

In 1926, about 43 years after Thevenin published his theorem, E. L. Norton, an American engineer at Bell Telephone Laboratories, proposed a similar theorem.

Norton’s theorem states that a linear two-terminal circuit can be replaced by an equivalent circuit consisting of a current source IN in parallel with a resistor RN, where IN is the short-circuit current through the terminals and RN is the input or equivalent resistance at the terminals when the independent sources are turned off.

Thus, the circuit in Fig. 4.14(a) can be replaced by the one in Fig. 4.14(b).

Figure 4.14 (a) Original circuit, (b) Norton equivalent circuit.

We are mainly concerned with how to get RN and IN. We find RN in the same way we find RTh. In fact, the Thevenin and Norton resistances are equal; that is,

RN = RTh                                                                                                               (4.7)

To find the Norton current IN, we determine the short-circuit current flowing from terminal a to b in both circuits in Fig. 4.14. It is evident that the short-circuit current in Fig. 4.14(b) is IN. This must be the same short-circuit current from terminal a to b in Fig. 4.14(a), since the two circuits are equivalent. Thus,

IN = isc                                                                                                                    (4.8)

Dependent and independent sources are treated the same way as in Thevenin’s theorem. Observe the close relationship between Norton’s and Thevenin’s theorems: RN = RTh as in Eq. (4.7), and

IN =VThRTh                                                                                                                (4.9)

This is essentially source transformation. For this reason, source transformation is often called Thevenin-Norton transformation.

We can calculate any two of the three using the method that takes the least effort and use them to get the third using Ohm’s law. Example 4.10 will illustrate this. Also, since

VTh = voc                                                                                                              (4.10a)

IN = isc                                                                                                                  (4.10b)

RTh =vocisc = RN                                                                                                (4.10c)

the open-circuit and short-circuit tests are sufficient to find any Thevenin or Norton equivalent.

Example 4.4 Find the Norton equivalent circuit of the circuit in Fig. 4.15.

Solution:

Figure 4.15 For Example 4.4.

We find RN in the same way we find RTh in the Thevenin equivalent circuit. Set the independent sources equal to zero. This leads to the circuit in Fig. 4.16(a), from which we find RN. Thus,

RN = 5‖(8 + 4 + 8) = 5‖20 =20 × 525 = 4 Ω

To find IN, we short-circuit terminals a and b, as shown in Fig. 4.16(b). We ignore the 5-Ω resistor because it has been short-circuited. Applying mesh analysis, we obtain

i1 = 2 A,             20i2 − 4i1 − 12 = 0

From these equations, we obtain

i2 = 1 A = isc = IN

Alternatively, we may determine IN from VTh/RTh. We obtain VTh as the open-circuit voltage across terminals a and b in Fig. 4.16(c). Using mesh analysis, we obtain

i3 = 2 A

25i4 − 4i3 − 12 = 0   ⇒ i4 = 0.8 A

and

voc = VTh = 5i4 = 4 V

Figure 4.16 For Example 4.4; finding: (a) RN, (b) IN = isc, (c) VTh = voc.

Hence,

IN =VThRTh =44 = 1 A

as obtained previously. This also serves to confirm Eq. that RTh = voc/isc = 4/1 = 4 Ω. Thus, the Norton equivalent circuit is as shown in Fig. 4.17.

Figure 4.17 Norton equivalent of the circuit in Fig. 4.15.

Practice problem: Find the Norton equivalent circuit for the circuit in Figure below.

Answer: RN = 3 Ω, IN = 4.5 A.

4.6 MAXIMUM POWER TRANSFER

In many practical situations, a circuit is designed to provide power to a load. While for electric utilities, minimizing power losses in the process of transmission and distribution is critical for efficiency and economic reasons, there are other applications in areas such as communications where it is desirable to maximize the power delivered to a load. We now address the problem of delivering the maximum power to a load when given a system with known internal losses.

The Thevenin equivalent is useful in finding the maximum power a linear circuit can deliver to a load. We assume that we can adjust the load resistance RL. If the entire circuit is replaced by its Thevenin equivalent except for the load, as shown in Fig. 4.18, the power delivered to the load is

Figure 4.18 The circuit used for

maximum power transfer.

p = i2RL =  VThRTh + RL2 RL                              (4.11)

For a given circuit, VTh and RTh are fixed. By varying the load resistance RL, the power delivered to the load varies as sketched in Fig. 4.19. We notice from Fig. 4.19 that the power is small for small or large values of RL but maximum for some value of RL between 0 and ∞. We now want to show that this maximum power occurs when RL is equal to RTh. This is known as the maximum power theorem.

Figure 4.19 Power delivered to the load as a function of RL

Maximum power is transferred to the load when the load resistance equals the Thevenin resistance as seen from the load (RL = RTh).

To prove the maximum power transfer theorem, we differentiate p in Eq. (4.11) with respect to RL and set the result equal to zero. We obtain

dpdRL = VTh2  (RTh + RL)2 - 2RL(RTh + RL)(RTh + RL)4 = VTh2  (RTh + RL - 2RL)(RTh + RL)3   = 0

This implies that

0 = (RTh + RL − 2RL) = (RTh − RL)                                                                      (4.12)

which yields

RL = RTh                                                                                                                (4.13)

showing that the maximum power transfer takes place when the load resistance RL equals the Thevenin resistance RTh. We can readily confirm that Eq. (4.13) gives the maximum power by showing that d2p/dR2L < 0.

The maximum power transferred is obtained by substituting Eq. (4.13) into Eq. (4.11), for

pmax =VTh2 4RTh                                                                                                         (4.14)

Equation (4.14) applies only when RL = RTh. When RL ≠ RTh; we compute the power delivered to the load using Eq. (4.11).

Example 4.5: Find the value of RL for maximum power transfer in the circuit of Fig. 4.20. Find the maximum power.

Figure 4.20 For Example 4.5.

Solution:

We need to find the Thevenin resistance RTh and the Thevenin voltage VTh across the terminals a-b. To get RTh, we use the circuit in Fig. 4.21(a) and obtain

RTh = 2+3+6‖12 = 5 +6 × 1218 = 9 Ω

Figure 4.21 For Example 4.6: (a) finding RTh, (b) finding VTh.

To get VTh, we consider the circuit in Fig. 4.21(b). Applying mesh analysis,

−12 + 18i1 − 12i2 = 0,           i2 = −2 A

Solving for i1, we get i1 = −2/3. Applying KVL around the outer loop to get VTh across terminals a-b, we obtain

−12 + 6i1 + 3i2 + 2(0) + VTh = 0 ⇒ VTh = 22 V

For maximum power transfer,

RL = RTh = 9 Ω

and the maximum power is

pmax =VTh2 4RL =2224 × 9 = 13.44 W

4.7 MILLMAN’S THEOREM

Through the application of Millman’s theorem, any number of parallel voltage sources can be reduced to one. In Fig. 4.22, for example, the three voltage sources can be reduced to one. This would permit finding the current through or voltage across RL without having to apply a method such as mesh analysis, nodal analysis, superposition, and so on.

FIG. 4.22 Demonstrating the effect of applying Millman’s theorem.

In general, Millman’s theorem states that for any number of parallel voltage sources,

(4.15)

and

(4.16)

Example 4.6: Using Millman’s theorem, find the current through and voltage across the resistor RL of Fig. 4.23.

Solution: By Eq. (4.15),

FIG. 4.23 Example 4.6.

The minus sign is used for E2 /R2 because that supply has the opposite polarity of the other two. The chosen reference direction is therefore that of E1 and E3. The total conductance is unaffected by the direction,

And

Eeq=+10V5Ω-16V4Ω+8V2Ω15Ω+14Ω+12Ω=2A-4A+4A0.2 S+0.25 S+0.5 S

=2 A0.95 S=2.105 V

FIG. 4.24 The result of applying Millman’s theorem to the network of Fig. 4.23.

with     Req=115Ω+14Ω+12Ω=10.95 S=1.053 Ω

The resultant source is shown in Fig. 4.24, and

IL=2.105V1.053Ω+3Ω=2.105 V4.053 Ω=0.519 A

with     VL= ILRL= (0.519 A)(3 Ω) = 1.557 V

*The dual of Millman’s theorem appears in Fig. 4.25. It can be shown that Ieq and Req, as in Fig. 4.25 are given by

FIG. 4.25 The dual effect of Millman’s theorem.

(4.17)

and

(4.18)