Free vibration and Buckling Behavior of Tapered Beam by Finite Element Method

Dhyai Hassan Jawad

Abstract

This paper deals with free vibration and buckling behavior of non-uniform Euler-Bernoulli beam under variation of tapered parameter and degree of flexural bending by using Finite Element Method and linked with Matlab Program. The results obtained were compared with those results given in the literatures and it is found that the natural frequency and buckling load decrease with increasing the tapered parameter and degree of flexural stiffness of tapered beams.

الخلاصة

هذا البحث يهتم بدراسة الاهتزاز الحر وسلوك الانبعاج  لعتبة اويلر- برنولي مستدقة غير منتظمة بتغير معامل الاستدقاق ودرجة الأس لجساءة الانحناء باستخدام العناصر المحددة و برنامج الماتلاب. النتائج التي تم الحصول عليها قورنت مع نتائج مصادر أخرى, ووجد بان التردد وحمل الانبعاج يقل بزيادة معامل الاستدقاق ودرجة الأس لجساءة الانحناء للعتبة المستدقة.

Nomenclature

Symbols               Meaning

mx              mass distribution per unit length

EIx             flexural stiffness per unit length

wx             displacement in z direction

qT            deflection vector

Ni              interpolation factors

Ni(ξ)       shape function

ξ=xl        non –dimensional length of beam element

U               potential energy

Ke          elastic stiffness

T             kinetic energy

me          mass matrix

V            strain energy

λ           a critical  bucking load

Po              value of axial compression force

Kg             geometrical stiffness matrix

rm          tapered beam for mass

rs              tapered beam for stiffness

ß          non- dimensional natural frequency

n         Number of element

i         Number of node

l         length of beam

ωn      natural frequency

1-Introduction

Tapered members are widely used in the modern construction industry, because of their (i) structural efficiency, which in turn may lead to significant material savings, (ii) ability to meet architectural and functional requirements and (iii) competitive fabrication costs. However, a designer can only take full advantage of the benefits of beam tapering provided that he is equipped with reliable and efficient methods of analysis, which (i) lead to accurate predictions of the tapered member structural behavior and, at the same time, (ii) do not involve a computer effort prohibitive for routine applications.. The strength of laterally unrestrained beams is frequently governed by the lateral buckling (or flexural-tensional buckling) failure.

The vibration and bulking problems of non-uniform beams have been extensively studied by several investigators. Several cases of tapered beams with different end conditions were obtained by Mabie and Rogers ,while Laura treats various cases of non-uniform beams with different conditions of end restraints. A direct solution for the transverse vibration of Euler–Bernoulli wedge and cone beams was obtained by Naguleswaran, while Abrate obtains the exact solution for the vibration of non-uniform rods and beams. Lee et al. were studied the analysis of non-uniform beam vibration by a green function method in the Laplace transform domain. Brown was studied the lateral buckling load of a tapered beam by finite difference analysis. In this method, the effect of tapering could not be completely taken into account in the expressions of nonlinear strains, which may lead to incorrect lateral buckling loads. Ronagh et al. was found the errors in lateral buckling loads caused by this method cannot be eliminated merely using fine mesh configuration in the finite element analysis. Ronagh et al. and recently Andrade and Camotim were investigated the lateral buckling of tapered beams employing the FE method, based on their total potentials presented.

Si Yuan et. were studied the exact dynamic stiffness method for non-uniform Timoshenko beam vibration and Bernoulli Euler column bulking while Jung et. deals with free vibration problems of non-uniform Euler-Bernoulli beam under various supporting conditions. Zang Lei. was presented a new theory for the lateral buckling of wet-tapered I-beam while Vaidotas Sapalau studies a theoretical and numerical analysis of tapered beam- columns subjected to a bending moment and axial force. Aniosio A. was studied the lateral torsional buckling of singly symmetric web-tapered thin –walled I beam by using FEM.

In this study a finite element method is introduced to solve the vibration and buckling of non-uniform beam with clamped-free boundary condition to obtain natural frequencies and buckling loads.

2- Theoretical Analysis

A schematic of mass m(x) and flexural stiffness per unit length EI(x) is shown in fig(1), W(x) is the displacement in Z direction. In the present work, is a considered Euler-Bernoulli beams for analysis of out-of-plane bending vibration. The following function is used in the analysis to represent deflection which  is given by  Si Yuan

W(x)=a1+a2x+a3x2+a4x3                                                                                    (1)

The deflection vector of the elemental finite element is

qT=[V1θ1V2θ2]                                                                                               (2)

After application of the boundary conditions of clamped- free to calculate the interpolation functions (N1,N2,N3,N4) to describe the distribution of displacement. Eq.(1) can be written as in finite element formulation,

W(x)=N1V1+N2θ1+N3V2+N4θ2                                                                            (3)

Where

N1(ξ)=1−3ξ2+2ξ3

N2(ξ)=L(ξ−2ξ2+ξ3)                        shape factors

N3(ξ)=3ξ2−2ξ3

N4(ξ)=L(ξ3−ξ2)

Where, ξ=xL, is the non-dimensional length of beam element (element coordinates).

The potential energy of the beam element is given by  Si Yuan :

U=120LEI(x)[(W''(x,t))2]dx                                                                                      (4)

Eq.(4)  can be written in matrix form as

U=12qTkeq                                                                                                                (5)

Where, ke, elastic stiffness matrix.

ke=0LEI(x)Ni″Nj″dx                                                                                                (6) Where,  Ni} } = {  {d rSup { size 8{2} } N rSub { size 8{i} } }  over  {dξ rSup { size 8{2} } } } } {

The kinetic energy of the beam element is

T=120Lm(x)[W(x)]2dx                                                                                             (7)

Eq.(7)  can be written in matrix form as

T=12qTmeq                                                                                                                (8)

Where, me, mass matrix

me=0Lm(x)NiNjdx                                                                                                    (9)

The strain energy V denotes the work down by a critical load (λPo)

Given by the equation

V=0Lλp0[W']2dx                                                                                                      (10)

Where p0is a value of axial compression force.

Eq.(10)  can be written in matrix form as

V=12qTkgq                                                                                                              (11)

Where, kg, geometrical stiffness matrix

kg=0LλPoNi′Nj′dx                                                                                                   (12)

Where,  Ni'=dNidξ

Mass and stiffness matrices of each beam element are used to form global mass and stiffness matrices. The dynamic response of a beam for a conservative system can be formulated by means of Lagrange’s equation of motion in which the external forces are expressed in terms of time- dependent potentials, and then performing the required operations the entire system leads to the governing matrix equation of motion:

Mq⋅+[Ke−λPoKe]q=0                                                                                        (13)

The above equation represents the solution of two relate problems, they are given by  Si Yann

:-

1- Free vibration                            [Ke−β2M]q=0                                               (14)

2- Buckling behavior                    [Ke−λPoKg]q=0                                              (15)

m(x),EI(x)

L

b

x

h(x)

Fig (1)A: Cantilevered tapered beam with

rectangular Cross-section

W(x)

v1                         v2

θ1                        θ2

Normal to x-y plane

i=1                                               i=n

1              2

B: element of beam

2 nodes  4- DOF

3- Results and Discussion

Three case of mass and stiffness distribution were studied. In the first case linear mass and linear stiffness distribution, the second the mass is linear and stiffness is 2nd order. And third, the mass is linear and stiffness is 4th order.

Case(1): [Linear mass, Linear stiffness]

m(x)=mo(1−rmξ)

EI(x)=EIo(1−rsξ)

Case(2):[Linear mass, second degree stiffness]

m(x)=mo(1−rmξ)

EI(x)=EIo(1−rsξ)i=2

Case(3):[Linear mass, forth degree stiffness]

m(x)=mo(1−rmξ)

EI(x)=EIo(1−rsξ)i=4

Where (mo) parameter for mass distribution per length ,at x=0, (EIo) parameter for stiffness  distribution  of flexural rigidity, at x=0, and  (rm), (rs)are the tapered beam for mass and stiffness and (i) is super index refer to the order of non linearity for both mass and stiffness distribution , respectively.

For all cases, value of  (rm) and (rs) are equals and vary from (0→1) step 0.25 to obtained its effect on the natural frequency and corresponding buckling loads for five modes only.

Table (1), (2) and (3) show an important comparison of non-dimensional natural frequency (ß) and critical buckling load of a uniform cantilever beam with results from Jurg et al. Si Yuan et al. and Zhang Lei.

Table:(1) Comparisons of non-dimensional natural frequency ( β )

of a uniform cantilever beam.

Mode No             Present work                 Jung                  Si Yuan

1                    3.5155                         3.5160                 3.5161

2                    22.0336                       22.0345               22.0345

3                    61.6963                       61.6972               61.6972

4                    120.9018                       -------               120.9017

5                    199.8575                        ------                199.8573

Table: (2) Comparison of critical buckling load of a uniform cantilever beam.(λ)

Mode No                 Present work                   Zhang Lei

1                          2.4673                         2.4682

2                          22.2057                       22.2055

3                          61.6837                       61.6835

4                          120.8966                    120.8965

5                          199.8592                    199.8590

Table(3): Comparison of non-dimensional natural frequency and critical buckling load of tapered cantilever beam.

Tapered

Parameter        Mode No          (Non-dimensional Natural Frequency)1/2             (Critical Buckling Loads)1/2

Present work       Jung              Present work     Si Yuan

rm=0.8rs=0.95       1                     1.6768          1.6742                0.9712          0.9705

2                    4.1978          4.1969                2.1937           2.9131

3                    7.0244          7.0232                4.8562           4.8557

4                    9.8332          9.8314                6.7987           6.7982

5                   12.6428        12.6422               8.7412           8.7408

rm=0.5rs=0.5        1                    1.8585           1.8580                1.3352            1.3347

2                     4.6525           4.6523                4.0058            4.0050

3                     7.7853           7.7847                6.6764            6.6761

4                     10.8998        10.8990               9.3470            9.3465

5                     14.0122          -------               12.0176          12.0171

All the frequencies and buckling load predicted agree very well with the published results.

Figs (2,3), (4,5) and (6,7) present the non dimensional natural frequency and critical buckling loads  for the first , second  and third case respectively. From the first sight it was observed that the tapered ratio has a great effect on the vibration characteristics of the beam.

In general, it was noted that the natural frequency and corresponding critical buckling loads decrease with increasing in the taper ratio (rm) and (rs) for all modes, its magnitude are different from mode to other.

In other words, natural frequency of the 2nd   mode (as example) for different values of tapered ratio are smaller than that corresponding for 3rd , 4th ,and 5th modes, respectively.

The band width of effectively is increased with increasing the modes number due to the increasing in the natural frequency and buckling loads when the mode number is increased also.

In addition, since the natural frequency is a structured property (means that its value is the same at each point in the structure), therefore, its value is largely depends on the effective stiffness and mass at same point. This leads to that natural frequency in the first case is greater than that corresponding to the second and third case, respectively. due to effect of super index (i) which reduces the stiffness contribution to obtain the frequency.

Fig (2): Variation of non-dimensional frequency with tapered parameter for case (1)

Fig (3): Variation of critical buckling load with tapered parameter for case (1)

Fig (4): Variation of non-dimensional frequency with tapered parameter for case (2)

Fig (5): Variation of critical buckling load with tapered parameter for case (2)

Fig (6): Variation of non-dimensional frequency with tapered parameter for case (3)

Fig (7): Variation of critical buckling load with tapered parameter for case (3)

4-Conclusions

In this paper the finite element method has been further extended to vibration and buckling of non-uniform Euler-Bernoulli beam from the results, some of the conclusions can be drawn as below

- The non-dimensional frequency and critical buckling load predication obtained for the uniform and tapered beams are compared with the corresponding values mentored in the literature and found to agree very well.

- The natural frequency and buckling load decrease with increasing of the tapered parameters.

- The degree of the flexural load effect on the natural frequency and buckling load was decreased with the increasing in the degree of flexural bending stiffness.

5-References

H.H. Mabie, C.B. Rogers "Transverse Vibrations of Double-Tapered Cantilever Beams with End Support and With End Mass, Journal of the Acoustical Society of America 55 , 1974, 986–991.

P.A.A. Laura,and  R.H. Guitierrez, "Vibrations of a Elastically Restrained Cantilever Beam of Varying Cross Section With Tip Mass of Finite Length" Journal of Sound and Vibration 108 ,1986,  123–131.

S. Naguleswaran "Vibration of an Euler–Bernoulli Beam of Constant Depth and With Linearly Varying Breadth" Journal of Sound and Vibration 153 (3), 1992, 509–522.

S. Abrate "Vibration of Non-uniform Rods and Beams" Journal of Sound and Vibration 185 (4) ,1995, 703–716.

S.Y. Lee, H.Y. Ke, Y.H. Kuo "Exact Solutions for The Analysis of General Elastically Restrained Non-uniform Beams" Transactions of the ASME: Journal of the Applied Mechanics 59,  1992, 205–212.

Brown TG "Lateral-Torsional Buckling of Tapered I-Beams" Journal of the Structural Division, ASCE  107(4), 1981, 689–697.

Ronagh HR, Bradford MA, Attard MM  "Nonlinear Analysis of Thin Walled Members of Variable Cross-Section, Part I: Theory" Computers and Structures 77, 2000, 285–99

Andrade A, Camotim D. "Lateral-Torsional Buckling of Singly Symmetric Tapered Beams Theory and Applications" Journal of Engineering Mechanics  131(6), 2005, 586–597.

Si Yuan  , Kangsheng Ye ,Cheng Xiao. Exact Dynamic Stiffness Method for Non-uniform Timoshenko Beam Vibrations and Bernoulli–Euler Column Buckling"  Journal of Sound and Vibration  303, 2007, 526–537

Jung-Chang Hsu, Hsin-Yi Lai, C.K. Chen "Free Vibration of Non-uniform Euler–Bernoulli Beams with General Elastically End Constraints using A Domain Modified Decomposition Method" Journal of Sound and Vibration  453, 2008, 412-429.

Zhang Lei, Tong Geng Shu "Lateral Buckling of Web-Tapered I-Beams: A new theory" Journal of Constructional Steel Research  43(12), 2008, 105-120

Vaidotas Sopalas ,michail Samofalov, Viaceslavas Saraskinast. "FEM Stability Analysis of Tapered Beam-Column"  Journal of civil Eng. and  management. 4(3), 2005, 211-216.

Anisio Andrade , Dinar Camotim, P. Borges Dinis "Lateral-Torsional Buckling of Singly Symmetric Web-Tapered Thin-Walled I-Beams:1D Model vs. Shell FEA" Computers and Structures 85, 2007, 1343–1359..

P.A.A. Laura, B.V. De Greco, J. Utjes, R. Carnicer, "Numerical Experiments on Free and Forced Vibrations of Beams of Non-uniform Cross Section" Journal of Sound and Vibration 120 ,1988, 587–596.

Yang YB, Yau JD "Stability of Beams With Tapered I-Sections. Journal of Engineering Mechanics, ASCE ,113(9), 1987, 1337–57.