Application of Water Quality Index for Evaluation of Groundwater Quality for Drinking Purpose in Dibdiba Aquifer, Kerbala City, Iraq

Fadhil Mohamed Al- Mohammed 	 	Abdulkider Aziz Mutasher

Foundation of Technical Education, Technical Institute/Kerbala

Abstract

Water quality index (WQI) is a mathematical tool used to transform large quantities of water quality data into single number which represents water quality level. This research is accomplished to evaluate the quality of groundwater in Dibdiba aquifer/Kerbala city-Iraq for the purpose of drinking by using Water Quality Index (WQI). Groundwater samples were collected in April, 2010 from 20 wells. Various physicochemical parameters: pH, total dissolved solids, potassium, bicarbonate, sulphate, total hardness, sodium, calcium, magnesium, and chloride have been calculated in all the water samples. In most of the parameters the concentration observed were found to be above the permissible limits of the Iraqi standard for drinking purpose. The results showed that WQI values for the groundwater of the study area ranged from 432.6 to 184.5, which mean water was found to be severely contaminated and unsuitable for drinking purpose at all sites of the wells. Also this research concluded that further improvement is required to treat the water of Dibdiba aquifer wells for using as drinking purpose.

Keywords: Kerbala, Water Quality Index, groundwater, Iraq.

الخلاصة

ان مؤشر نوعية المياه هو اداة رياضية تستخدم لتحويل عدد من المتغيرات الخاصة بنوعية المياه الى رقم منفرد يعبر عن مستوى نوعية المياه.تم انجاز البحث لتقييم نوعية المياه الجوفية في خزان دبدبة ضمن نطاق مدينة كربلاء المقدسة وتقدير صلاحيتها لغرض الشرب باستخدام مفهوم مؤشر نوعية المياه. جمعت نماذج المياه الجوفية خلال شهر نيسان 2010 من 20 بئر. تم حساب عدد من المتغيرات لكل نموذج وهي درجة التفاعل والاملاح الذائبة الكلية والبوتاسيوم والبيكاربونات والكبريتات والعسرة الدائمية والصوديوم والكالسيوم والمغنيسيوم والكلور.ان التراكيز التي تم الحصول عليها لمعظم المتغيرات  هي اكبر من الحدود المسموح بها وفقا للمواصفة العراقية لمياه الشرب. اوضحت الدراسة بان قيم مؤشر نوعية المياه للمياه الجوفية في منطقة الدراسة يتراوح بين 432.6 و 184.5 , وهذا يعني ان مياه جميع الابار ملوثة وغير ملائمة للاستخدام لغرض الشرب.وقد بينت الدراسة بان المياه الجوفية لمنطقة الدراسة تحتاج الى معالجة لاستخدامها لغرض الشرب والاستهلاك البشري.

Introduction

Groundwater is used for domestic and industrial water supply and irrigation all over the world. In the last few decades, there has been a tremendous increase in the demand for fresh water due to rapid growth of population and the accelerated pace of industrialization.

The availability of water in Iraq shows a great deal with spatial and temporal variability. The increase in population and expansion of economic activities undoubtedly leads to increasing demand of water use for various purposes.

Geographically, Iraq is one of the Middle-East countries. Water in this region is inherently scarce as a result of naturally arid climatic conditions (Hamdy A., Trisorio Liuzzi G., 2005).

The Tigris and Euphrates rivers represent the main source of water in Iraq, which are used for various purposes. Turkey, located at the headwaters of the Tigris and Euphrates, controls water flowing downstream through Iraq and to the Arab Gulf. In the mid to late 1990,s, Turkey began to implementation of the great Anatolia project GAP that consists of building 24 dams along the upper Tigris and Euphrates to generate electricity and irrigate a large area of land. The GAP project has effectively reduced water flows in the Euphrates to 1/3 its original annual average.

Karbala city depends mainly on the Euphrates river to meet its water needs for drinking, irrigation, and other purposes. The reducing of the Euphrates flow rate in the future requires researchers to put plans to avert the threat of water scarcity by exploring and classifying other water sources like groundwater as quantity and quality to fulfill water needs of the city for various purposes.

The water quality index WQI method for groundwater quality assessment is widely used around the world due to the capability of fully expression of the water quality information and is one of the most effective tools and important parameters to the evaluation and management of groundwater quality (Wu Jianhua, et al. 2011). WQI is defined as a rating reflecting the composite influence of different water quality parameters (Ramakrishnalah et al., 2009).

The concept of WQI was firstly used by Horton, (1965).The general WQI was developed by Brown et al. (1970) and improved by Deininger for the Scottish Development Department (1975). Over the years, many indices have been calculated for special purposes (Dalkey 1968; Liebman 1969; Prati et al. 1971; O’Connor 1972; Harkins 1974; Walski and Parker 1974; Inhaber 1975; Shaefer and Janardan 1977; Provencher and Lamontagne 1979; Couillard and Lefebvre 1985; House and Ellis 1987). The objective of the present study is to investigate the groundwater quality of Dibdiba Aquifer, Kerbala City, Iraq for human consumption by using a water quality index (WQI).

Methodology

Dibdiba aquifer covers the desert region extending between the two cities Kerbala and Najaf as shown in Fig. (1). The study area is apart of Dibdiba aquifer that located within Kerbala city. Kerbala city is located at central zone of Iraq, about 100 km south of Baghdad, which is the capital city of Iraq. Geologically, Dibdiba formation is unconfined aquifer, it consists of mainly from sandstone with fine gravel, clay siltstone, and silty clay stone and it covered with gypsum and sandy soil layers (Ayob M.S. et al., 1995 and Al-Jiburi H. K., 2002).The climate of the study area is under the condition of the western desert which is characterized by cold winter with low rainfall and hot and dry summer.

Groundwater samples were collected in April, 2010 from 20 wells. Coordinates of each well location were identified using GPS as shown in Table (1). Fig (1) shows the Geographical location of the study area and the sites of the wells. Each of the groundwater samples was analyzed for 10 parameters which were pH, total dissolved solids (TDS), total hardness, calcium, magnesium, sodium, potassium, alkalinity, chloride, and sulphate using standard procedures recommended by (APHA: AWWA: WEF, 1998). All the water samples were collected in 3 L plastic bottles which were washed with distilled water before sampling and transporting them to the laboratory. Chemical analysis of the water was carried out at the central laboratory of the Agriculture directorate of Kerbala.

Table (1).Geographical locations of the studied wells in UTM.

| Well No. | UTME | UTMN |
| --- | --- | --- |
| 1 | 412311 | 3600481 |
| 2 | 411893 | 3600553 |
| 3 | 411433 | 3600657 |
| 4 | 410920 | 3600753 |
| 5 | 410290 | 3600774 |
| 6 | 408634 | 3601664 |
| 7 | 408125 | 3602027 |
| 8 | 407695 | 3602313 |
| 9 | 407281 | 3602586 |
| 10 | 406999 | 3602789 |
| 11 | 406704 | 3603010 |
| 12 | 406283 | 3603288 |
| 13 | 405028 | 3604175 |
| 14 | 404528 | 3604529 |
| 15 | 404328 | 3604729 |
| 16 | 404046 | 3605133 |
| 17 | 402948 | 3606893 |
| 18 | 402726 | 3607128 |
| 19 | 401669 | 3611241 |
| 20 | 401669 | 3611657 |

1

6

Wells

Dibdiba

5

12

8

9

3

2

4

7

10

11

13

14

15

16

20

19

Kerbala city

18

17

Fig. (1). Groundwater sampling locations.

Water Quality Index

Water quality index (WQI) is defined as a technique of rating that provides the composite influence of individual water quality parameter on the overall quality of water. WQI has been calculated to evaluate the suitability of groundwater quality of Dibdiba aquifer (Kerbala city) for drinking purposes. The WQI was calculated for each well by using ten parameters, namely TDS, Ca+2, Mg+2, Na+1, K+1, SO4-2, CL-1, HCO3-1, TH, pH according to Iraqi drinking water standards for each chemical parameter illustrate in Table (2).For calculating WQI, a weighted arithmetic index method was used as below (Brown RM et.al, 1972 and Dhirendra M. J. et al., 2009):

1- Calculating the constant of proportionality by using Eq. (1) as shown in Table (3):

k=1Σi=1n1Si                                                                           ……………………….. (1)

in which

k= constant of proportionality

Si=permissible limit for the ith parameter

n= number of parameters.

2- Calculating of weightage of ith parameter (relative weight) Wi by using Eq.(2) as shown in Table (3):

Wi=kSi                                                                                     …………………….. (2)

3- Calculating sub index of ith parameter Qi by the expression:

Qi=100ViSi                                                                               .….………………… (3)

in which Vi is the monitored value of the ith parameter.

4-Calculating water quality index WQI as follows:

WQI=Σi=1nQiWiΣi=1nWi                                                                       ……………………. (4)

On the basis of the WQI, the quality of the water is categorized into five statuses from unfit for drinking purpose to excellent (Mishra and Patel, 2001) as shown in Table (4).

Table (2). The Iraqi standard for drinking purpose.

| Parameters | pH | TH | Ca+2 | Mg+2 | HCO3- | CL- | TDS | K+ | Na+ | SO4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Iraqi Standards, No. 417 | 6.5-8.5 | 500 | 200 | 150 | 200 | 600 | 1500 | 12 | 200 | 400 |

Table (3). Relative weight for each parameter.

| Chemical parameters | Highest permitted value for water(Si) | 1/Si | k | Wi |
| --- | --- | --- | --- | --- |
| pH | 8.5 | 0.117647 | 4.357671 | 0.512667 |
| Hardness | 500 | 0.002 |  | 0.008715 |
| Calcium | 200 | 0.005 |  | 0.021788 |
| Magnesium | 150 | 0.006667 |  | 0.029051 |
| Bicarbonate | 200 | 0.005 |  | 0.021788 |
| Chloride | 600 | 0.001667 |  | 0.007263 |
| TDS | 1500 | 0.000667 |  | 0.002905 |
| Potassium | 12 | 0.083333 |  | 0.363139 |
| Sodium | 200 | 0.005 |  | 0.021788 |
| Sulphate | 400 | 0.0025 |  | 0.010894 |
| Total |  | 0.22948 |  | 1.0 |

Table (4). Status of water quality based on WQI (Mishra and Patel, 2001).

| WQI value | 0-25 | 26-50 | 51-75 | 76-100 | >100 |
| --- | --- | --- | --- | --- | --- |
| Water quality statuses | Excellent | Good | Poor | Very poor | Unfit for drinking purpose |

Results and Discussion

The physicochemical parameters concentrations in the groundwater samples of the study area were presented in Table (5).

Table (5). Values of some water quality parameters of Dibdiba aquifer.

| well No. | PH | T.D.S mg/L | TH mg/L | Ca mg/L | Mg mg/L | K mg/L | Na mg/L | CL mg/L | SO4 mg/L | HCO3 mg/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7.32 | 3748 | 2527 | 618.8 | 91.74 | 57 | 595 | 978.4 | 2038 | 67.4 |
| 2 | 7.12 | 4320 | 2545 | 628.4 | 90.76 | 74.2 | 839 | 1227 | 1513 | 63.5 |
| 3 | 7.25 | 3962 | 2236 | 634.8 | 54.65 | 69.4 | 657 | 1049 | 1977 | 59.5 |
| 4 | 7.37 | 4320 | 2342 | 609.2 | 74.17 | 75.3 | 696 | 1985 | 2178 | 79.3 |
| 5 | 7.35 | 3605 | 2183 | 641.2 | 46.84 | 69.3 | 499 | 850.8 | 1958 | 59.5 |
| 6 | 7.4 | 3962 | 2166 | 593.1 | 59.53 | 64.9 | 472 | 779.9 | 1965 | 51.6 |
| 7 | 7.45 | 2887 | 2043 | 657.3 | 26.35 | 65.4 | 447 | 602.7 | 1840 | 39.7 |
| 8 | 7.35 | 2888 | 1955 | 471.3 | 73.2 | 57.6 | 442 | 623.9 | 2113 | 59.5 |
| 9 | 7.46 | 2760 | 2157 | 705.4 | 24.4 | 53 | 429 | 588.5 | 1406 | 71.4 |
| 10 | 7.25 | 2825 | 2175 | 737.4 | 16.59 | 60.1 | 463 | 850.8 | 2009 | 63.5 |
| 11 | 7.35 | 2887 | 2228 | 657.3 | 64.84 | 52.9 | 363 | 687.7 | 1830 | 47.6 |
| 12 | 7.15 | 2379 | 1955 | 474.5 | 72.22 | 63.7 | 338 | 609.7 | 1769 | 67.4 |
| 13 | 7.35 | 2569 | 2104 | 474.5 | 88.81 | 53 | 381 | 765.7 | 1890 | 43.6 |
| 14 | 7.23 | 2315 | 2087 | 522.6 | 67.34 | 39.7 | 341 | 673.6 | 1682 | 55.5 |
| 15 | 7.2 | 2728 | 2016 | 529.1 | 62.46 | 45.3 | 296 | 623.9 | 1720 | 47.6 |
| 16 | 7.23 | 2932 | 2113 | 481 | 87.84 | 50 | 226 | 531.8 | 1704 | 39.7 |
| 17 | 7.26 | 2728 | 1189 | 401 | 20 | 72.4 | 219.6 | 851 | 1769 | 0 |
| 18 | 7.37 | 2931 | 1160 | 348.7 | 47.6 | 79.7 | 268.2 | 716.5 | 1852 | 0 |
| 19 | 7.36 | 4320 | 1755 | 456.8 | 80.76 | 119.7 | 668.9 | 1146 | 2182 | 83.3 |
| 20 | 7.48 | 3533 | 1698 | 403.5 | 80 | 107.4 | 366.3 | 882.7 | 2008 | 67.4 |

In this study EXCEL is used to describe statistics of water quality parameters (median, standard deviation, minimum and maximum) for all studied wells as shown in Table (6).

Table (6). Summary of basic statistics for different water quality parameters (units are in mg/L except for pH which has no units).

| Chemical parameters | Mean | Median | Std. Deviation | Minimum | Maximum |
| --- | --- | --- | --- | --- | --- |
| pH | 7.32 | 7.35 | 0.1 | 7.12 | 7.48 |
| Hardness | 2031.7 | 2108.5 | 358.9 | 1160 | 2545 |
| Calcium | 522.25 | 561.1 | 110.1 | 348.7 | 737.4 |
| Magnesium | 61.55 | 66.09 | 24.27 | 16.59 | 91.74 |
| Bicarbonate | 53.34 | 59.48 | 21.81 | 0 | 83.28 |
| Chloride | 851.23 | 772.8 | 328.73 | 531.8 | 1985 |
| TDS | 3229.95 | 2909.5 | 670.99 | 2315 | 4320 |
| Potassium | 66.5 | 64.3 | 19.3 | 39.7 | 119.7 |
| Sodium | 450.35 | 435.5 | 167.17 | 219.6 | 839 |
| Sulphate | 1870.15 | 1871 | 204.92 | 1406 | 2182 |

Parameters

1.Total Dissolved Solids (TDS)

The groundwater is gaining various compositions from feeding area and access to discharge area due to exposure to multiple processes cause a change in chemical composition, such as evaporation, increase the concentration of elements and conveyed through the percolating water, and mixing processes and mitigation.

The concentration of TDS ranged from (2315) mg/L to (4320) mg/L as shown in Fig. (2). Increasing in TDS concentration may be attributed to the geological formation for Dibdiba aquifer (Gypsum soil, sandstone, and silty clay stone). Al-Jiburi H. K., (2002) also reported high concentration of TDS for the study area.

Very high values of TDS were recorded at all the wells much beyond permissible Iraqi drinking water standards limit. According to the Iraqi standard for drinking purpose, water containing more than 1500 mg/L of TDS is not considered desirable for drinking water supply.

Fig. (2).TDS values for all wells.

2. Cations Concentrations

The ranges of Ca+2, Mg+2, Na+, and K+ were (348.7 to 737.4) mg/L, (16.59 to 91.74) mg/L, (219.6 to 839) mg/L, and (39.7 to 119.7) mg/L respectively as shown in Figures (3), (4), (5), and (6). High and Low concentrations of calcium ion are observed in Wells (10) and (18) respectively due to geological formation of the study area. Fig.(4) shows that the concentration of magnesium ion is low according to the Iraqi standard for drinking purpose that refer to nature of geological layers in this region formation from (claystone) (Al-Jiburi H. K.,2002). Sodium ions (Na+1) was observed in lower concentration in the groundwater through the wells (16 and 17) lies in the east part of Dibdiba formation as shown in Fig. (5). Sodium ion concentrations are exceeded limits the Iraqi standard for drinking purpose. This result agreed with the results of previous studies of Jiburi H. K., 2002. Potassium ions (K+1) are results from the dissolve of the minerals which are available in rocks and the using of chemical fertilizers near the study area. Potassium ions were observed greater than the allowable concentration in all wells as shown in Fig. (6).

Fig. (3). The values of calcium ions for all wells.

Fig. (4). The values of magnesium ions for all wells.

Fig. (5). The values of sodium ions for all wells.

Fig. (6). The values of potassium ions for all wells.

3. Anions Concentrations

The ranges of anions concentration in the study area groundwater for sulphate (SO4-2), chloride (Cl-), and (HCO3-) were (1406 to 2182), (531.8 to 1985), and (0 to 83.28) mg/L respectively as shown in Figures (7), (8), and (9). The sulphate concentration in groundwater is attributed to the dissolving of the gypsum by surface water which is seeping to the groundwater and also due to dissolving the sulfur oxides by rainwater. Fig. (7) showed the high values of SO4-2 were recorded at all the wells much beyond permissible Iraqi drinking water standards limit, water for all wells containing more than 400 mg/L of SO4-2. This result agrees with the results of Al-Jiburi H. K., 2002. Fig. (8) shows that chlorides ions concentrations are greater than the allowable concentration in most wells. That is because various sedimentary nature for geological layers in this region. Dibdiba aquifer was observed low concentration of bicarbonate as shown in Fig. (9), this may be attributed to the nature of geological layer for this formation. Bicarbonate ions were observed lower than the allowable concentration in all wells.

Fig. (7). The values of sulphate ions for all wells

Fig. (8). The values of chloride ions for all wells.

Fig. (9). The values of bicarbonate ions for all wells.

4. Hardness

The range of Hardness concentration was (1160) mg/L in well (18) to (2545) mg/L in well (2) as shown in Fig. (10). The hardness is attributed to the effects of the two ions, calcium and magnesium. Therefore, the geological formation (Gypsum, Claystone) in study area was the main causes for rising in hardness concentration.

Fig. (10). The values of hardness for all wells.

5. pH

The pH is one of the most important factors that serve as an index for the pollution. The pH values of the study area groundwater ranged from (7.12 to 7.48) as shown in Fig. (11). The results showed that the groundwater tend to be neutralized or little alkaline.

Fig. (11). The values of pH for all wells.

Water Quality Index

The results of groundwater classification for 20 wells in the study area using WQI model are shown Fig. (12). WQI values at all wells are much above 100 indicating unsuitability of wells water for drinking purpose. According to Mishra and Patel (2001) index value more than 100 or above is unsuitable for drinking and propagation of wildlife and fish culture.

WQI values for the study area ranged from 432.6 for well (19) to 184.5 for well (14). The prime causes of deterioration groundwater quality are increasing in concentration of TDS, Hardness, and SO4-2 in water of wells.

Pervious studies of state company of geological survey and mining showed that the movement direction of groundwater flow to the study area is from west to east and from southwest to northeast (Ayob M.S. et al., 1995 and Al-Jiburi H. K., 2002). This movement leads to increased pollution of groundwater due to dissolution of rocks with various chemical composition during the distance traveled until reaching to the locations of the selected wells. The other reason for high WQI values were continuous discharge of agricultural runoff.

Fig. (12). The WQI values and their classifications.

Conclusions

According to the results of the WQI values, the following conclusions were found:

- The WQI for the groundwater samples of Dibdiba aquifer (Kerbala city) ranges from 432.6 mg/L to 184.5 mg/L.

- WQI values at all wells are much above 100 indicating unsuitability of wells water for drinking purpose.

- From WQI values, it is suggested that further improvement is required to treat the Dibdiba aquifer wells for using as drinking purpose.

- The prime causes of deterioration groundwater quality are TDS, hardness, and sulphate SO4.

Acknowledgements

Thanks and gratitude to Mr. Majid Hammed Tlefeh, computer center in technical Institute/Kerbala and Mr. Dalil Majid Hameed, central laboratory of the Agriculture directorate of Kerbala, for their assistance in providing facilities for the completion of this study.

References

Al-Jiburi H.K., 2002. Hydrological and hydrochemical study of Kerbala. Quadrangle Sheet (NI-38-14), State company of geological survey and mining, Iraq.

APHA: AWWA: WEF 1998. Standard Method for the Examination of Water and Wastewater. 20th ed., Published by American Public Health Association, Washington, DC 20005-2605.

Ayob M.S., Ismail S. K., and Ali B. M., 1995. Project to reduce groundwater levels in Kerbala city, AL-Furat center for irrigation studies. State for the operation of irrigation projects, Ministry of water resources, Iraq.

Brown R. M., McCleiland N. J., Deininger R. A., O’Connor M. F. 1972. A water quality index - crossing the psychological barrier (Jenkis, S.H. ed.). Proc. Int. Conf. Water Poll. Res. Jerusalem, 6: 787-797.

Brown, R. M., McClelland, N. I., Deininger, R. A., and Tozer, R. G. 1970. A water quality index: Do we dare? Water & Sewage Works, 117, 339–343.

Couillard, D., & Lefebvre, Y. 1985. Analysis of water quality indices. Journal of Environmental Management, 21, 161–179.

Dalkey, N. C., 1968. An objective water quality index. The Rand Corporation Harkins. Journal - Water Pollution Control Federation, 46, 588–591.

Dhirendra M. J., Alok K., and Namita A., 2009. Studies on Physicochemical Parameters to Assess the Water Quality of River Ganga for Drinking Purpose in Haridwar District. Rasayan Journanal of Chemical, Vol.2, No.1 (2009), 195-203, ISSN: 0974-1496.

Ghazi, Amer A. 2001, Methods of Protection and improvement of factories environmental. Printing in Dar Al-Harf Al- Arabi, Baghdad, p.p. 316.

Hamdy A., Trisorio Liuzzi G., 2005. Water scarcity management towards food security in the Middle-East region. Proceedings of the LNCV (Landau Network-Centro Volta) International Forum on “Food Security under Water Scarcity in the Middle East: Problems and Solutions”. November, 24-27, 2004, Villa Olmo, Como (Italy). Options méditerranéennes, Series A: Mediterranean Seminars, Number 65, 2005. ISBN: 2-85352-316-0.

Harkins, R. D. 1974. An objective water quality index. Journal–Water Pollution Control Federation, 46,588–591.

Horton, R. K. 1965. An index number system for rating water quality. Journal–Water Pollution Control Federation, 37, 300–305.

House, M. A., & Ellis, J. B. (1987). The development of water quality indices for operational management. Journal of the Water Science and Technology, 19(9), 145–154.

Inhaber, M. 1975. An approach to a water quality index for Canada. Water Resources, 9, 821–833.

Liebman, H. 1969. Atlas of water quality methods and practical conditions. Munich: R. Oldenburg.

Mishra, P.C., and R.K. Patel. 2001. Quality of drinking water in Rourkela, Outside the steel township. J. Env. And Poll. 8:2, 165-169.

O’Connor, M. F. 1972. The application of multi attributive scaling procedures of indices of value. Ann Arbor: Engineering Psychology Laboratory of the University of Michigan.

Prati, L., Pavanello, R., & Pesarin, F. 1971. Assessment of surface water quality by a single index of pollution. Water Resources, 5, 741–751.

Provencher, M., & Lamontagne, J. (1979). A method for establishing a water quality index for different uses. Quebec: Gouvernment du Quea’ bec, Ministea’ re des richesses naturelles, le Service de la qualitea’ des eaux, Bibliotequea’ nationale du Quea’ bec.

Ramakrishnalah, C.R; Sadas hivalah C. and Ranganna G 2009. Assessment of water quality index for the groundwater in Tumkur Taluk , Karnataka state , India. E-Journal of chemistry 6(2), 523-530.

Scottish Development Department 1975. Towards cleaner water. Edinburgh: HMSO, Report of a River Pollution Survey of Scotland.

Shaefer, D. J., & Janardan, K. G. 1977. Communicating environmental information to the public a new water quality index. The Journal of Environmental Education, 8, 18–26.

Walski, T. M., & Parker, F. L. 1974. Consumers water quality index. Journal of the Environmental Engineering Division, 100, 259–611.

Wu Jianhau, Li Peiyue and Qian Hui, 2011.roundwater Quality in Jingyuan County,a Semi-Humid Area in Northwest China. E-Journal of Chemistry http://www.e-journals.net, 8(2), 787-793, ISSN: 0973-4945.