Deriving And Calculation Of Point Spread Function For Array Of Obscured Circular Synthetic Apertures

Ban Hussein Ali AlrueshdyM , Adnan Falih Hassan Aldehadhawe

University of Kufa/ College of Sciences / Department of Physics

Abstract

This research includes the equation of point spread function (PSF) which has been derived for array of obscured circular synthetic apertures; it studies the effect of changing the obscuration ratios (ε =0, 0.25, 0.5, 0.75 ), MathCAD programs were used to simulate the equation of the point spread function, we obtain from the result increasing the intensity of secondary peaks and the best resolving power comparison with resolving power for obscured individual circular aperture.

الخلاصة

تضمن البحث اشتقاق معادلة دالة الانتشار النقطية لصف من الفتحات المركبة الدائرية المعاقة ودراسة تأثير تغير نسب الإعاقة =0,0.25,0.5,0.75) ε)  وذلك باستخدام برنامج الماثكاد , ووجد من خلال النتائج حصول زيادة في شدة القمم الثانوية وقدرة التحليل أفضل مقارنتاً مع قدرة تحليل الفتحة الدائرية المنفردة المعاقة.

Introduction

Most optical systems are used to create images: eyes, cameras, microscopes, telescopes ,etc. These image forming instruments use  lenses or mirrors whose properties, in terms of geometrical optics(A. Lipson et al.,2011). The image of a point source object formed by a lens system is known as the point spread function (PSF) of the lens, it is  one of the most complete function for describing the performance of an optical system and can be extended to include the effects of an obstructed aperture, apodization and any factor external to the optical system(A.F. Aljebory, 2004). The properties of the optical system can be described by a point spread function, point spread function is related to the phase at these exit pupil apertures through a Fourier transform (K.K. Sharma, 2004). A PSF is an irradiance distribution representing the image of an ideal point source , namely a PSF is a diffraction limited intensity distribution in response to a point source such as a distant star (Soon-Jo Chung,2002). Diffraction plays an important role in optical systems (R. Murugeshan, 2008). An imperfection or distortion in an image is called an aberration, an aberration can be produced by a flaw in a lens or mirror, but even with a perfect optical surface some degree of aberration is unavoidable (B. Crowell,2007). The method adapted to seeing the close objects as separate objects is called resolution. The ability of an optical instrument to produce distinctly separate images of two objects located very close to each other is called its resolving power(N. Subrahmanyam et al., 2009; R. Murugeshan, 2008). Synthetic aperture techniques are commonly used to obtain high resolution from data acquired using low resolution sensors, these techniques are commonly used in modern sonar and radar systems, being designated by Synthetic Aperture Sonar (SAS) and by Synthetic Aperture Radar (SAR) systems, respectively; this kind of systems is presently used in civil and military applications (P. Marques et al.,2004). " Synthetic Aperture " Can be defined as a  structure to separate optical systems of large individual aperture function sometimes called "  Mosaic" or  "Segmented Mirror", Synthetic aperture is an image system for independent optical system which are together sharing the image domain(A.F. Aljebory, 2004).

Deriving the Equation of Point Spread Function for Array of Obscured Circular Synthetic Apertures

The representation of the array of circular synthetic apertures with radius (R) inside it array of obscured circular synthetic apertures with radius (ɛ R)as shown in Figure (1)

.

(a) Obscured Individual Circular Aperture

(b)Obscured Synthetic Aperture N=4,                when N is the Number of Obscured Aperture

Figure (1) The Integral Boundary for Array of Obscured Circular Synthetic Apertures

The ratio between the radius of obscured circular synthetic apertures and the radius of circular synthetic apertures is called obscuration ratio and it is symbolized (ɛ) (G.C. Sloon et al., 2003; G.S. Alshabaan,2001; I.M. Alkhafaji,2011)

The equation for array of circular synthetic apertures is given by the relation (see Figure(1)):

x2+y2=(RN)2 ..…………………….………………….……(1)

and when R=1 , then:

x=±1N−y2         and             y=±1N    ……………………………(2)

The equation for array of obscured circular synthetic apertures is given by the relation

x+y=(εRN)2………………………………………………….. (3)

x'=±ε2N−y   and	         y'=±εN………………………………(4)

The intensity in point spread function for array of obscured circular synthetic apertures, can be calculated by the relation (G.C. Sloon et al., 2003).

PSF=F2=F1−F22………………………………………………..(5)

where

F1: is the complex amplitude function for array of circular apertures.

F2: is the complex amplitude function for array of obscured circular apertures.

We can express the complex amplitude in the point (u',v')in the image plane by using Fourier transform to pupil function( B.J. Davis et al.,2004; G.A. Cirino et al.,2006; J.W. Goodman,1998; K.J. Gasvik,2002; S. Wang et al.,2008).

f(x',y')=τ(x',y')eikw(x'.y') …………………………………………………..(6)

where

τ(x',y'): represents the real amplitude distribution in exit pupil and which is called " pupil transparency " or " transmission function " and often chooses equal one unit .

eikw(x',y'): wave front of aberration function.

w(x',y')  : aberration factor

(x',y')   : exit pupil coordinates (G. Chartier, 2005; M. Bass et al.,2010).

We can express point spread function for circular aperture in its integral form, by

putting the exit pupil coordinates of form(x,y) instead of (x',y')  , for simplicity .

F1(u',v')=n.fyxf(x,y)ei2π(u'x+v'y)dxdy    …………………………..(7)

and for  N  from obscured circular apertures (Figure(2)), we get:

∵x=x'+xjy=y'+yj  …………………………………………………….…(8)

y

Figure (2) Multiple – Obscured Aperture Configuration

F1(u',v')=n.fj=1Nyxf(x,y)ei2π[u'(x'+xj)+v'(y'+yj)]dxdy  …………………...……..(9)

F1(u',v')=n.fj=1Nyxf(x,y)ei2π(u'x'+v'y').ei2π(u'xj+v'yj)dxdy ………………………(10)

F1(u',v')=n.fyxf(x,y)ei2π(u'x'+v'y')dxdy.j=1Nei2π(u'xj+v'yj)  ………………………..(11)

By substituting the equation (6) in equation (11), we obtain:

F1(u',v')=n.fyxτ(x,y)eikw(x,y).ei2π(u'x'+v'y')dxdy.j=1Nei2π(u'xj+v'yj)    ……………..…(12)

Let     τ(x,y)=1

F1(u',v')=n.fyxeikw(x,y).ei2π(u'x'+v'y')dxdy.j=1Nei2π(u'xj+v'yj)        …..………………..(13)

where   k=2πλ

F1(u',v')=n.fyxei2π[w(x,y)+(u'x'+v'y')]dxdy.j=1Nei2π(u'xj+v'yj)     …………………….(14)

By substituting the integral boundary from equation (2) the area of synthetic aperture includes

F1(u',v')=n.f−1N1N−1N−y21N−y2ei2π[w(x,y)+(u'x'+v'y')]dxdy.j=1Nei2π(u'xj+v'yj)    …………………(15)

where       eiθ=cosθ+isinθ

F1(u',v')=n.f−1N1N−1N−y21N−y2cos2πw(x,y)+u'x'+v'y'+isin2πw(x,y)+u'x'+v'y'dxdy.j=1Ncos2π(u'xj+v'yj)+isin2π(u'xj+v'yj)..............................................................(16)

Let     z'=2πu'  and    m'=2πv'     become :

F1(z',m')=n.f−1N1N−1N−y21N−y2cos2πw(x,y)+z'x'+m'y'+isin2πw(x,y)+z'x'+m'y'dxdy.j=1Ncosz'xj+m'yj+isinz'xj+m'yj.........................................................................(17)

The intensity distribution on the two axes (z',m')is symmetric, so; we can reduce it to one axis only; let (m'=0), then equation (17) will take a new form .

F1(z')=n.f−1N1N−1N−y21N−y2cos2πw(x,y)+z'x'+isin2πw(x,y)+z'x'dxdy.j=1Ncos(z'xj)+isin(z'xj).....................................................................................(18)

Equation (18) represents the complex amplitude for array of circular synthetic apertures .

The complex amplitude function for array of obscured circular apertures from equation (6) can be found as:

F2(u',v')=n.fyxf(x',y')ei2π(u'x'+v'y')dx'dy'    ………………………………….(19)

By using the same steps and substituting the integral boundary from the equation (4) we obtain .

F2(z')=n.f−εNεN−ε2N−y2ε2N−y2cos2πw(x',y')+z'x'+isin2πw(x',y')+z'x'dx'dy'.j=1Ncos(z'xj)+isin(z'xj)..........................................................................................(20)

By using the physical conception of the equation (5), we can obtain the following:

PSF=n.falignl1N

From the relation x+iy2=x2+y2 the equation above become:

PSF=n.falignl−1N1N−1N−y21N−y2cos2πw(x,y)+z'x'dxdy.j=1Neiz'xj−−εNεN−ε2N−y2ε2N−y2cos2πw(x',y')+z'x'dx'dy'.j=1Neiz'xj+−1N1N−1N−y21N−y2sin2πw(x,y)+z'x'dxdy.j=1Neiz'xj

This equation represents the point spread function for array of obscured circular synthetic apertures.

By substituting the value of normalizing factor (1(π−ε2π)2)(B.H. Alrueshd ,2011).in equation (21),

we obtain:

PSF=1(π−ε2π)2−1N1N−1N−y21N−y2cos(z'x')dxdy.j=1Neiz'xj−−εNεN−ε2N−y2ε2N−y2cos(z'x')dx'dy'.j=1Neiz'xj+−1N1N−1N−y21N−y2sin(z'x')dxdy.j=1Neiz'xj

Equation (23) represents the point spread function for array of obscured circular synthetic apertures with diffraction – limited system .

Figure(2)Point Spread Function When (w20=0,R=1, ε=0.25)

Figure(1)Point Spread Function When (w20=0, R=1, ε=0)

Figure(3)Point Spread Function When (w20=0, R=1, ε=0.5)

Figure(4)Point Spread Function When (w20=0, R=1, ε=0.75)

Discussion

Through out noticing the results that we have obtained from the MathCAD program for the point spread function(PSF) for array of obscured  circular synthetic apertures, which has the circular synthetic apertures (N=1,5 and 6) with different obscuration ratios (ɛ=0,0.25,0.5 and 0.75), for diffraction limited system and the separating distances (R=1) (where R represents distance from the center of synthetic aperture to the center of Cartesian coordinates), we can conclude that : By using the obscured circular apertures system the one part transfers from central spot energy (Airy disk) to secondary peaks in diffraction pattern. The number of secondary peaks increase with increasing the obscuration ratio (ɛ), and the resolving power for obscured aperture with obscuration ratio ɛ=0.75, is better than resolving power with obscuration ratio ɛ=0.5 and ɛ=0.25. This is shown in Figures(1),(2),(3) and(4), which represent the values of PSF for different values of obscuration ratios (ɛ=0,0.25,0.5 and 0.75) for diffraction limited system having individual circular aperture. We notice from the Figures above, with increasing the obscuration ratio increasing the secondary peaks with clear shape, because increase the diffraction ratio of the light rays, and increment of deviation of the light passing through it and appearance the another secondary peaks as result. Figures (1),(2),(3)and (4) represent the intensity distribution curves of PSF for diffraction limited system with obscuration ratios (ε=0,0.25,0.5 and 0.75), when the number of apertures (N=5 and 6). We notice that by increasing the number of apertures we see the positions of secondary peaks were moved inside (near the PSF axis), This means increasing the intensity of secondary peaks. We notice from the Figures above an increase in the resolving power with increasing the obscuration ratio which is equivalent with increment for the number of apertures which was became best from the obscured individual aperture.

The cause of  increase in resolving power when we use a design having the obscured circular synthetic apertures is the presence of obscuration in circular synthetic apertures which in turn causes the diffraction of the light passing through it and when increasing obscuration ratio. The diffraction will increase which means redistribution of the intensity in the image plane.

The intensity for PSF in this research (array of obscured circular synthetic apertures) represents the intensity distribution of an array of similarly oriented identical  circular aperture equal intensity distribution of an individual aperture function multiplied by intensity sum result from a set of point sources arrayed in the same configuration.

References

Aljebory; A.F.H (2004), PH.D. thesis , " Improvement Resolving Power of Optical System Design by Using Circular Synthetic Aperture", Almustansiriya University .

Alkhafaji I.M.; (2011), MSC thesis , " Calculating the Point Spread Function of  Circular Aperture with Moving Circular Obscuration",  Alkufa University.

Alrueshd B.H.; (2011), MSC thesis , " Performance Enhancement of the Optical System    by Using the Obscured Circular Synthetic Apertures" , Alkufa University.

Alshabaan ; G.S.  (2001), PH.D. thesis, " Calculate the Total Lighting in the Image of Point Object", Almustansiriya University.

Bass , M. V.N. Mahajan ; (2010),"  Handbook of Optics ", Optical Society of America, Vol. 1 ,3th edition.  , USA.

Chartier; G. (2005),  " Introduction of Optics " ,  USA .

Cirino , G.A R. Barcellos , L.G. Neto; (2006),  "Annals of Optics ".

Crowell B; (2007), " Optics Light and Matter " , 2nd edition.

Davis , B.J.  W.C. Karl, A.K. Swan , M.S. Ünlü , B.B. Goldbery; 23  August. (2004), "Capabilities and Limitations of Pupil-Plane Filters for Supersolution and Image Enhancement " , optics express 4150, Vol. 12 , No. 17 , USA.

Gasvik K.J.; (2002), " Optical Metrology " , 3th edition.

Goodman ; J.W.  (1998), " Introduction to Fourier Optics  ", 2nd edition.

K.K. Sharma ; (2006),  " Optics Principles and Applications " , USA.

Lipson A., S.G. Lipson , H. Lipson ; (2011), " Optical Physics " , 4th edition , USA.

Marques , P. I. Dias , E. Fernandes; (2004), "An Experimental Synthetic Aperture Sonar Platform " .

Murugeshan R.; (2008), "Optics and Spectroscopy", 6th edition, India.

Sloon , G.C.  P.S. Nerenberg , M.R. Russell;  21 July (2003), " Infrared Spectrograph Technical Report Series ", IRS – TR 03001 : The Effect of Spectral  Pointing – Induced Throughput Error on Data from the IRS.

Soon - Jo Chung; (2002), MSC thesis, " Design Implementation and control of a sparse aperture imaging Satellite " ,Korea.

Subrahmanyam N., B. Lal, M.N. Avadhanulu ; (2009), "A Text Book of Optics " ,India.

Wang , S.  C. Rao , W. Jiang ; 11, 20 June (2008), " Diffraction-Limited Performance and Aberration Tolerance of a Sparse Optical Synthetic Aperture Imaging System with Four Sub-Aperture ", Journal of Modren optics , Vol. 55, No , 1771 – 1785.