Chemical analysis of Urinary Calculi using local kit

Sawsan M. Ali    ,     Aiman H. kreem.

University of Babylon /collage of Agriculture

Mooaid Selman Hassan,

Vaccine and sera institute/ministry of health

- Abstract

A better understanding of chemical principles underlying the formation of calculus has led to a need for more precise information on the chemical composition of stones. A qualitative procedure for the chemical analysis of urinary calculi which is suitable for routine use is presented. The procedure involves five simple qualitative tests These data are used to identified the composition of the stone in terms of calcium oxalate, and ammonium phosphate..We tested and identify the mineral composition of urinary stones by simple diagnostic kit locally invented and approved by vaccine and sera instituted. Our study has been shown to provide exceptionally high quality imaging of the fine structure of urinary calculi.

الخلاصة

إن حصى الكلى يعانى منها الكثير  من المرضى, إن تحديد نوع الحصى والتركيب الكيمياوى ذو أهمــــــــــية كبيرة للطبيب المعالج وللمريض نفسه, إذ على ضوء ذلك يتم تحديد نوع العلاج المستخدم ونوع الغذاء الواجب إن يتناوله المريض . لمعرفة  المكونات الكيماوية لحصى الكلى يتطلب ذلك استخدام طريقة دقيقة لتحديد تلك المكونات ,قمنا بتصنيع

عدة تشخيص كيماوية وهى ملائمة للاستخدام المختبرى الروتيني ,لتحديد التركيب الكيميائي لحصى الكلى والتي تستعمل للكشف عن الكربونات ,الامونيا ,الاوكزالات ,الفوسفات ,الكالسيوم ,حامض اليوريك باستعمال محاليل كيميائية قياسية,أظهرت هذه العدة التشخيصية كفاءة عالية لتحديد المكونات الكيميائية المذكورة .

Introduction

Urinary stones are crystals, mainly of oxalate, and /or phosphate and urate formed in the kidney. Statistical studies showed that men with ages in the range of 20 to 40 have the highest risk. Most stones are formed due to dietary factors, like the intake of high dairy products or salts which would increase the amount of Ca in the urine. Low intake of water would increase the percentages of stones in the urine(Pearle et al 2005,Delvecchio et al 2003,Menon et al 2002,Mandel N. 1997,ED Edelson). Genetic effects, intake of vitamin C may play also a roll in the formations of stones. Pain in the lower back below the ribs and blood in the urine indicate the presence of stone. Stones - smaller than 5mm- will usually pass through micturation. Clinical laboratory assessment of urinary stones is typically using chemical methods of analysis [Pearle et al 2005,GM,Assimos et al 1997,Chandhoke 2005) and is usually identify stones by their primary mineral content. Stones get classified as having multiple components, and describe the pattern of different minerals in heterogeneous stones. Thus, stones are commonly classified as being calcium oxalate ,calcium phosphate, or a combination of these two mineral crystals called (hypercalciuria), Struvite stones are made up of ammonia, and phosphate  and normally result from bacterial urinary infections. The bacteria produce urease, an enzyme that promotes the development of struvite crystals by making the urine alkaline. The most common bacteria associated with struvite stones is Proteus mirabilis. Uric acid stones form when there is too much acid in the urine. this type of stone is closely related to diet and most often affects people who consume large quantities of animal protein.Cystine stones are rare, accounting for only about 1 percent of all urinary stones.  When this information makes its way to the patient's chart, the individual may be classified simply (GM,Assimos 1997,Lotan et al 2004,Dellabella et al 2005). It has been determined that the vast majority of stones actually contain more than one type of mineral [Chandhoke 2002,Qiang et al    2004,Borghi et al 1996]. Knowing the mineral composition of a patient's stones has obvious value in determining a treatment plan(Klee et  al 1991,GRASESF ET AL 1990,krambeck et al 2006,Finkielstein et al 2006). The purpose of this study is to present simplified methodology for the analysis of urinary stone composition. In the present study, we observed to providing  chemical structure of human urinary stones and differentiate six common mineral constituents by using  local urinary calculi analysis kit .

- Material and method:

Twenty human urinary stones were received from Al Kadimea Hospital  and Al-Wedded Lab were examined at vaccine and sera instituted /research department for  the chemical composition   from 1998  to   2000 .  Analysis for mineral that was positively identified by chemical reagent, using Urinary calculi analysis kit such and such materials Nessler reagent, Saturated ammonium oxalate ,ammonium molybidate solution, 0.05N potassium permanganate,0.1 N sodium carbonate ,Phosphotungestic acid,  1N HCL , artificial chemical stone.Human urinary stones were obtained following analysis laboratory. A representative collection of urinary stones of pure and heterogeneous mineral content was selected for use in this study. Compositions of 20 stones used for identification. The analysis on stones were completed using standard artificial stone(artificial stone contain ammonia,Calcium,phosphate,oxalate,uric acid). Wash the stone with water and dry on a filter paper. Note the external    appearance(single, multiple, rough ,smooth,horned,or waxy).Crush stone  about ( 0.1 gm), then add 5ml of HCL,if babble appear  carbonate is positive, centrifuge ,then add 0.5 ml of supernated solution in each test tube using

(5 test tube).

The Method of analysis is summarized in table (1).

| Stone composition | Method for identification | Result |
| --- | --- | --- |
| 1.Ammonia | 0.5 ml Nessler reagent | Orange-brown |
| 2.calicium | 1 ml of Saturated ammonium oxalate | White precipitate |
| 3.phosphate | 1 ml Ammonium molybidate solution | Yellow precipitate |
| 4.oxalate | 0.01ml potassium permanganate | colorless |
| 5.Uric acid | 0.5 ml sodium carbonate,0.5 ml Phosphotungestic acid | Blue color |

The results of qualitative analyses are completed with 20 minute after receipt of the specimen. A descriptive report were including a gross description and chemical identification of the stones, result written as category.

- . Results:

urinary stones showed excellent resolution of structural detail that could be correlated with structure of pure mineral  artificial, values were collected for six minerals that could be found in that appeared to be pure, including uric acid  10 %, struvite 10-15%, calcium oxalate / calcium phosphate  75%,. Analysis of intact stones showed excellent resolution of structural detail and could discriminate multiple mineral types within heterogeneous.

Twenty stone were obtained from alkademia hospital  and  alweddad laboratory ,  result  show agreement  with artificial reference standard.

- The same stone analyzed by Iraqi reference laboratory using our kit and it was evaluated and pass analysis and obtain a certificate No 2690 at 2000.

Table (2) chemical composition of the analyzed stones.

| chemical composition of 20 stones, showing that there was a predominance of calcium oxalate and calcium phosphate in almost all the stones |  |
| --- | --- |
| Chemical Composition of Stones | Percentage % |
| Calcium oxalate/Calcium phosphate | 75 |
| ammonium phosphate | 10-15 |
| Uric acid | 10 |
| Mixed calcium, uric acid | 5-10% |

- Discussion:

Chemical kit yield excellent high resolution analysis of stone structure. It is a relatively fast method, taking approximately 20 minute for a complete urinary stone diagnosis. Establishing standards for complete stone analysis was obtained in the present study. A recent study showed that amounts small enough about o.1 gm can detected. The vast majority of stones contain more than one component [Chandhoke 2002], but many of these mixed stones show obvious separation of the different materials .chemical analysis kit is a simple reliable inexpensive diagnostic tool. Mineral content in stones could yield important information useful in determining proper treatment for the patient.

- Conclusions:

Analysis of urinary calculi is an essential step in the examination and initial treatment of the patient with kidney stones (urolithiasis). Knowledge of the composition of calculi yields fundamental information concerning the pathogenesis of the disease, including metabolic abnormalities, presence of infection, and even drug metabolism. Our report to the physician notes the weight, size, and shape of stones, the constituents of the stone, Six common stone minerals were found, which allows the identification of mineral types by using local chemical kit for stone analysis, The results of analysis confirm the mineralogical results.

Uric acid stone	   calcium-oxalate stone	calcium carbonate

Struvite  stone	uric acid stone                 calcium-oxalate stone

- References:

Borghi L, Meschi T, Amato F, Briganti A, Novarini A, Giannini A. 1996. Urinary volume, water and recurrences in idiopathic calcium nephrolithiasis: a 5-year randomized prospective study. J. Urol ., 155:839-43.

Chandhoke PS. 2002. When is medical prophylaxis cost-effective for recurrent calcium stones? J. Urol ., 168: 937-40.

Chandhoke PS. 2005. Metabolic abnormalities and the medical management of calcium oxalate nephrolithiasis .Minerva Urol.Nefrol. Mar ,57(1);9-16 .

Dellabella M, Milanese G,  Muzzonigro G.2005. Randomized trial  of the efficacy of tamsulosin, nifedipine and phloroglucinol in medical expulsive therapy for distal urethral calculi. J. Urol. July,174(1):167-72 .

Delvecchio FC, Preminger GM. 2003.Medical management of stone disease. Curr Opin Urol. 13:229-33. Ed ,Edelson. Kidney stone Shock Wave Treatment Boosts Diabetes, Hypertension Risk- Study suggests link, but doctors say it's too early to abandon this therapy", Health Finder, National Health Information Center. Finkielstein VA, Goldfarb DS. 2006. Strategies for preventing calcium oxalate stones . CMA J. May 9; 174(10):1407- 9 . GM, Assimos DG, Dretler SP, Kahn RI, Lingeman JE, et al. 1997.Urethral Stones Clinical Guidelines Panel summary report on the management of urethral calculi. J. Urol., 158:1915-21. Klee LW, Brito CG, Lingeman JE. 1991. The clinical implications of brushite calculi. J. Urol., 145: 715-71. GRASESF.,MILLANA.,CONTE,A.1990..Production of calcium oxalate monohydrate, dihy- drateortihydrate.Acomparitivestudy.Urol .Res.18:17-25. Krambeck AE. Gettman MT,Rohlinger AL,Lohse CM, Patterson DE, Segura JW.2006.Diabetes mellitus and hypertension associated with shock wave lithotripsy of renal and proximal urethral st0nes at 19 years of follow up .J. Urol.175 (5):1742- 7.

Lotan Y, Cadeddu JA, Roerhborn CG, Pak CY, Pearle MS. 2004. Cost-effectiveness of medical management strategies for nephrolithiasis. J. Urol ., 172(6 pt 1):2275-81.

Mandel N. 1996. Mechanism of stone formation.Semin Nephrology., 16:364-74.

Menon M, Resnick MI. 2002. Urinary Lithia sis: etiology, diagnosis, and medical management. In: Campbell MF, Walsh PC, Retik AB, eds. Campbell's Urology. 8th. ed. Philadelphia, Pa.: Saunders.

Pearle MS, Calhoun EA, Curhan GC. 2005. Urologic diseases in America project: urolithiasis. J. Urol.,173:848-57.

Qiang W, Ke Z. 2004.Water for preventing urinary calculi. Cochrane Database Syst .Rev. (3):CD004292.