Design a Classification System for  Brain Magnetic Resonance Image

Hussein  Attya  Lafta     Esraa  Abdullah Hussein

University  of  Babylon/ College  of  Computers and Sciences

Abstract

Automated  and  accurate classification  of  brain  MRI  is such important that  leads  us to present  a new  robust  classification  technique  for  analyzing  magnetic  resonance  images[Chris 2003].

In  this  work , the  proposed  method  consist of  three stages  collection of  images, feature extraction , and  classification . We are  used gray-level co-occurrence matrix (GLCM)  is  used  to extract  features  from  brain  MRI . These features  are  given as input  to  k-nearest  neighbor( K-NN) classifier  to classify images as normal  or  abnormal  brain  MRI .

Keywords

Brain MRI , Feature extraction , GLCM , K-NN .

ألخلاصة

التصنيف  الدقيق  لصور  رنين  الدماغ  مهم  مما  يعطينا  دافع  لتقديم  طريقه  تصنيف  قويه  ودقيقه  لتحليل  صور  رنين  المغناطيسي .في هذا البحث , الطريقة المقترحة تتكون من ثلاث مراحل  تجميع  الصور ,  استخلاص المميزات و التصنيف . استخدمنا طريقه  gray-level co-occurrence matrix (GLCM)لاستخلاص المميزات  من صور رنين الدماغ . هذه المميزات  تعطى كـ  إدخال إلى طريقه التصنيف K-NN  لتصنيف الصور إلى  صور دماغ طبيعي أو غير شاذ أو صور دماغ شاذ أو غير طبيعي.

1 . Introduction

Brain tumor  is any mass that  results  from abnormal growths  of  cells  in the brain . It  may  affect  any  person  at  almost  any age . Brain tumor  effects  may  not be  the same for  each  person , and  they  may  even  change  from  one  treatment session to  the  next. Brain  tumors can have a variety of  shapes and  sizes , it can appear  at  any  location  and  in  different  image  intensities . Brain  tumor  can  be malignant or  benign[Ahmed  2010] .

Many  procedure  and  diagnostic  imaging techniques  can  be performed  for  the early  detection  of  any  abnormal changes in  tissues  and  organs such as computed tomography  scan  and  magnetic  resonance  imaging (MRI) . Although  MRI  seems to  be  efficient  in  supplying  the  location  and  size of  tumors , it  is  unable  to classify  tumor  types [Ahmed  2010] .

In  medical  image  analysis , the determination of  tissue  type( normal or abnormal) and  classification of  tissue  abnormal are  performed  by using  texture . MR  image  texture  proved  to  be  useful  to  determine  tumor type [Ahmed  2010 ] .

In  this  work  classification  method  based  on  K-NN . Extraction  of  texture features  from  images  represents  the  information  about  the  texture  characteristics of  the  image and  the  variations in  the  intensity  or  gray level .

The paper is organized as follows: Related work  is  represented  in section 2. Details of  the  proposed  method  are  described  in  section 3.Section 4 contains details of  experimental results . Conclusion presented  in section 5 and  future work  is presented in section 6 .

2 . Related Works

In  2003, Chris  A.Cocosco , Alex  P. Zijdenbos  and  Alan  C. Evan

Presented  a  fully  automatic  and  robust  brain  tissue  classification  method  by  using  a pruning  strategy . Starting  from a set of  samples generated  from  prior  tissue  probability  maps (a ‘model’) in a standard, brain-based  coordinate  system, the  method  first  reduces the  fraction of  incorrectly  labeled  samples  in  this  set  by  using  a minimum spanning  tree  graph-theoretic  approach. Then, the  corrected  set  of samples  is  used  by  a supervised  K-NN  classifier  for  classifying  the entire  3D  image.

In  2006, Chaplot  S., Patnaik  L.M., and  Jagannathan  N.R.  introduced  a method  for  classification  of  magnetic  resonance  brain  images  by  using  wavelets transform  as a  tool  for  extraction  features  from  brain  MRI  images  based  on  GLCM  method  and  use  these  features  as  input  to  support  vector  machine  and  neural  network  classifier .

In  2009,  Jesmin  Nahar  , Kevin  S.Tickle, A B M  Shawkat  Ali  and  Yi-Ping  Phoebe  Chen  discussed  how  image  data  classification  plays a vital  role  in  detecting  cancer, they  proposed  a hybrid  approach  combining  both  microarray  data  and  image  data  for  early  detection  of  cancer.

In  2010, Ahmed  Kharrat  ,  Karim  Gasmi, Mohamed  Ben  Messaoud ,  Nacera  Benamrane  and  Mohamed  Abid  introduced  a hybrid  approach  for  classification  of  brain  MRI  tissues  based  on  genetic  algorithm(GA)  and  support  vector  machine (SVM).The  optimal  texture  features  are  extracted  from  brain  MRI  images  by  using  GLCM . These  features  are  given  as  input  to  the  SVM  classifier. The  choice  of  features  are  solved  by  using  GA.

In 2011, V.Sivakrithika  and  B.Shanthi  are  proposed  comparative  study  on  cancer  image  diagnosis  using  GLCM  to  extract  features  from  images  and  neuro-fuzzy  as  tool  for  classification  stage.

In  January ,  2012  Sahar  Jafarpour  , Zahra  Sedghi  and  Mehdi  Chehel  Amirani  proposed  method  for  classification  brain  MRI  images  they  used  GLCM  to  extract  features  from  brain  MRI  and  for  selecting  the  best  features ,PCA+LDA  is  implemented. The  classification  stage  is  based  on  artificial  neural  network(ANN)  and k-nearest  neighbor( K-NN) .

3 . Proposed Work

The methodology of the MRI brain image classification is as follow :

-    Collection of images .

-    Feature extraction .

-   Classification using K-NN .

The proposed system are implemented on a real human brain dataset. The input dataset consist in 19 images: 10 images are normal, 9 abnormal images. These normal and abnormal images used for classification, are 256×256 sizes and acquired at several positions of the transaxial planes. These images were collected from the Harvard Medical School website[Harvard] .

( A )

( B )

Figure (1) : Harvard dataset (A) normal (B) abnormal .

3.1 Feature  Extraction

Feature  extraction  means  identifying  the  characteristics  found  within  the image , these  characteristics are  used  to  describe  the  object . Image  features  are useful  extractable  attributes  of  images or  regions  within an image [Gose 2009] .

Feature  extraction  methodologies  analyze  objects and  images  to  extract  the features  that  are  representative of  the  various classes of  objects . Features  are used  as inputs  to  classifiers that  assign  them  to  the  class  that  they  represent . In this  work  grey-level co-occurrence matrix (GLCM)  features  are  extracted [Ahmed  2010] .

3.1.1  Grey-level co-occurrence matrix (GLCM)

A well-known  statistical  tool  for  extracting  second-order texture  information from  images  is  the grey-level co-occurrence . The GLCM  matrix is one of  the most popular  and  effective  sources of  features  in texture  analysis . For a region , defined  by  a user  specified  window , GLCM  is the  matrix  of  those measurements  over  all  grey  level  pairs [Acharya 2005] .

The GLCM  matrix of  an a image f(i,j) , containing  pixels  with  gray  levels {0,1,. . . , G-1 }, is a two-dimensional  matrix C(i,j) where  each element of  the matrix  represents  the  probability of  joint  occurrence of  intensity  levels i and j at a certain distance  d  and  an angle  θ . If  there  are  L brightness  values  possible  then the  GLCM  matrix  will  be  an L × L  matrix  of  numbers  relating  the  measured statistical  dependency  of  pixel  pairs . Generally , four  directions  corresponding  to angles  of   θ = 0 , 90 , 45 , 135  are used [Pratt 2007] . There  will be  one  GLCM matrix  for each  of  the  chosen  values of  d and θ [Acharya 2005][Pratt 2007] . Figure (2) shows  the four  directions of  this  texture  analysis  technique .

| 45 | 90 | 135 |
| --- | --- | --- |
| 0 | pixel |  |

Figure (2) : Four directions of  GLCM matrix .

This approach computes an intermediate  matrix of  statistical measures from an image . It  then  defines  features as functions of  this  matrix . These features  relate to texture  directionality , contrast , and  homogeneity on a perceptual  level . The  values of  a GLCM  matrix  contains  frequency  information  about  the  local  spatial distribution  of  gray  level  pairs . Various  statistics  derived  from  gray  level  spatial  dependence  matrices  for use  in  classifying  image  textures [Ritter 1996] . Figure (3) shows  an example of  a 5 × 5 image  and  its  GLCM  matrix  for  right

| 1 | 1 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 | 0 |
| 2 | 2 | 2 | 2 | 0 |
| 3 | 3 | 3 | 2 | 2 |
| 3 | 3 | 3 | 2 | 2 |

neighbors ( θ = 0 and d= 1) .

| J | i |
| --- | --- |

(a)

( b)

j

3    2     1     0

| 0 | 1 | 2 | 2 |
| --- | --- | --- | --- |
| 0 | 0 | 4 | 0 |
| 2 | 5 | 0 | 0 |
| 4 | 0 | 0 | 0 |

0

1

i

2

3

(c)

Figure (3) : (a) Template . (b) Original image . (c) GLCM .

Then  each GLCM  matrix will be  normalized  by dividing  each  element  in C(i,j)  by  the  total  number  of  pixel pairs , which  is  represented  as :

L-1

L-1

Cnorm (i,j) = C(i,j) / ∑   ∑    C(X,Y)

X=0

Y=0

Several  texture  measures are directly  computed  from  the  normalized  spatial gray  level dependence  matrix, Cnorm(i,j) . These  texture  measures are called textural  features. Using  the  normalized  GLCM  matrix(Cnorm(i,j)), the  texture features  are  computed  as follows [Gonzalez 2002] :

Max Probability :

F1= Max (Cnorm(I,J))

Entropy :

L-1

L-1

F2= - ∑   ∑  Cnorm(I,J) Log(Cnorm(I,J))

J=0

I=0

Contrast :

L-1

L-1

F3=  ∑   ∑  (I-J)2  Cnorm(I,J)

I=0

J=0

Inverse Difference Moment (IDM) :

L-1

L-1

F4= ∑  ∑  Cnorm(I,J) / 1+(I-J)2

I=0

J=0

Angular second moment :

L-1

L-1

F5= ∑  ∑  Cnorm(I,J)2

J=0

I=0

Mean :

L-1

L-1

F6= ∑  ∑  Cnorm(I,J) / L *L

I=0

J=0

Dissimilarity :

L-1

L-1

F7=  ∑   ∑  (|I-J| * Cnorm(I,J))

J=0

I=0

Homogeneity :

L-1

L-1

F8= ∑  ∑ Cnorm(I,J) / 1+|I-J|

I=0

J=0

3.2  K-Nearest Neighbor (K-NN)

The  K- nearest  neighbor  classifier  is  a simple  supervised  classifier that  has  yield  good  performance . This  classifier  computes  the distance  from  the  unlabeled  data  to  every  training  data  and  selects the  K  neighbor  with  shortest  distance . No  requirement  for  training process  makes  this  classifiers  implementation  simple . In t his  work , the  Euclidean  distance  is  used  for  distance  and  K=3 .

4 . Experimental  Results

Image  dataset  consist  of  19  images  including  normal  and abnormal  images . We  are  use 10 images (out  of 10 images 6 are normal  and  4  are  abnormal)  are  used  for  training  process, and  9 images(out  of  9 images 5  are  abnormal  and  4  are  normal)  for  test process . Features  are  extracted  from  the  images . Figure (4)

show  the feature  extracted  from  the  images .  The  extracted  features  are directly fed  to  K-NN  classifier .

Figure (4) : The feature extracted from images .

In  programming  for  simplicity  we  use " 0" to  indicate  for  images that  have  class  normal  and  " 1 "  to  indicate  for  images  that  have class  abnormal , the  results  in  test  stage  are  shown  in  figure(5) . The accuracy  of  the  proposed  system  is  computed  by using  the  equation as  follow :

Accuracy =( Number of successful classification / Total  number  of test  images) * 100% .

The accuracy of this system is 88 % .

Figure (5) : Results from test stage .

5 . Conclusion

In  this  work , we  are  proposed  a medical  decision  system  with  two  class  sets  as normal  and  abnormal. This  automatic  detection system  which  is  designed  by  gray-level co-occurrence matrix (GLCM) and  supervised  learning  method (K-NN)  obtain  promising  results  to assist  the  diagnosis  brain  disease .The  methodology  in  this  paper  is based  on  using  image  features  and  employing  K-NN  classifier  to distinguish  normal  and  abnormal  brain  MRI .The accuracy  of   the system  is  88% .

6 . Future Work Suggestions

The  suggestions  for  future work  are :

1 – Using  neural  network in  classification  phase .

2 – Perform  GLCM  matrices  in  eight  directions  0, 45 , 90 , 135 , 180 , 225 , 270 , and 315 .

3 – Using  one  of  the  enhancement  methods  to  decrease  image  noise.

4 -  The  accuracy  could  be  improved  by  including  more  number  of sample  images  in  dataset.

References

[Ahmed  2010]  Ahmed K., Karim G., Mohamed B. M. ,Nacera B. and Mohamed A.," A  hybrid  approach  for  classification  of  brain  MRI using  genetic  algorithm  and  support  vector  machine ,Issue 17, July-December  2010 P. 71-8 .

[Acharya  2005]   Acharya  T., and Ray A.  K., "Image  Processing Principles  And  Applications", Wiley-Interscience, USA,2005 .

[Chris  2003]     Chris  A.  Cocosco, Alex  P.  Zijdenbos   and   Alan  C.  Evans, "A Fully  Automatic  And  Robust Brain MRI  Tissue Classification  Method", Elsevier (2003)  513-527 .

[Gose  2009]    Gose  E., Johnsonbaugh  R., and  Jost  S., "Pattern Recognition  And  Image  Analysis" , New  Delhi  ,  2009  .

[Gonzalez  2002]    Gonzalez  R. C., and  Woods  R. E., "Digital  Image processing " , prentice  Hall , USA, Second  Edition , 2002 .

[Harvard]   Harvard  Medical  School, Web, data  available  at http://med.harvard.edu/AANLIB/.

[Pratt  2007]   Pratt  W. K., "Digital  Image  Processing ", Fourth  Edition ,Wiley-Interscience,USA,2007.

[Ritter  1996]    Ritter G. X., and  Wilson J. N., "Handbook  of  Computer Vision  Algorithms  In  Image  Algebra" , It  Knowledge , 1996 .