Data Acquisition of Greenhouse Using Arduino

Diaa Mehdi Faris      Mahmood Basil Mahmood

Technical college of Mosul/ Technical computer engineering department

DMF54@yahoo.com     tech_n2010@yahoo.com

Abstract

In this paper, the Design had been aimed data acquisition in  greenhouse for multiple sensors to use data for  simulation or processing to achieve the better enhancement of growth in greenhouse, this  data has  effect on the climate of  greenhouse. Graphical User Interfaces (GUI) had been used through LabVIEW, firmware of arduino as software and arduino board and sensors as hardware. by using arduino mega board provides multiple inputs analogs and I/O digitals to made read data sensor easy to take temperature, humidity, CO2 gas, also measuring the soil moisture that needed for irrigation plants and the intensity of  lights that applied for  greenhouse . These factors has the major  effect on increase growth plants. Greenhouse environments monitoring present difference  changes to parameters, the  system for this purpose had been provided  and given ability to control on climate of greenhouse.

Keywords: data acquisition, arduino mega, LabVIEW, temperature, humidly, sensors

الخلاصة

تم بحث التصميم  في اكتساب البيانات من مجموعة من المتحسسات الموزعة داخل البيوت الخضراء حيث تستخدم تلك البيانات في عمليات المحاكاة أو المعالجة للوصول إلى أفضل عملية أنماء داخل البيوت الخضراء من خلال توفر الظروف المناسب  .برنامج اللاب فيو استخدم للمحاكاة بالإضافة إلى البرنامج المسقط على الاردوينو  حيث إن استخدم كارت اردوينو ميجا يسهل ويوفر العديد من الإدخالات التناظرية بالإضافة إلى الإدخال والإخراج الرقمي والذي يجعل من قراءة الحرارة والرطوبة وباقي المتحسسات والتي له التأثير في أنبات النباتات من خلال مراقبة تلك المتغيرات حيت تعطي القدرة على السيطرة  الصناعية على المناخ في  البيوت الخضراء

الكلمات الدالة :  اكتساب البيانات, اردوينو ميج, لاب فيو, الحرارة, الرطوبة, المتحسسات .

# 1.Introduction

The crop agriculture in greenhouse is higher affected by the surrounding conditions. The significant environmental factors for the quality and better productivity of the plants growth are temperature, relative humidity, Lighting, moisture soil, and the CO2 amount in greenhouse. Continuous monitoring of these factors gives relevant information pertaining to the individual effects of the various factors towards obtaining maximum crop production [J. H. Shin et al., 1998].

Arduino is an open-source electronics prototyping platform based on flexible, easy-to-use hardware and software. It's intended for artists, designers, hobbyists, and anyone interested in creating interactive objects or environments. [David et al., 2007]. Arduino can sense the surroundings by receiving input signal from a variety of sensors and can affect its environment via controlling heater, Water pump, and other actuators. The AVR Atmega2560  on the board is programmed using the Arduino programming language (depended on Wiring) and the Arduino development environment (depended on processing). Arduino projects can be stand-alone or they can communicate with software running on a computer (e.g. Flash, Processing, MaxMSP)

A greenhouse is seen as a multivariable process presents a nonlinear nature and is influenced by biological processes [Herrero et al., 2007].The five most important parameters must be taken into consideration when design a greenhouse are temperature, relative humidity, ground water, illumination intensity and CO2 concentration. This parameters is important to realize that the five parameters mentioned above are nonlinear and extremely interdependent [Fourati  et al., 2007; Blasco et al., 2007; Putter and J. Gouws, 1996]. the computer control system for the greenhouse involves the series steps [Melrolho, 1999]:

1. Acquisition of data through sensors.

2. Processing of data, comparing it with desired states and finally deciding what must be done to change the state of system.

3. Actuation component carrying the necessary action.

This paper describes a solution to the first part of the system. The information is obtained from multi-sensors station and is transmitted through USB port to computer.

Mahmoud presented  an  method for  controller and monitoring, this research used microcontroller to transfer data via the RS232 port [MAHMOUD OMID, 2004].

H. Mirinejad et al. presented  an  approach for greenhouse and control system based on the (SCADA) tools, supervisory system designed and simulated to serve as a user-friendly interface with the operator[H. Mirinejad, 2008].

Anuj Kumar et.  al.  had been made an attempt to devise a DSP processor based environment monitoring system (EMS) to real time monitor the climatic parameters which, directly or indirectly, have a vital say in the growth of the greenhouse plant[Anuj Kumar et  al. , 2010].

Kiran Sahu et  al.  had been design a simple and easy to installing by used microcontroller-based circuit to monitor and record, to achieve  the maximum plant growth for greenhouse [Kiran et  al., 2012]

M. GUERBAOUI et al.  proposed a contribution to the development of greenhouse manufacture in Morocco. The suggest solution include the development of an controller system, a data acquisition card PCL-812PG controlled via personal computer [M. GUERBAOUI  et  al. , 2013].

2.System Architecture

The global architecture of the greenhouse data acquisition system shows in Figure(1) below. The system design consists of two parts Hardware  and Software.

Arduino

mega

light

Soil

moisture

MG8-11

DHT11

Sensors circuits

USB

Figure (1): The global  architecture

# 2-1 Hardware part

The core of the hardware part is the data Acquisition card that is used as a mean between PC and sensors circuit ,arduino mega 2560 r3 is used here for this purpose. The Arduino Mega 2560 is a microcontroller board use AVR ATmega2560.It has16 analog inputs pins and 54 digital input/output pins (of which 15 can be support work as Pulse-width modulation outputs).

- Temperature and humidity sensor

Temperature and relative humidity sensor (DHT11) is multifaceted with  a calibrated digital signal output features (D-Robotics, 2010), it  enclose high  accuracy and stellar long-term  steadiness. Single-bus  data  form is used to communicate and  synchronize control unit and DHT11 sensor. One communication handling is about 4ms. Data  composed of decimal  and  integral  portions.  A  entire data  transmission  is  40bit,  and  the DHT11 sends response as higher bit first. Also the time of sensor is very sensitive to distinguish zero and one bits as show in Figure(2).

Figure (2): Timing diagram depict the protocol include in the connection between a control unit and the DHT11 sensor (D-Robotics, 2010).

Data form: 8bit integral RH data + 8bit decimal RH data + 8bit integral T data + 8bit decimal T data + 8bit check sum. If the data transmission is valid,  the  check-sum should be the last 8bit of "8bit integral RH data + 8bit decimal RH data + 8bit integral T data + 8bit decimal T data".

In the present work one of digital I/O of arduino mega is used for DHT11 sensor. By approach of a handshake the values are pulse out over the single digital line and connected GND and VCC as show in Figure (3).

Figure (3): Typical Application Schematics

- Soil moisture sensor

The second sensor Soil Hygrometer detection module soil moisture sensor had been used for greenhouse monitoring this module has the important benefit in process of irrigation. The sensor as show in Figure (4)

Figure (4): Soil Moisture Sensor Module

Two approach in the soil moisture, less than a set threshold value when the D0 pin output high, when soil moisture surpass the threshold value is set, the module D0 pin output low, digital outputs D0 can be directly connected with the MCU, arduino mega r3 to distinguish high and low, and thus to alarm about soil moisture. Analog output A0 connected through the A/D converter or to any pins of analog onboard, as well as use analog  pins can give more accurate values of soil moisture

Figure (5):  Soil humidity module circuit diagram

# Many of these sensors can be used and distributed in greenhouse, using analog inputs  for arduino mega .

# Lighting sensor

# The third sensor is Photosensitive resistance module light sensor to adjustment amount of lighting, the condition circuit is same as the soil moisture sensor shown in Figure (5) also connect to any analog input (A0-12) of arduino mega .

# CO2 concentration

# The fourth sensors for CO2 concentration (MG-811) ,there  is  an  onboard  signal conditioning circuit for amplifying output signal and an onboard heating circuit for heating the sensor. The MG-811 is highly sensitive to CO2 and less sensitive to alcohol and CO. this sensor need one analog input and one digital for connect the sensor to arduino mega. Typical Application Schematics shown in Figure (7), the sensor module shown in Figure (6)

Figure (6):  MG-811 CO2 sensor module

Heating Control Circuit and Switching Mode regulator

MG-811

Signal Amplifier

And Pass Low

6-12VPower

supply

Ardunio mega

ADC

HEAT

GND

GND

VOUT

VCC

BOOL

Figure (7):  Typical Application Schematics

The MG-811 sensor is basically a cell which gives an output in the range of 100-600mV (400—10000ppm CO2). The LMC662 is used as the amplifier because of its ultra-high input impedance. According to the datasheet  of  MG-811,  this  sensor  require  an  input  impedance  of  100-1000Gohm

From Figure (8) we can calculate output from multiple input by gain

Vout= Vin *(1 + R4/R1)

In this specific application

Vout= 8.5*Vin.

Figure (8):  Signal Conditioning circuit

Also this sensor module  has chip MP2359 for Switch Mode Heating Voltage Regulator of  MG-811 heating circuit.

# 2-2 Software Part

The hardware design had been completed, the next step is to define the  software  part. Which is composed of two main parts, software  part configuring in the  arduino mega 2560 r3 (Firmware)  and  software for LabVIEW .

In order to interface The NI LabVIEW with arduino, the NI LabVIEW Interface for Arduino Toolkit (LIFA) support to readily interface with the Arduino board using LabVIEW. With LabVIEW toolkit data can be controlled or acquired from the Arduino board. Once the data is in LabVIEW, it will be analyzed using the hundreds of built-in LabVIEW functions, improve algorithms to control the Arduino hardware, and present the  findings on a polished UI.

The LabVIEW toolskit given features :

- Easy used  Arduino digital input/output , analog input, I2C, and Serial Peripheral Interface from LabVIEW.

- I/O engine sketch to load on Arduino

- communication wireless via  XBee or Bluetooth.

- Loop rates: USB tethered (200 Hz) and wireless (25 Hz).

- IDE arduino sketch and LABVIEW toolkit VIs help to specification functionality.

The Firmware took from LabVIEW toolkit and upload to arduino mega using integrated development environment (IDE) arduino as shown in Figure (9).

Figure (9):  IDE arduino

Now the firmware wrote in C\C++ language support all sensors in the greenhouse except the DHT11 sensor because this sensor is very sensitive for time (cannot process by LabVIEW), thus we needed to edit or modified firmware by add two files to firmware one DHT11.cpp contain the main program of  DHT11 sensor and declaration of parameters, DHT11.h contain process and function of  DHT11 sensor, these files are called by main firmware. The software design of LabVIEW is a development environment which is the user graphical programming to interact with real world data. The block code shown in Figures (10,11,12,13) illustrate the design.

Figure (10):  DHT11 sensor design

Figure (10) shows the required block code for operation DHT11 for acquisition data in greenhouse, so it is  needed to define Init that includes the VISA resource (COM34), type of arduino board, type of connection (USB/serial, XBEE, Bluetooth), bate rate and at the end define close session.

Figure (11):  IDE arduino

Figure (12):  Light Intensity design

The Figure (11,12) shown acquisition data of light and soil moisture and final Figure shown the CO2 concentration read from sensor

Figure (13):  CO2 sensor design

# 3.Results and Conclusion

The system monitoring based on computer and arduino mega board given ability for data acquisition from greenhouse. The system applies for small greenhouse and the changes for many parameters had been gotten that may be process in the future in order to give appropriate action by using one of control methods .

As shown in Figure (14) had been shown the change of temperature and humidity ,that take through average half hours between each one.

Figure (14):  The temperature and  humidity data measured in the greenhouse.

Figure (15) shows the measured value taken in a time period of one hour between each reading. In additional the change of light intensity and carbon dioxide in Figures (15,16),notice that the CO2 concentration is very low because light intensity is high .

At the end, the conclusion of this paper: the acquisition data can be done using arduino mega board with the ability of increasing the size and number of greenhouses by using one board because it has multiple analogs inputs, digitals inputs/outputs in additional to other specifications

Figure (14):  Soil moisture measured

Figure (15):  Monitoring of light intensity

Figure (16): Carbon dioxide data monitored in the greenhouse.

Reference

Anuj Kumar, Abhishek Singh, I. P. Singh, and S. K. Sud, (2010).”Prototype    Greenhouse Environment Monitoring System”, Vol. 2.

Blasco,X.,M. Marti'nez, J.M. Herrero, C. Ramos and J. Sanchis, (2007). “Model-based predictive control of greenhouse climate for reducing energy and water”. Comp. Elect. Agric., 55:49-70.

David A. Mellis,  Massimo Banzi, David Cuartielles and Tom Igoe, (2007). “Arduino: An Open Electronics Prototyping Platform”, conference on human factors in computing systems San Jose, California, USA.

Fourati, F. and M. Chtourou, (2007). A greenhouse control with feed-forward and recurrent neural networks. Simulation Modelling Practice and Theory,15:1016-1028.

GUERBAOUI, M.  Y. EL AFOU, A. ED-DAHHAK, A. LACHHAB and B. BOUCHIKHI, (2013). ”PC-BASED AUTOMATED DRIP IRRIGATION SYSTEM”, Vol. 5, No. 1.

Herrero, J.M., Blasco,M. Marti'nez, C.Ramos and J. Sanchis, (2007). Non-linear robust identification of a greenhouse model using multi-objective evolutionary algorithms . Biosys. Eng. ,98:335-346.

KiranSahu and Mrs. SusmitaGhosh, ( 2012). ”Digitally Greenhouse Monitoring and Controlling of System based on Embedded System”,Vol. 3, Issue 1

MAHMOUD OMID , (2004).”A Computer-Based Monitoring System to Maintain Optimum Air Temperature and Relative Humidity in Greenhouses”, Vol. 6, No. 6.

Melrolho, J.C., C.M.J.A. Serodio and C.A.C.M. Couto, (1999). “CAN based actuation system for greenhouse control”. Industrial Elect.,pp:945-950.

Mirine H. jad,S.H.Sadati,S.Hasanzadeh,A.M. Shahri and M.Ghasemian, (2008).” Design and Simulation of an Automated System for Greenhouse usingLabVIEW”,Vol. 3 No. 2.

Putter, E. and J. Gouws, (1996). “An automatic controller for agreenhouse using a supervisory expert system”, Electrotechnical Conference, MELECON 96, 8th Mediterra-nean,pp:1160-1163.

Robotics D UK, (2010). “DHT11 Humidity & Temperature Sensor”, www.micro4you.com/files/sensor/DHT11.pdf

Shin, J. H. W. S. Hahn, and Y. M. Lee, (1998). “Development of the environmental control network system in greenhouse”, Agriculture  Information Technology in Asia and Oceania, pp. 82-83.