Design of  Free - Distortion and Rotation

Magnetic Lens

Ban Ali Naser

Department of Physics, College of Science, University of  Babylon.

Ban_aljomaa@yahoo.com

Abstract :

In this present work, an investigation of a magnetic projector lens with tapered- cylindrical pole pieces has been designed and constructed, consisting of two double magnetic lenses. By using this lens a rotation and distortion free images have been obtained, after the electron-optical properties have  been computed by computational programs which were previously prepared. Results indicated the possibility of designing such a lens to give rotation and distortion free images to be used in Transmission Electron Microscopes (TEM) over a wide range of accelerating voltages.

Key words:  Electron  Optics, Magnetic Lenses , Aberrations.

الخلاصة:

تم في هذا البحث دراسة تصميم وبناء عدسة مغناطيسية مسقطية عديمة الدوران و التشويه بأقطاب اسطوانية- مقطعة الشكل, تتكون من عدستين مغناطيسيتين ثنائيتي القطب. وباستخدام هذه العدسة تم الحصول على صور  عديمة الدوران و التشويه ، بعد أن حسبت الخواص البصرية الالكترونية بوساطة برامج حاسوبية معدة مسبقا. أوضحت النتائج إمكانية تصميم مثل هذه العدسة لتعطي صورا خالية من الدوران و التشويه لاستخدامها في المجاهر الالكترونية النفاذة لمدى واسع من فولتيات التعجيل.

الكلمات المفتاحية :  البصريات الالكترونية، العدسات المغناطيسية، الزيوغ.

1. Introduction:

In the present work a computational method has been developed to correct the two distortions of the doublet projector magnetic lenses (Chalab,2010).The magnetic projector lens in the Transmission Electron Microscope (TEM) is considered an important lens in the imaging system. This lens magnifies the image formed by the objective lens, and projects it on the screen, but it suffers from fundamental defects, the most significant of which are the radial and spiral distortions (Wischnitzer,1981). The important feature for the magnetic projector lens is the rotation of image about the optical axis  as the imaging electron beam passes through the lens magnetic field (Al-Nakeshli,1994)The rotation angle is given by the following equation (Hawkes,1972):

θ = 0.1863 ( NI / Vr1/2 ) …………………(1)

Where Vr is the relativistically corrected acceleration voltage and NI is the number of ampere- turns. Equation ( 1 ) shows that the direction of rotation depends on the direction of the current in the coil .

The first theoretical and practical study about  projector lenses with  rotation free  image has been investigated by ( Juma, 1975, Juma & Mulvey, 1974,1976,1978) .The simplest formula of the projector magnetic lens of rotation free is the doublet lens which consists of  identical two lenses in design and are working under the condition (NI)1 = - (NI)2 so that the net rotation of the image zero.(Tsuno and Harad ,1981) designed the doublet projector magnetic lens. In the first rotation they obtained an image with spiral distortion rate of 0.07% at excitation parameter (NI/ Vr1/2) where the radial distortion equal to zero.

(Al- Obaidi,1991) designed a doublet magnetic lens consists of two lenses, air gap width S1 of the first lens have different forms of air gap width S2 for second lens, which earned image of  rotation and distortion free  in the great magnification,While (Al-Saady,1991) obtained an images of rotation and distortion free in the low and high maximum magnification region using doublet projector magnetic lenses with  the cylindrical pole pieces .

(Al-Saady,2008) got an image of free of rotation and distortion in on points of bones first and second magnetic lenses using double projector magnetic lenses  but with conical pole.

Several computer  codes written in Fortran language were employed in the present calculation, such as ( Al-Saady &Al-Shummary & Al-Shfii,2005) to calculate the magnetic flux density distribution and (Al-Shfii,1993) to calculate the projector  focal properties of the doublet lens.

2. Lens design:

In the present work , in an attempt to obtain free of rotation and distortion in first great magnification region for (TEM)  , a doublet projector magnetic lens with tapered- cylindrical pole pieces has been designed . It consists of two lenses , both of them has double - tapered cylindrical pole piece   (Lencova,1988) the air gap of each lens is equal to (5mm),as shown in  Figure (1). The coil is located within each lens and the installation of steel about (5 mm) from the face of the pole (Marai ,1977).The section area of each coil is (A = 1200 mm2).

In order to get an images with a free of rotation in this lens, the excitation in the first lens must be equal and opposite to the excitation of the second lens means that 1000A.t., this means that the number of coil laps of the first lens N1 equals the number of coil laps of second lens N2, and the current I1 passes through the first lens in opposite direction to the current I2  in the second lens .

3. Results and discussion:

Figure (2) shows the axial magnetic flux density distribution  Bz of the doublet projector lens for bore ratio D1: D2: D3 = 7:2:2 and the air gap width of each  lens equal S1 = S2 = 5mm. In the first magnification region, as presented  in figure (2), the magnetic field distribution of the first  lens is opposite in direction to that of the second lens so as to obtain rotation-free image, the maximum magnetic flux density Bm1 of the first double-tapered cylindrical lens is equal to 0.203 T and located in the middle of air gap due to a symmetrical of magnetic field, while the maximum magnetic flux density Bm2 of the second double tapered-cylindrical lens is equal to 0.270 T and located in the middle of air gap due to a symmetrical of magnetic field too.

The distribution of the axial magnetic flux density appeared in figure (2) were used to calculate of the focal properties of the doublet projector lens. Where the figure (3) shows  change the projector  focal length (Fp), radial distortion coefficient (Dr) and the spiral distortion coefficient (Ds) as a function of excitation parameter (NI/Vr1/2). Noting that the minimum value of  the projector  focal length (Fp) is equal to 32.670 mm and located at the excitation parameter NI/Vr1/2 = 5, the radial distortion coefficient (Dr) and the spiral distortion coefficient (Ds) vanishes at the same excitation parameter of about NI/Vr1/2 = 5, a distortion –free at high magnification region is obtained.

Figure (1):Cross section of the  upper half of doublet projector magnetic lens.

Figure(2):Axial magnetic field distribution Bz for doublet lens at  bores ratio

D1:D2:D3= 7:2:2 and width of the  air gaps S1 = S2 = 5mm   .

Fp

Ds

Dr

Fp (mm)

Dr (mm)-2

Ds (mm)-2

Figure (3): Variation of the projector focal length Fp ,radial distortion coefficient Dr and Spiral distortion coefficient Ds with the

excitation parameter NI/Vr1/2 at the first magnification region.

4. Conclusions:

It appears that the correction of distortion depends on the magnetic geometry. In order to cancel the distortion at first  magnification region, the axial magnetic flux density of the first lens must be lower than that of the second, to be equivalent to a small displacement from the optical axis in the second lens.

The results shows the possibility of designing rotation-free and distortion-free doublet projector magnetic lenses at high magnification in the transmission electron microscope (TEM) and scanning transmission electron microscope (SEM) over a wide range of accelerating voltages.

References

Al-Saady A. K. 1999, “Correction of spiral distortion by a doublet magnetic lens” , J. Babylon  University , 4,3,875-882.

Al-Saady A. K. 2008 “Design of project magnetic lens with conical pole piece and equable face” J. Al-Qadisiya for pure science ,Vol.13,No.1 128-134.

Al-Saady A. K. , Al-Shummary B.A.N. , and Al- Shfii T.M.A. 2005 "Magnification of a Doublet  Projector Lens for the Transmission Electron Microscope" Journal of the University of   Karbala , 3 , 12 , 30-35.

Al-Shfii T. M. A. 2007 “Computational  Investigation for Desgin Lenses of Transmission Electron Microscope” Ph.D. Thesis, University of Al-Mustansiriyah, Badhdad, Iraq.

Al-Nakeshli  I. S. Al-Obaidi, H. N. and A1-Naimi, M. 1994,"Iron   Magnetization of the Doublet  Projector Electron Lens"., J. Col. of Edu, Al-Mustansiriyah University, 4,101 -111.

Al-Obaidi H. N. 1991 “Design of Electromagnetic Lens”. M.Sc. Thesis University Al- Mustansiriyah, Baghdad, Iraq.

Chalab   M .R. 2010,"Investigation for the Projective Properties of the Symmetric Tapered Cylindrical Double  Pole Piece Magnetic Lens" J. University of Thi-Qar ,4,5,1-8.

Hawkes   P.W. 1972 Electron optics and electron microscopy

( London: Taylor and Francis ).

Juma  S. M. 1975 "Rotation Free Magnetic Electron Lenses".Ph.D.Thesis, University of   Aston Birmingham, England, UK

Juma  S. M. and Mulvey  T1974 "New Rotation Free Magnetic Electron Lenses "., vol. l, ed. J. Sanders and D.J. child Good (Australian Academy of Science, Canberra). 134-135.

Juma S. M. and Mulvey T. 1976 "A new Experimental Electron Microscope with a Rotation Free Projector System"., Developments Electron Microscopy and Analysis ed. J. A. Venables (London: academic).

Juma  S. M. and Mulvey, T. 1978 ,“ Miniature Rotation-Free Magnetic Electron Lenses for the Electron Microscope”. J. Phys. E: Sci. Instrum. 11, 759-764.

Lencova 1988, “Program AMAG for the Computation of Vector Potential Distribution in Rotationally Symmetric Magnetic Lenses by Finite Elements Method”, Czechosl. Acad. Sci .Brno. Kralovopolska, 147, 612-664.

Marai F.Z1977 “Electron optical properties of single- pole magnetic electron lenses” , Ph.D Thesis , University of Aston  Birmingham , UK .

Tsuno  K. and Harada, Y 1981, "Minimization of Radial and Spiral Distortion in Electron  Microscopy  through the Use  of a triple Pole piece Lens" J.Phys. E:Sci. Instrum., 14,313-319.

Wischnitzer S1981 “Introduction to Electron Microscopy “, 3rd ed . Pergamon press .