Mathematical Modeling of Flexible Robot Arm

Using Finite Element Method

Mustafa T. Hussein    Mohamed J. Al-Robaiy

Mohammed A. Al-Shjary

College of Engineering, University of Babylon

Abstract

In this paper derivations of the dynamic modeling of flexible robot arm are presented, the finite element method is used in the modeling of this work. The link is modeled as single finite element to simplify the problem; two types of boundary conditions are used; clamped-free boundary conditions, and pinned-free boundary conditions. The payload is added to the model by attaching additional masses to the arm at any specific location, many parameters are added to the model such as total viscous friction of the system, and total mass moment of inertia of the system, to increase the validity of the model. The model is presented in two forms the first one is traditionally formed and the second is the state-space form due to the importance of this form in the equations of the control systems. The model is highly effective for symbolic calculations due to the simple and low number of parameters which deals with it.

الخلاصة

في هذا البحث تم اشتقاق معادلة الحركة لذراع الانسان الالي المرنة بمستوي واحد باستخدام طريقة العناصر المحددة FEM)) في التحليل الرياضي. تم معاملة الذراع على انها قطعة واحدة (SFE) وذلك لتسهيل التمثيل الرياضي للمسألة . تم اختيار نوعين من الشروط الحدية في المسألة الأولى مع وجود ميل في قاعدة الذراع (P-F) اما الحالة الثانية فهي بعدم وجود ميل في قاعدة الذراع (C-F). تم تمثيل الاحمال المضافة على الذراع بواسطة اضافة كتل حسب المواقع المطلوبة, تم اختيار النقطة النهائية للذراع لاضافة الكتل باعتبارها اهم نقطة لرفع الاحمال. كذلك تم اضافة معاملات الاحتكاك بالنسبة للذراع ومصدر تجهيز الحركة وكذلك عزم القصور الذاتي الكلي للنظام وذلك لبيان تأثير هذه العوامل في التمثيل الرياضي. تم تقديم معادلات الحركة بطريقتين الاولى هي طريقة المصفوفات والثانية هي طريقة (state-space) وذلك لتسهيل استخدام النموذج الرياضي في معادلات السيطرة.  إنّ النموذجَ فعّالُ جداً للحساباتِ الرمزيةِ بسبب العددِ البسيطِ والمنخفضِ للعوامل التي يستخدمها وذلك يرجع لاستخدام طريقة (FEM) في التحليل وكذلك يمكن استخدامه ببساطة في معادلات السيطرة.

1. Introduction

Flexible manipulator arms have been modeled for about 30 years for the purpose of describing their dynamic behavior and improving it. Modeling is still very difficult to do in a practical and accurate way. Modeling techniques have advanced in sophistication over the years and nonlinear models with extensive consideration of the distributed parameter nature of the idealized mechanism can be constructed. In many cases the model needed for control design is much simpler and even linearized [Whitney and Book 1974].

Robot manipulators have for some time been widely used both in hostile environments and for industrial applications. However, experience has shown that for many tasks, presently available robots are neither sufficiently accurate to be effective nor fast enough to be economical. The requirements for higher speeds and better system performance make it is necessity to consider the dynamic effects of structural flexibility in the design of manipulators [Kanoh 1986] .

The issue of modeling and control of flexible robot arms is studied by many investigators, due to the demands of light-weight robot arms and faster and more accurate robot motions. The elasticity of robot arms under consideration now falls into two categories: joint elasticity and link elasticity. It is widely accepted that the joint elasticity comes mainly from the backlash of the gear and torsion of the shaft in the driving train [Kanoh], while link elasticity results from the elastic property of the material of which the robot arm is made. There are a number of dynamic model for one link Euler-Bernoulli beam robot arms [Hastings and Book 1987], with or with out joint flexibility. Some of them were derived in the form of a partial-differential equation (PDE) with boundary conditions, and then reduced to a set of ordinary differential equations (ODE) finite expansion method. Others were derived directly using finite expansion and Hamilton's principles to get a set of ODE's [Feliu et al 1992].

The large mass and energy requirements of standard rigid link manipulators have led to a desire for flexible link manipulators characterized by low-mass links and actuators with low power requirements. This is particularly desirable in certain applications, such as space systems, where mass and energy requirements must be minimized for transport purposes. Flexible link dynamics are also found in certain mechanical pointing systems [Taylor and Lu 1995] and in systems with links having high length-to-width ratios. These dynamics make the system outputs such as tip position more difficult to control. If flexibility is not taken into account in controller design the system can become unstable. Therefore, before flexible link manipulators can be realistically implemented, it is necessary to study the nature of flexible link manipulators and determine effective methods for end-point position control.

An appropriate beam model for the flexible arm must be chosen based on the boundary conditions at the ends of the arm. These boundary conditions describe constraints on the ends of a beam, such as whether a beam is free to rotate or not. For different boundary conditions the beam will vibrate at different frequencies [Brandeen 2001].

Previous studies choose either clamped-free or pinned-free boundary conditions for the beam model. This paper shows that a system with friction can exhibit both pinned-free and clamped-free behavior due to a change in boundary conditions. The changing nature of these boundary conditions can be observed in the system behavior.

Therefore, two different beam models was used  in this paper for modeling the flexible arm portion of the system a clamped-free model, and a pinned-free model. The beam models will combine with the rigid body system in different ways. The clamped model will use an explicit coupling term while the pinned model does not. A combined rigid-flexible body model must be determined for both the pinned and the clamped cases. The boundary conditions of the simulation will then determine when each model is used.

2. Dynamic Modeling

The flexible body dynamics of the system refer to the deflection of the beam from the expected rigid body position. The equations for this motion can be determined using the Lagrange equations and the assumed modes method [Mustafa 2007]. If the deflection is assumed to be the sum of the generalized coordinates multiplied by shape functions, the equations can be determined by computing the generalized mass, generalized stiffness, generalized force, and generalized moment. The generalized coordinates and shape functions can be determined using the finite-element method as presented in this paper. This is less labor intensive than solving the Euler-Bernoulli beam equation, which is a fourth order partial differential equation [Hastings and Book 1986].

Also, system constants such as hub inertia and payload mass must be included in the boundary conditions when solving the partial differential equation. Thus for any changes in these values the equations must be re-solved before a model can be obtained. In the finite-element method this is not necessary and in the resulting total system such values can be updated directly if they change. This is advantageous when modeling a system with varying payload, especially if the model will be used for real-time control.

2.1. Pinned Free Model

Figure 1 shows single link flexible with pinned free boundary conditions. For simplicity, the beam is modeled as a single finite element. This is adequate since the system behavior is dominated by the first mode, which is a relative simple shape function [Thomson 1998]. If a pinned-beam is modeled as a single finite element, there are three generalized (independent) coordinates: slope of the beam at the hub,θ1 the tip deflection, v2 and the tip slopeθ2. The deflection of the beam at any point along its length is then:

yx,t=θ1tφ1x+v2tφ2x+θ2tφ3x                                                                (1)

Where φ1 , φ2 and φ3 are the shape functions and x is the distance along the beam.θ1,v2 and θ2 are coordinates of time only, andφ1, φ2 and φ3are functions of space only (distance,x)

Figure 1 pinned free beam deflections in the x-y plane.

The generalized mass, MBand generalized stiffness,KB are determined from these shape functions [Mustafa 2007]. Knowing these values as well as the generalized forces,M1, M2 andF2, the equation for the flexible body system is:

MBθ1v2                                                                                (2)

The shape functions must be chosen to satisfy the boundary conditions of the system. For the case of a pinned-free arm the boundary conditions are:

y0,t=0 yL,t=v2y'0,t=0 y'L,t=θ2                                                                                   (3)

The beam deflection at any time is chosen to be a third-order polynomial in since this is the lowest order polynomial that can be a solution to the fourth order homogeneous Euler-Bernoulli beam equation. If the beam deflection is chosen as:

y(x,t)=a0+a1x+a2x2+a3x3                                                                            (4)

Then based on the boundary conditions, the constants are found to be:

a0=0a1=θ1a2=−2Lθ1+3L2v2−θ2La3=1L2θ1−2L3v2+θ2L2

Substituting these constants into Equation (4) and rearranging gives:

yx,t=θ1tx−2x2L+x3L2φ1+v2t−2x3L3+3x2L2φ2+θ2tx3L2−x2Lφ3                       (5)

yx,t=θ1tφ1x+v2tφ2x+θ2tφ3x                                                             (6)

The generalized mass matrix is given as [Thomson 1998]:

MB=mij          (7)        Where mij=mxφiφj dx

Where mx is the mass per unit length on the beam (assumed to be constant, i.e.mx=m)

The generalized stiffness matrix is given as:

KB=kij            (8)         Where kij=EIφiφj dx

Where E is the modulus of elasticity and I is the second moment of area of the arm.

For the pinned system, the torque at the hub causes the movement of the arm that leads to a change inθ1, and the subsequent beam vibration. From the rigid body equations [Brandeen 2001] the torque at hub in terms of motor gear angle is:

τhub=M1=Nτa−Jmθm−bmθm−bsθmN−Jg2θmN                                               (9)

F2=0M2=0

Whereτa,Jm,Jg2,bm: Is the torque supplied by the motor, the moment of inertia of the motor, the moment of inertia of arm gear, the viscous friction of the motor.

The momentM1 is the equation that links the rigid body to the flexible body. Since the arm angle is related to the motor angle through the gear ratio, the substitution θ1=θmN  is made in Equation (2), and the resulting system has the state variablesθm,v2, and θ2. Therefore, the dynamic equation for the combined rigid-flexible body system is:

MTθmv2                                                                          (10)

Where

MT=MB11+NJm+Jg2NMB12MB12MB21MB22MB23MB31MB32MB33BT=Nbm+bsN 01×202×102×2

KB=KT Q=10 τ=Nτa

Where bs the viscous friction of the arm shaft.

These equations are written in state-space format as:

x=Ax+Buy=Cx                                                                                                            (11)

x=θmv2θ2θmv

A= 03×3 I  , B= 0

A payload mass acts as a force on the end of the beam,Fp where Fp=mpv2 . To include a payload in the calculations, only the mass matrix, MB of the flexible body equations changes such that:

MBpayload =MB Nopaylaod+0 0 00 mp 0                                                             (12)

2.2 Clamped-Free Beam Model

For the clamped-free beam shown in figure 2 (modeled as a single finite element, there are only two generalized (independent) coordinates: the tip deflection, v2 and the tip slopeθ2. The deflection of the beam at any point along its length is that:

yx,t=v2tφ1x+θ2tφ2x                                                                           (13)

Where φ1 and φ2 are the shape functions and x is the distance along the beam. Note that v2 and θ2  are functions of time only, φ1 and φ2 are functions of space only.

Figure 2 clamped free beam deflection in x-y plane.

The generalized mass, MB and generalized stiffness,KB are determined from these shape functions. Knowing these values as well as the generalized forces, M2 and F2 the equation for the flexible body system is:

MBv2                                                                             (14)

The shape functions must be chosen to satisfy the boundary conditions of the system. For the case of a clamped-free beam the boundary conditions are:

y0,t=0 yL,t=v2y'0,t=0 y'L,t=θ2                                                                                (15)

If, as in Section 2.1, the beam deflection at any time is chosen to be a third-order polynomial inx as in equation (4).

Then based on the boundary conditions, the constants evaluate to:

a3=a4=0a2=3L2v2−θ2La1=−2L3v2+θ2L2

Substituting these constants and rearranging gives:

yx,t=v2t−2x3L3+3x2L2φ1+θ2tx3L2−x2Lφ2                                                    (16)

yx,t=v2tφ2x+θ2tφ3x

Depends on the equations (7, & 8) and by using equations (15, 16) can find the generalized mass and stiffness matrix. For the combined system, the loading on the clamped beam is caused by the accelerating hub. The acceleration of the hub,θa causes a distributed force on the beam,px which can be represented by a force and a moment at the tip of the beam,F2  andM2 . These can be calculated from the generalized force equations:

F2=pxφ1xdxM2=pxφ2xdx                                                                                      (17)

Hub

Flexible arm

Figure 3 direction of torque along the arm

Where px=−mθax  is the distributed force on the beam:

F2=7mL220θa=−7mL220Nθm=FmθmM2=mL320θa=mL320Nθm=Mmθm

Figure 3 illustrates the positive direction of the torque acting on the hub (Mo) due to the flexing of the beam when a torque ofτ1, has been applied to the hub. Note that for the deflection shown in Figure 3 the value of Mo will be negative since it opposes the applied torque. The variablesτ1, ωm and Mo act on the motor side of the gears.

The moment at any point along the arm can be found from:

Mx=EI {y''x,t

The equation for the rigid body can now be changed to:

τa+Mo=Jθm+bθm                                                                                       (18)

M0=M0N=EI {y''0,tN=EIN6L2v2−2Lθ2

Where Mo is the moment on the hub caused by the flexible body referred to the motor side of the system. The static friction component of Tfmust now opposes both τa andMo, when the speed is below the static fiction threshold. Combining the flexible and rigid bodies gives the dynamic equation:

MTθmv2                                                                  (19)

Where

KT=−6EINL2EIN0KB00 , Q=10 , MT=00JMB−Fm−Mm , BT =00b000000

x=Ax+Buy=Cx                                                                                                   (20)

x=θmv2θ2θmv

A= 0IMT−1KTMT−1BT , B= 0

Similar to Section 2.1, if a tip mass is included in the calculations, only the mass matrix, MB of the flexible body equations changes such that:

MBpayload =MB Nopaylaod+mp000                                                                (21)

3. Symbolic Example

This example presents the dynamic model of flexible robot arm in the symbolic form the model is calculating symbolically by using MatLab 6.5 software which the powerful program dealing with symbolic form of equations,  to easing the dealing with this model, and can be combining model matrices in the state-space form. First the model of pinned-free models is based on the equations (5, 6) was derived (whereMB−p, KB−p mass and stiffness matrices of the single flexible link with pinned-free boundary conditions) as shown:

MB−p=mL4204L213L−3L213L156−22L−3L2−22L4L2, KB−p=EL34L2−6L2L2−6L12−6L2L2−6L4L2

And then the clamped-free model is also based on the same equations (whereMB−c, KB−c mass and stiffness matrices of the single flexible link with clamped-free boundary conditions).

MB−c=mL420156−22L−22L4L2 , KB−c=EIL312−6L−6L4L2

4. Conclusion

A modeling process of single link flexible robot arm was presented; the equation of motion of the system was derived using finite element method (FEM). The equation of deflection (shape function), the mass and stiffness matrices was derived based on the finite element method. Two types of boundary conditions pinned and clamped to show the differences when using them in the modeling process. The nonlinear terms such as payload inertia and viscous friction factors where added to the model matrices. The symbolic example will finally present to show the configuration of the mass and stiffness matrices for each type of boundary conditions, the model was presented in two forms in matrices form and state space form by using the MatLab 6.5 software.

5. References

Brandeen McDonald, 2001 "Modeling and Control of a Flexible-Link Manipulator", master thesis, Control Engineering Faculty of Engineering Lake head University, Ontario, Canada.

Feliu, V., Rattan, K. S., Brown Jr., H. B., 1992 "Modeling and Control of Single-Link Flexible Arms with Lumped Masses" Journal of Dynamic Systems. Measurement and Control, Vol. 114, pp. 59-69.

Hastings G. G. and Book W. J., 1987 "A Linear Dynamic Model for Flexible Robotic Manipulators" IEEE, Control System Magazine.

Hastings, G. G. and Book, W. J., 1986 "Verification of a Linear Dynamic Model for Flexible Robotic Manipulators" Proceedings IEEE International Conference on Robotics and Automation, pp. 1024- 1029.

Kanoh H., et al, 1986 "Modeling and Control of Flexible Robot Arms" Proceedings of 25th Conference on Decision and Control Athens, Greece.

Mustafa T. H., 2007 "Modeling and Control of Planar Flexible Robot Arm", Master Thesis, Babylon University, Babylon, Iraq.

Sakawa, S., Matsuno, F., and Fukushima, S., 1985 "Modeling and Feedback Control of a Flexible Arm", Journal of Robotic Systems, 2(4), pp. 453472.

Taylor, J. H. and Lu, J., 1995 "Robust Nonlinear Control System Synthesis Method for Electro-Mechanical Pointing System with Flexible Modes". Journal of Systems Engineering, Vol. 5, pp. 192-204.

Thomson, T. and Dahleh, M.D., 1998 "Theory of Vibration with Application", Prentice-Hall, Inc.

Whitney, D.E., Book, W.J. and Lynch, P.M., 1974 "Design and Control Considerations for Industrial and Space Manipulators," Proceedings of the Joint Automatic Control Conference.