Mobile Robot Control Using Fuzzy Logic

Hussein A. Lafta         Zainab Falah Hassan

University of Babylon science collage for women

Zainab_ga@yahoo.com

Abstract:

In this work , intelligent fuzzy controller for mobile robot in various (known and unknown) environments is build to control of mobile robot. A successful way of structuring the navigation problem treat with the issues of individual behavior design and action coordination.

The inputs to the proposed fuzzy control method consist of a heading angle between a robot and a specified target and the distances between the robot and the obstacles to the left, front, and right to its locations, being acquired by sensors. The output of controller are the speeds of right and left wheels. Simulation results verified the effectiveness of the controller.

Key words : Mobile Robot , Environment ,Fuzzy Logic , Obstacle avoidance,.

الخلاصة:

في هذا العمل مسيطر مضبب ذكي للروبوت المتحرك في مختلف البيئات (معروفة و غير معروفة) يتم بناؤه للسيطرة على الروبوت المتحرك.الطريقة الناجحة لتشكيل مسألة القيادة مرتبطة مع مسائل تصميم سلوك مستقل وتنسيق الفعاليات.

المدخلات لطريقة المسيطر المضبب المفترض تتكون من الزاوية الرأسية بين الروبوت والهدف المحدد والمسافات بين الروبوت و العوائق إلى مواقع اليسار , اليمين و الأمام للروبوت , المكتسبة بواسطة المتحسسات .المخرجات للمسيطر هي سرع العجلات اليمنى واليسرى .نتائج المحاكاة تؤكد فعالية هذه المسيطر .

الكلمات المفتاحية: الروبوت, البيئة, المنطق الضبابي, تجنب العوائق.

1 Introduction

Current developments of mobile robot have attracted the attention of researchers in the areas of engineering, computer science,  mining and others. This is due to the high potential of mobile robots application. Autonomous mobile robots are robots which can perform desired tasks in unstructured environments without continuous human guidance [Frank and Goswami, 2004;Ibrahim and Fernandes, 2004];[ Kim and Cho, 2006];[Waterman, 1989]. Many kinds of robots are autonomous.

A fully autonomous robot in the real world has the ability to:

• Gain information about the environment.

• Travel from one point to another point, without human navigation assistance.

• Avoid situations that are harmful to people.

• Repair itself without outside assistance.

A robot may also be able to learn autonomously. Autonomous learning includes the ability to:

• Learn or gain new capabilities without outside assistance.

• Adjust strategies based on the surroundings.

• Adapt to surroundings without outside assistance.

Navigation for mobile robots can be well-defined in mathematical (geometrical) terms. It also involved many distinct sensory inputs and computational processes. Elementary decisions like turn left, turn right, run or stop are made on the basis of thousands of incoming signals [David, 1990; Gallistel, 1990; Parhi, 2005].  Navigation is traditionally defined as the process of determining and maintaining a trajectory to a goal location [Gallistel, 1990].

Humans have uncertain and imprecise information. The main advantages of a fuzzy navigation strategy lie in the ability to extract heuristic rules from human experience, and to obviate the need for an analytical model of the process [Seraji and Howard, 2002 ].

In this search, Fuzzy Logic System (FLS) is used to produce the control inputs for the robot with inputs from various sensors. Sensor signals are fed to the FLS, and the output

provides motor control commands (e.g., turn right or left). The FLS learns the full dynamics of the mobile robot online. Fuzzy controller for mobile robot has three inputs and two outputs. Both inputs and output have three membership functions. Each membership function is considered as triangular membership function. This search provides a Fuzzy logic framework to be implemented in the mobile robot for behavior design and coordination.

2 Kinematic Model:

Assumption considered for analyzing path constrained of a wheeled mobile robot:

• There are no flexible parts in the wheeled mobile robot.

• There is no transitional slip between the wheels and the surface.

• There is enough rotational friction between the wheels and the surface; so, the wheels can rotate without disturbance.

• The two driving wheels are identical.

The navigation for a two-wheeled mobile robot is controlled by the speed change of the robot. The kinematical scheme of a mobile robot can be depicted as in Fig. 1, where V is the centre velocity of the robot, VL is the velocity of the left wheel, VR is the velocity of the right wheel, r is the radius of each wheel, L is the distance between two wheels, x and y are the position of the mobile robot, and θ is the orientation of the robot.

According to the motion principle of rigid body kinematics, the motion of a mobile robot can be described using equations (1) and (2), where ωL and ωR are angular velocities of the left and right wheels respectively, and ω is the centre angular velocity.

VR=r WR		  	,VL=r WL						(1)

W=(VR-VL )/L			,V=(VR+VL)/2						(2)

This result:

W=(r/L)*(WR-WL) 		,V=(r/2)*( WR+WL)					(3)

The position of the robot in the unknown environment in any moment, is defined in the reference coordinates; [X, Y]. Considering the equations (1) and (2), the kinematic model of such a robot, is defined with the below equations:

It should be mentioned that the robot movement is divided into two:

- Turning situ and positioning in a new  direction.

- Moving in a direct path in the new direction till reaching the new point.

3 Fuzzy logic :

Intelligent controller for mobile robot enables the robot to avoid the obstacle and improve target seeking ability. In the absence of the obstacle, robot moves towards the endpoint (goal), otherwise the robot will react to the obstacles and its relative position from the goal, based on the information gained from the sensor located in the center robot. While moving towards the goal (with the obstacle avoidance strategy), the robot changes it’s angular velocity and linear velocity.

Fig. 2(a).				           Fig. 2(b).

Fig.2:Fuzzy logic techniques for behavior based control of mobile robot.

The fuzzy controller used for leading the robot, has: 4 inputs and 2 outputs as follows (Figure (2)).

Inputs: The four numerical inputs are fuzzified by the membership functions . These membership functions are defined based on the expert knowledge . Three inputs of fuzzy controller are the distances from robot to the left , front and right obstacles . One input represent the head angle between robot location and goal position .

Three triangle membership functions considered for each  input of fuzzy system as shown in the fig. 2(b), membership functions for distances are:

- Near.

- Medium.

- Far.

Three triangle membership functions considered for head angle of fuzzy system are:

- Front.

- Left.

- Right.

Outputs: The two numerical outputs are fuzzified by the membership functions as shown in Figures 2(b) . These membership functions are defined based on the expert knowledge .

Three triangle membership functions for velocity are:

- Slow.

- Medium.

- Fast.

The distance D can be calculated by applying the distance formula using the two value of dx and dy . where dx and dy are the difference in distance between the robot and the destination point in x and y coordinate .the following equations in calculating the input variables are :

Distance =√(dx)2 * (dy)2

Θ=tan -1 (dy/dx).

Here  , Θ is the desired angle that the robot should face every instance as it goes to the destination point .

3 The Fuzzy Rules:

The main idea in the fuzzy algorithm is the usage of the lingual rules when facing

conditions produced by controlling a process. In this search, the lingual rules are inspired by the human behavior. The fuzzy rules are an if- then statement that consist  of a premise (antecedent) and consequent .

According to the Table (2), if the distance between the robot and the obstacle is far, reaching the goal point is the first priority; otherwise the first aim would be increasing the distance between the robot and the obstacle Table (1).

4 Robot behavior:

The mobile  robot has three  wheels ,mobile robot  with two driving wheels, arranged parallel to each other , which are driven separately by two independent motors, and one castor is provided for stability of the robot is modeling a three wheeled mobile robot . The front wheel is a castor wheel.

The relations between the rigid body motion of the robot, the angular velocity and driving controls of the wheels guarantee that the motion of robot is stable.

According to the information acquired by the robots using their sensors, some of the fuzzy control rules are activated. The outputs of the activated rules are combined by fuzzy logic operations to control the velocities of the driving wheels of the robots. These are denoted by left_v and right_v, for the velocity of the left wheel and the velocity of the right wheel of each robot respectively. These parameters can be used to generate different fuzzy rules. e.g.

Rule: If (left-obs is far and right-obs is medium and front-obs is near and head-ang is N) Then (left-v is slow and right-v is medium).

By fuzzy reasoning and the centre of maxima defuzzification method, Rule related to the obstacle avoidance wall-following and target seeking behaviors, are weighted to determine an appropriate control action.

4.1 Obstacle avoidance:

When the acquired information from the sensors shows that there exist obstacles nearby robot, it must reduce its speed to avoid obstacles. When a robot is close to an obstacle, it must change its speed and steering angle to avoid the obstacle. The fuzzy rules used for obstacle avoidance by the robots are listed in the following as rules 1 to 27. All the rules in the table are obtained heuristically.

Table 1:List of rules for obstacle avoidance.

RuleNo: (Action)(Left_obs)( Right_obs)( Front_obs)( Head_ang )( Left_V  Right_V)

1. 	     AO 		Near   		Near		 Near           	Any	    Slow          	Fast

2.	     AO		 Near   		Near		 Medium	 Any 	    Slow 	Slow

3. 	     AO	 	Near 	 	Near 		Far		 Any 	     Med 	Med

4. 	     AO	 	Near  	 	Medium 	Near 		Any 	     Med 	Slow

5. 	     AO		Near   		 Medium 	Medium	 Any       Med 		Slow

6. 	     AO 		Near 		 Medium	 Far		 Any	     Fast 	Med

7. 	     AO		Near 		 Far 		 Near 		 Any 	     Fast 	Slow

8. 	     AO		Near 		 Far 		 Medium 	 Any 	     Med 	Slow

9. 	     AO		Near		 Far 		 Far 		 Any 	     Fast 	Med

10. 	     AO 		Medium 	 Medium 	 Near 	   	 Any 	     Fast 	Slow

11.	     AO 		Medium 	 Medium  	 Medium 	 Any	     Slow	Slow

12. 	     AO		Medium 	 Medium 	 Far		 Any 	     Fast 	Fast

13.	     AO 		Medium 	 Near 		 Near		 Any 	     Slow 	Fast

14.	     AO 		Medium 	 Near 		 Medium 	 Any 	     Slow	Med

15.	     AO 		Medium 	 Near 		 Far 		 Any 	     Slow	Med

16. 	     AO		Medium 	 Far 		 Near		 Any 	     Med 	Slow

17. 	     AO 		Medium 	 Far		 Medium 	 Any	     Med 	Fast

18. 	     AO 		Medium 	 Far 		 Far 		 Any 	     Fast 	Med

19. 	     AO 		Far 		 Near 	 	 Near		 Any	     Slow 	Med

20.	     AO 		Far		 Near 		 Medium 	 Any	     Med 	Fast

21.	     AO 		Far		 Near 		 Far		 Any 	     Med 	Fast

22. 	     AO 		Far 		 Medium	 Near 		 Any 	     Slow	Fast

23.   	     AO 		Far		 Medium 	 Medium	 Any 	     Slow 	Med

24. 	     AO 		Far 		 Medium 	 Far 		 Any 	     Med 	Fast

25.	     AO 		Far 		 Far 		 Near 		 Any 	     Fast 	Slow

26. 	     AO 		Far 		 Far 		 Medium 	 Any 	     Fast 	Med

27. 	     AO 		Far 		 Far 		 Far 		 Any 	     Fast 	Fast

When the robot sense obstacle near to it or the robot moves at curved and narrow roads, it must reduce its speed to avoid collision with obstacles. In this case, its main reactive behavior is decelerating for obstacle avoidance.

4.2 Target seeking behavior:

When the acquired information from the sensors shows that there are no obstacles around robot, its main behavior is target steer. List of some rules for target seeking are shown :

Table 2: List of rules for target seeking .

RuleNo: (Action)(Left_obs)( Right_obs)( Front_obs)( Head_ang )( Left_V  Right_V)

34	    TS		 Far		 Far		 Far 		Positive       Fast 	   Med

35	    TS 		 Far 		 Far		 Med		Negative      Med 	   Fast

36  	    TS 		 Far 		 Far 		 Far 		Zero		Fast 	   Fast

37	    TS 		 Far 		 Far 		 Med 		Positive	Slow 	    Med

38	    TS 		 Far 		 Med 		 Far                 Negative	Med 	  Fast

40 	    TS 		 Med 		 Far 		 Far 		ZERO	 	Fast 	   Fast

Here, the 34 and 35 for realizing this behavior as follows:

If (left-obs is far and right_obs is far and front-obs is far and head-ang is P) Then (left-v is fast and right-v is med).

If (left-obs is far and right_obs is far and front-obs is medium and head-ang is N) Then (left-v is med and right-v is fast).

These fuzzy logic rules show that the robot mainly adjusts its motion direction and quickly moves to the target if there are no obstacles around the robot. In general, the weights of the behaviours, obstacle avoidance and target steer, depend largely on the distances between the robot and the obstacles to the left, front, and right locations.

5 Fuzzy Inference Mechanism:

Fuzzy inference is the process of formulating the mapping from a given input to  an output using fuzzy logic. The mapping then provides a basis from which decisions can be made, or patterns discerned. The process of fuzzy inference involves Membership Functions and If-Then Rules. The used inference fuzzy mechanism is the MAMDANI (Min-Maxinferences).

6 Defuzzification:

There are several ways for performing defuzzification. The strategy selected here is the center of maxima method.

7 simulation Results and discussions:

The simulations were conducted with the visual basic language . To show the effectiveness and the robustness of the proposed method, simulation results on mobile robot navigation in various environments are explained.

The obstacle avoidance behavior is activated when the reading from any sensors are less than the minimum threshold values. This is how the robot determines if an object is close enough for a collision. When an object is detected too close to the robot, it avoids a collision by moving away from it in the opposite direction.

When the acquired information from the sensors shows that there are no obstacles around robot, its main reactive behaviour is target steer. Intelligent controller mainly adjusts robots motion direction and quickly moves it towards the target if there are no obstacles around the robot as shown in Figures 5,6,7. In the proposed control strategy, reactive behaviors are formulated by fuzzy sets and fuzzy rules, and these fuzzy rules are integrated in one rule base.

5 Conclusions

This paper shows the fuzzy logic method for intelligent navigation control of mobile robots. Very well agreement between the simulation and experimental results show the robustness of the fuzzy controller.

Planning systems may use a number of techniques to make the planning process practical. Determining the state of the world and guiding action requires the ability to gather information about the world, though sensors such as sonar or cameras. These sensors data have been used by the fuzzy controller for navigational task.

References:

David B. Leake, 1990. Van Nostrand Scientific Encyclopedia, Ninth Edition, Wiley, New York(U.S.A.).

Frank K., Goswami D.Y., 2004. Hand book of Mechanical Engineering CRC Press, New York(U.S.A.).

Gallistel C.R., 1990. The organisation of learning, MIT Press, Cambridge-MA.

Ibrahim M.Y., Fernandes A., 2004. Study on mobile robot navigation techniques, IEEE Int. Conf. on Industrial Technology, Tunisia, 1, 230-236.

Kim M. Y., Cho, H., 2006. An active trinocular vision system of sensing indoor navigation environment for mobile robots, Sensors and Actuators, 125(2), 192-209.

Li Wei, Xun Feng, 1994. Behaviour fusion for robot navigation in uncertain environments using fuzzy logic, Systems, Man and Cybernetics, 2, 1790-1796.

Parhi D.R., 2005. Navigation of Mobile Robots Using a Fuzzy Logic Controller, Journal of Intelligent and Robotic Systems , 42, 253-273.

Seraji H., Howard A., 2002. Behaviour-based robot navigation on challenging terrain A fuzzy logic approach, Robotics and Automation IEEE Transactions, 18(3), 308-321.