Performance Analysis Between Distance Vector Algorithm (DVA) & Link State Algorithm (LSA) For Routing Network

Asmaa Shaker Ashoor

University of Babylon

Asmaa_zaid281@yahoo.com

Abstract

In this paper, we survey the public presentation between two adaptive routing algorithms: Link state algorithm (LSA), which is centralized algorithm and Distance vector algorithm (DVA), which is distributed algorithm. The primary purpose of this paper is to compare two dynamic routing algorithms. Besides, we represent an overview of these algorithms distinguish similarities and differences between (LSD) & (DVA). A major part of this paper is to survey these algorithms, and analyze the results.

Keywords: LSA, DVA, routing, network traffic, comparison, simulation, analysis.

الخلاصة

في هذا البحث، نقدم مسح عام بين اثنين من خوارزميات التوجيه، وهي خوارزمية حالة الارتباطLSA) ) وهي خوارزمية مركزية وخوارزمية متجه المسافات ( (DVAوهي خوارزمية توزيعية. ان الغرض الرئيسي من هذه الورقة هو المقارنة بين الخوارزمية المركزية والخوارزمية التوزيعية الى جانب ذلك نقدم لمحة عامة عن هذه الخوارزميات عن طريق تميز اوجه التشابه والاختلاف بين (LSA) و (DVA) والجزء الاكبر من هذا البحث هو مسوحات لهذه الخوارزميات وتحليل النتائج.

الكلمات المفتاحية: خوارزمية LSA، خوارزمية DVA، الموجه، حركة مرور الشبكة،المقارنة، المحاكاة، التحليل.

- Introduction

A router is used to supervise network traffic and detect the better route from source to destination in a network (Larry, 2003, Andrew, 2003 ). A router should have some information about the network position to carry any determination. The router utilizes the routing algorithms to calculate the better route from source to destination. A routing is rules which are stored in the router memory. The role of the routing algorithms is working, determination for the router to determine the best paths. This paper identifies two cases of adaptive routing algorithms. These are (LSA) (Andrew, 2003, James, 2003) and (DVA) (Andrew 2003, Gambiroza 2004), as well known as globally and un-global routing. In bout the algorithms, every node has a routing table to save the information around the position of the network. The routing tables, like: fixed table, fixed central table and dynamic table can be established in dissimilar path, relying on the algorithm utilized (Larry, 2003, Andrew, 2003).

2. Survey of routing algorithms

Routing algorithms are dependable for furtherance the data packets through routes to offer a carrying out. In this paper, we explain the important aspects that relate to the two cases of adaptive routing algorithm (LSA and DVA). They are called next-hop routing, these are, all data packet that has a destination node Id and all nodes ensure adjacent node Id can denote that the following hop to follow that packet. The following hop of that node can become nil, in that instance, these nodes do not recognize where to follow data packets to that destination. The primary concept in the end comparison between LSA and DVA is introduced and improvements for briefly.

3. Distance Vector Algorithm

The original routing algorithm, designed in 1969, was a  Distributed Adaptive Algorithm) DAA) utilizing calculated delay as the operational measure and Bellman-Ford algorithm (DBF) (William, 2011). A distributed adaptive algorithm that calculates shortest path between each couple of nodes in the dynamic network. DVA demands each node to keep a distance vector. A distance vector has an entry for every probable destination, consisting of the case of the better usable path to pass the address and the following hop on the track. The better path can change and is based on administrative policies like shortest path or lowest cost path. To better illustrate of DVA, Figure (1) explain a sample topology of three nodes. Regarding node X explained in Figure (1) that is concerned with routing to address Y by that immediately linked Z.

Y

X

Z

Figure (1) sample topology

of three nodes

Node X length board input, Dx(Y, Z) are a total that value a direct first skip associate at "X" with "Z, c (X, Z)", with  near "Z" is presently called small value track of themselves "(Z)" of "(Y)" (V. Gambiroza 2004) there are:

"Dx(Y,Z)=c(X,Z)+minw{DZ(Y,W)}"………….(1)

Then, that "Dz (Y, W)" are the shortest path from the "Z" to the neighbor W. A simple algorithm can calculate the metrics which is based on this precept. Immediately connected networks are constantly specified metric 0.  DVA does not store information about a route to calculate a minimum cost, while it calculates the cost gradually. Whenever DVA sees a route with a smaller metric than the one in its table, it substitutes it with the new one. However, the nodes will choose the shortest route for information about various paths (Xu, 1997; James, 2003).

4. Links-State Algorithm

In the LSA each node keeps a perfect picture for that network topology with a value for each link (Larry, 2003; Andrew, 2003). For that, to maintain these views update, all node orderly broadcasts that the local case of every node to another node in the network utilizing a protocol like flooding. Hence, that every node informs the topology of the network and the case of every link. The algorithm is identified as observed:

- Detected each node its neighbors and the matching link costs.

- Each node propagates this information to all other nodes (utilizing flooding). The requisite estimate of flooding is that each node follows its link case information to its integral, immediate connected links and each node receives this information, following it out on its integral links exclude for the one that sent the information (Larry, 2003; Andrew, 2003). This procedure goes on until the information has passed each node in the net.

- Utilizes every nodes "link state packet (LSP)" from all other nodes to calculate map of network and better routes to any destination utilizing one of the shortest-path algorithms from graph theory. To calculate the shortest track to give "source node" that each  another ""node" by changing that track in order to increase the path length use Dijkstra's algorithm (William, 2011).

5. Comparison LSA and DVA

The similarities and differences are linked with the two adaptive routing algorithms, which are LSA & DVA. In LSA, each node has a calculate figure of each network picture. All nodes examine the value of the link to all of its neighbor nodes and overflows sporadically updated data to each other node in the network. After all nodes receive these updated data about the value of each link in the network, all nodes compute the shortest track to all possible destinations. When each node needs to forward a packet to some destination, it transports the packet to the next hop node based on the better track to the destination assumed for the update data. In universal, likened to the DVA, this protocol needs a greater extent CPU time for counting the complete shortest route to all possible destinations and more network bandwidth for sending the routing updates from all nodes to every another node that all net. Every node in DVA has information about neighboring nodes exclusively, hence to calculate the route on distributed way. Therefore, the two routing algorithms can be shown in the following phases. They are both dynamic routing algorithms that are every node preserve up-to-date information in terms of the best passing any other destination in the network. Each node in the network incessantly observes variation in topology and it knows the other nodes of those cases utilizing route updates. The network has the most current picture of the network topology at every node, so every node can be capable of making the best routes at any time. Therefore, in LSA, every node must have whole information about other nodes in the network. It stores each topology information at every node. It computes the route from the root to the address at the center. While, in DVA all nodes have information about neighboring nodes only. It computes the route on distributed mode (Roch 2002; Comrmen 2002). In the table 1.1 explains abbreviate characterized of some of the primary specifications of these algorithms. The comparisons explain standard and some of the suitable specifications that a routing algorithm should achieve for in the dynamic manner environment (Bertsekas 1992; Mourtada 2000).

Table 1.1 shows abbreviate characterized

DVA & LSA routing algorithm

|  | DVA | LSA |
| --- | --- | --- |
| Forwarding Table | A node transmits to its neighbors the all routing tables (its DV). | A node transmits to each the other one the case of the link with the stream neighbors by a dependable oversupplying. |
| Route Updates | Are sent periodically or when a topological variation is observed. | Are transmit sporadically or when a topological or when a topological variation is observed. |
| Path computation | It is established on a propagated edition of the classical bellman-ford algorithm (DBF) | A node can then make the chart of the network and can compute the track from itself to each other node independently. |
| recg | A node running a distance vector protocol does not recognize the network topology. | Because of flooding, all nodes eventually receive each the link-cases from each the other nodes of the network. |
| Topology Store | All node stores the neighbor's information at the end on the network topology. |  |
| Topology Change | DVA protocols may poorly affect to a topology variety ago, it endures from very low convergence (calculate-to-infinity trouble) and may make a provisional closed circuit. | LSA converges faster than DVA but it needs a higher overhead. |
| Example | A well-known example of DVA is RIP | A well-known example of LSAis OOSPF . |

6. Simulation Method of Topology

In this paper, simulation work is beginning with four nodes completely connected topology mesh, five nodes and so until the numbers of nodes go up to 13 nodes. A simulator of the network supplies appropriate manner to estimate and check a network protocol's implementation before it is utilized in the real network. In this simulation, we want to carry out comprehensively simulations to assure the veracity and generality of the results. A simulation is to take a packet from root nodes function and transfer it to the destination node functions. Moreover, links are warranted to transfer packets and so without loss. Thus, to prepare the estimate more efficient, we analyze the results with the assistance of statistical to guarantee that the illustrations of the results are proper.

7. Analysis the results work

In this paper, simulation experiments between two adaptive routing algorithms LSA & DVA by using variety mesh topologies. The network topology of simulator model starts out with four nodes and increments them, to reach sequence, to third nodes in which the cost of outgoing link is variable from that of the cost of incoming links. This study was done only in two cases. In the first case we explain, the cost of a link has been changed and in the second case we explain, when a link has failed. So note that the DVA broadcasts the updated information only to those nodes, which fluctuate by the single link-cost change. Thus, the total number of packets sent from the node can be varied from that of those received by some other nodes. On the other hand, explain fixed manner roughly of the LSA because the same link-cost update must be sent to each node. All changes in two algorithms prepare one at a time and the algorithms have time to converge before the next change happens.Therefore, the following parameters are used to compare between these algorithms, to make this evaluation more effective: Time to converge, number of steps to reach the convergence, the aggregate number of messages: (number of messages transmitted by all nodes, the number of messages obtained by all nodes), time delay for each packet sent. We can show simulation model results for two adaptive routing algorithms (DVA & LSA). First one, we can compute the time that the algorithms take to pass the convergence, for that case the algorithm stops sending and receiving packets, for that there is no packet in the medium, each packet is sent, and it will be prepared to represent with any varieties or failures in the links. Each of the computations has been made in two stages. When the cost of the link is changed and in case of link failure. All changes are performed one by one at a time and the algorithms have time to converge before the next change can occur.

Fig1.1 (TC) in LSA & DVA                                      Fig 1.2 (TC) in LSA & DVA

(Link change)                                                (Link failure)

The two algorithms in the Fig1.1 & Fig1.2 looking for about the other links from that node to another node. Recalculates "the shortest" path of each node in that network for LSA, it engages more time to pass the convergence time. For that, this time will increments with the increment in topology size. Whereas, DVA re-calculates the path only to some destination, therefore it appears to be better than LSA. Thus, it does not need more time to pass to the convergence. These patterns are in terms of number of nodes versus the convergence time (CT) in Fig1.2.

In case of link change & link failure

Y=a+b*x   (x=No. of nodes and y=CT for LSA)

Y=a+b*x+c*x2+d*x3+e*x4+f*x5(x=No. of nodes and y=CT for DVA)

When the parameters in case of link change and link failure its different

Fig1.3, illustrate the total (TMS) for all nodes in topology network is computed in two phases: when the cost of the link is varied and when the value of the link is a failure. All changes are executed one by one and the algorithms have time to converge before the next change can happen.

In case of link change & link failure

Y=a*xb (x=No. of nodes and y=TMS for LSA)

Y=a+b*x+c*x2+d*x3+e*x4+f*x5

(x=No. of nodes and y= TMS for DVA)

When the parameters in case of link change and link failure its different

Fig 1.3 (TMS) in LSA & DVA                             Fig 1.4 (TMS) in LSA & DVA

(Link change)                                                       (Link failure)

Fig1.4 illustrates No. of nodes versus Total Message Sent (TMS). Moreover, these patterns are utilized to present an expected value of any number of nodes. The algorithms have time to converge before the next change can happen and all changes are executed one by one at a time. Fig1.5 illustrates the number of steps for LSA increments when the size of topology increments. Furthermore, the patterns can be utilized to give an expected value for any number of nodes verses the number of steps needed.

In case of link change & link failure

Y=a*xb (x=No. of nodes and y=No. of steps for LSA)

Y=a+b*x+c*x2+d*x3+e*x4+f*x5

(x=No. of nodes and y= No. of steps for DVA)

When the parameters in case of link change and link failure its different

Fig 1.5 No. of steps                                     Fig 1.6 No. of steps

convergence in LSA & DVA                         convergence in LSA & DVA

(link change)                                                     (link failure)

Furthermore, we can calculate the average delay each packet required to pass the destination. For that, we can illustrate the computation in Fig1.7 & Fig1.8 when the cost of the link is changed and when the link failure by using the patterns for the average time delay. Fig1.7 illustrates that there is no big difference in time delay between the two algorithms in case of link change and Figure 1.8 illustrate the results of time delay between the two algorithms in case of link failure.  Furthermore, these Figures below illustrate that the DVA is better than the LSA.

The rule for the average time delay.

Di=DepartTimei -AravedTimei                               0<i< No. of packets

For that, the total delay can be summed as:

TDi=i=0NgDi                                   4<j<N, N = No. of Nodes

The average delay can account as:

ADi = DTiNj                           Nj = No. of packets sent from the node j

Fig 1.7 (ADT) for LSA & DVA                       Fig 1.8 (ADT) for LSA & DVA

(Link change)                                                    (Link failure)

8. Related Work

Dissimilar features of LSA and DVA protocols are introduced in [Jochen, 1994]. It also mentions alternation to the basic DVA protocol that presents DVA best suitable for the Internet [Keshav 1997]. It introduced a carrying out an analysis of an effective distance vector based on routing protocol. Lately, a great attempt has gone into preparing routing protocols at the Inter domain level further unchanging [Udaya 1992; Labovitz 1998; Craig ,1999].

9. Conclusion

In this paper, to compare between two adaptive routing algorithms, DVA distributed algorithm and LSA centralized algorithm they are based on the most routing algorithms utilities in the internet. The DVA distributed algorithms need some calculation for every node in the network, but centralized algorithms are fast to pass to network changes and have a chance of failure in this algorithm slight. In DVA, each node has local information, but in LSA, each node has general information and also, DVA is utilized to update link case table.  This paper illustrates simulation for dissimilar network sizes as mesh topology with estimate to dissimilar parameters, which are convergence time, number of steps to pass a convergence, average delay. The simulation results illustrate that the DVA is best convergence than LSA and it takes a number of steps to pass convergence, on the other hand, the DVA is better than LSA in average delay and the traffic load by sending and receiving messages.  Moreover, the DVA gives better results than LSA because it does not require storing all topology information as LSA does for mesh topology. Then the results illustrate, DVA has the step by step for this type of a network.

References

Andrew S. Tanenbaum, 2003, "Computer Networks, "Fourth Edition, New jersey, Prentice Hall, Inc.

Bertsekas D. and R. Gallager, 1992, "Data Network," Prentice-Hall Inc, 2nd edition.James F. Kurose & K. W. Ross, 2003, "Computer Networking: A Top-down Approach Featuring the internet," 2nd Ed, Peaison Education Asia.

Comrmen, T.H ; Leiserson, C.E. ; and Rivest, R.L. ; (2002) "An Introduction to        Algorithms," Second Edition, MIT Press, Boston.

Craig Labovitz, Abha Ahuja, and Farnam Jahanian, (1999), " Experimental  study of internet stability and wide-area backbone failures",In FTCS,

Gambiroza, V. ;Yuan, P.; Balzano, L. Liu, Y.; Sheafor, and Knightly, E. (2004),      "Design, Analysis, and Implementation of DVSR: A Fair High-Performance Protocol for packet Rings," In IEEE/ACM Transaction on Network, Vol. 12, No.1.

Jochen Behrens, J.J.Garcia-Luna-Aceves, 1994 ‘Distributed Scalable Routing Based       on Link State Vectors’

Keshav, S.; (1997), "An Engineering Approach to Computer Networks",  Addison-       Wesley Publishing Company.

Labovitz C., Malan, G.  and Jahanian, F. ; 1998, "Internet routing instability",        IEEE/ACM transactions on Networking.

Larry L. Peterson, Bruce S. Davie, (2003), "Computer Networks: A Systems        Approach," 3rd Edition, Morgan Kaufmann, MIT.

Mourtada, Y. (2000) "Routing Basics & Protocol, " Internet draft.

Roch Guerin and Ariel Orda, (2002) " Computing Shortest Paths for any Number of       Hops," In IEEE Transaction on Network, Vol. 10, No. 5.

Udaya Shankar A., Cengiz Alaettinoglu, Ibrahim Matta,and  Klaudia Dussa-Zieger,(1992), "Performance comparison of routing protocols using mars: Distance- Vector versus linkstate", At ACM SIGMETRICS Conference on Performance.

Varadhan, K.; Govindan, R. and Estrin, D.(2000), "Persistent route oscillations in inter- domain routing", Computer Networks.

William Stalling, (2011), "Data and Computer Communications, pearson Education,       Inc., Publishing as Prentice Hall, New Jersey.

Xu Z. and et.al, (1997).  "A more Efficient Distance Vector Routing Algorithm," In proc.IEEE MILCOM '97 Proceeding, Vol.2.