Interfacial Friction Factor in Horizontal and Inclined Annular Two-Phase Flow in Pipes

Ahmed Saib Naji

Electrochemical Engineering Department/College of Engineering/Babylon University

Abstract

In the present work, a simple model to predict the interfacial friction factor in annular two-phase flow is suggested. The experimental data conducted are by two different sources for same operation and design system. The comparison procedure has achieved using the RMS function which is based on the average error between the experimental readings and the theoretical results for the whole used methods included the proposed model. The results have displayed in tabular form and graphically. The comparison reveals that the best performance of the suggested model.

الخلاصة:

في العمل الحالي: تم اقتراح موديل بسيط  لتخمين معامل الاحتكاك التداخلي لجريان ثنائي الطور للجريان الاسطواني.  البيانات العملية استحصلت من مصدرين مختلفين لنفس موائع الجريان ولظروف مختلفة. تمت المقارنة باستخدام دالة (RMS) والتي تعتمد على معدل الخطأ بين القراءات العملية النتائج النظرية لكل الطرق المستخدمة بضمنها الطريقة المقترحة .تم عرض النتائج بأستخدام اسلوب الجداول والرسومات. بينت المقارنة أن الأداء الأفضل للموديل المقترح.

Nomenclatures:                                            The Dimensionless Groups:

A          Cross Section Area        m2               Nvg   Gas velocity number

f           Friction Factor              less                NLv   Liquid velocity number

Pav      Average Pressure          N/m2             NL   Liquid viscosity number

Tav      Average Temperature    oC                Nd    Diameter number

s            Perimeter                       m                 Re    Reynolds number

V          Velocity                         m/s

h          Liquid Leveling              m

Q         Flow rate                        m3/s

fical       Calculated friction factor

fimeas     Measured friction factor

Greek Symbols:                                              Subscripts

δ            Liquid Film Thickness    m                 L      liquid film

τ             Shear Stress                    N/m2           sg      superficial Gas core

μ            Viscosity                         N.s/m2         sL    superficial liquid

ρ            Density                            N/m3            i     interfacial

σ            Surface Tension              N/m             t      translation

φ            inclination angle                                 wg   wall-gas

ε            roughness                                           wL    wall liquid

Superscripts                                                     c       gas-core

*    modified          ≈    dimensionless

Introduction:

The interfacial friction factor represents one of the most significant concepts affect on gas-liquid, two-phase flow in pipe. This factor was treated in numerous studies. When the gas phase flows in contact to the liquid phase there are several ripples or waves will forming which will achieve type of resistance to the flow, this resistance is more similar to the resistance where rigid bodies move on each other. Due to this resistance some of pressure will loosed. In annular flow pattern the gas will flow near the center of the pipe while the liquid will be near to the inside walls, because of the roughness of the wall, the liquid will flow slower than the gas which will be in high velocity. Now, between the two surfaces of the liquid and the gas there is an interfacial shear stress will occur. This stress will try to prevent the gas to flow fast than the liquid. This process will loose the pressure force of the two-phase flow. Therefore the study of this interfacial surface is important to overcome the happened shear stress.

In the literature, there are more than tens of investigators have developed correlations to predict the interfacial friction factor empirically or semi- empirically as: Kowalski (1987), Laurinat et al. (1985), Crowley and Rothe (1986), Lee and Bankoff (1983), Tsiklauri et al. (1979), Eck (1973), Xiao et al. (1990), Paras et al (1994), Spedding and Hand (1997), Ben Asante (2000), Petolaz and Aziz (1998), Vlachos et al (1997), Taitel-Dukler (1976)… etc.

Some investigators used the interfacial friction factor which is developed in stratified flow to operate in annular flow and vice versa such as; Naji, (2004) and (2006).  To this time, no one approached to mechanistic model to predict it, all these correlations are developed empirically used experimental tests and the accuracy of any method is related with the volume of tests. The aim of this work is to develop a mechanistic model to predict it.

Measured Friction Factor:

In Annular flow shown in figure (1), it is possible to summarize the configuration of the flow geometrically. The treating of such flow will be considered as two phases flowing together in form of two-cylindrical shape.

Figure (1): Annular Flow Pattern Configuration

For the gas core stream the force balance:

−AgdPdL−τi si−gAg ρg sin φ =0       ---------------------- (1)

For the liquid film stream the force balance:

−ALdPdL+τi si−τwL sL−gAL ρL sin φ=0      --------------------- (2)

Taitel-Dukler, (1976) proposed that the pressure gradient dPdL will be same in each phase, the combination of equations (1) and (2) will result:

τwLsLAL−τi si 1AL+1Ag+ρL−ρg g sinφ=0      ------------------- (3)

The equation (3) is called the momentum equation of the annular flow pattern and (φ) is the inclination angle of the pipe flow.

Based on experimental information, the interfacial shear stress could be calculated from equation (3) and the friction factor is consequently calculated from the following equation:

fi=2 τiρg Vg2    ------------------------------------------------------------- (4)

The resulted magnitude of equation (4) will be considered as the measured interfacial friction factor (Xiao et al, 1990)

The Used Methods:

The semi-empirical methods which are used in the comparison procedure are outlined in table (1):

Table (1): The available models in the literature

| 1. | Andritsos and Hanratty [1987] | fifwg=1 if Vsg≤Vsgt fifwg=1+15 δL VsgVsgt−1 if Vsg > Vsgt Vsgt=PatmPav |
| --- | --- | --- |
| 2. | Taitel and Dukler [1976] | fifwg=1 |
| 3. | Andrussi and Persen [1987] | fifwg=1+29.7F1−0.360.67δL F1=VgρgρL−ρg1Ag 1g cosθ dALdhL |
| 4. | Baker et al [1988] | εi=34 σρg VL2 if Nwe Nμ≤0.005 εi=170 σNwe Nμ0.5ρg VL2if Nwe Nμ>0.005 |
| 5. | Cheremisinoff and Davis [1979] | fi=0.008+0.00005ReL ReL=ρL QLμLsL+si |
| 6. | Hamersma and Hart [1987] | εi=2.3 δL and Reg. |

| 7. | Hart et al [1989] |  | fifwL=0.00926 ResL0.726 |
| --- | --- | --- | --- |
| 8. | Kim et al. [1985] |  | fi=0.021+0.14 10−5ReL |
| 9. | Linehan [1968] |  | fi=0.021+0.23 10−5ReL |
| 10. | Shoham and Taitel [1984] |  | fi=0.0142 |
| 11. | Kowalski [1987] correlation |  | fi=7.5 ×10−5 HL−0.25 Reg−0.3 ReL0.83 Reg=ρg vg dμg and ReL=ρL vL dμL |
| 12. | Laurinat et al. [1984] |  | fifwg=2+2.5 × 10−5 ResLd 1-HL5/2 |
| 13. | Crowley and Rothe [1986] |  | fifwg=1 if fifwg<1 fifwg= 1+75 HL if fifwg> 1+75 HL |
| 14. | Bendiksen et al. [1989] |  | Δhw = hx + hx2−hy hx = ρg (Vg − VL)24 g (ρL−ρg) cos θ and hy = σg (ρL−ρg) cos θ |
| 15. | Lee and Bankoff (1983) |  | fi=0.012+5.179 × 10−7 ×Reg−Reg if Reg≤Reg fi=0.012+2.694× 10−7 ×Reg−Reg ReL10001.534 if Reg>Reg where: Reg=1.837 × 105 ReL−0.184 |
| 16. | Tsiklauri et al. (1979) |  | fi= 0.0055 + 2.6 × 10−5 ReL where: ReL=ρL QLμLsL + si |
| 17. | Xiao et al. (1990) |  | fi=0.053 Ngv0.23 NLv0.202 Nd−0.46 NL0.076 N=vg 4ρLg σ , N=vL 4ρLg σ N=d ρLg σ and N=μL 4g ρLσ3 |
| 18. | Eck [1973] | fi=0.06252.3 Sg3.715 d + 15Reg2 |  |
| 19. | Ellis and Gay [1954] | fi= 1.29 ReG−0.57 |  |
| 20. | Ben Asante [2000] | fi=0.61 ε0.35 × δ0.61 × Reg−0.52 × ReL0.6+0.32 |  |
| 21. | Wallis [1969] | fi = 0.005 + 1.5 δd |  |
| 22. | Petolas and Aziz [1998] | fifc = 0.24 σρc Vc2 dc0.085 Ref0.305 |  |
| 23. | Vlachos et al [1997] | fi = 0.024 1−HL0.35 ResL0.18 |  |

Present Model:

In the present work, a simplification to the equation (3) has done using the geometrical configuration in figure (1) by using the relations between each term of equation (3) and the thickness of the film zone in the annular flow pattern. From the literature, it is deduced that the inclination angle has a negligible affect on the estimation of the interfacial friction factor, equation (3) has solved yielded for (fi), and it is found that:

fifL = ρL VL2ρg Vg2 dcSL--------------------------------------------- (5)

Experimental Tests:

No experimental apparatus has done in the present work, but all the tests are conducted from tests published in the literature from two different sources as explained in table (2), the used system used the air as the gas phase while the kerosene as the liquid phase with the ranges shown in table (3). The properties of both fluids could be predicted by using the facilities correlations as cited in Abdul-Majeed (1996) in the following:

ρL=832.34−0.8333 Tav

ρg=Pav/0.287273+Tav

μL=0.001 exp0.0664−0.0207 Tav

μg=0.00001(1.7044+0.00613 Tav−0.0000314 Tav2)

σ=27.6−0.09 Tav

Table (2): The used Data

|  | The Source | No of tests | Inclination Angle |
| --- | --- | --- | --- |
| 1 | Abdul-Majeed (1996) | 20 | 0o |
| 2 | Mukherjee-Brill (1979) | 75 | 0o,-5o,-20o.-30o |

#### Table (3): Flow Conditions Ranges

| R | The Property | Minimum | Maximum |
| --- | --- | --- | --- |
| 1 | Superficial gas velocity m/sec | 24.06 | 48.908 |
| 2 | Superficial liquid velocity m/sec | 0.634 | 6.3 |
| 3 | Average Pressure KPa | 377.9 | 603.4 |
| 4 | Average Temperature oC | 21.9 | 47.8 |
| 5 | Liquid Holdup dimensionless | 0.0621 | 0.28 |

The statistical Tools:

To investigate which model or method has accurate prediction of the interfacial friction factor, RMS tool was used for this purpose. This tool measures the error (e) with respect to the reference line with zero error. Moreover, this line could be represented by line inclined with 45o, hence, the accurate prediction must be the nearest to this inclined line.

1. Average Error:

---------------------------- (6) AE=1ni=1nei

2. Absolute Average Error:

--------------------------- (7)AAE=1ni=1nei

3. Root Mean Square based on Error:

------------------------(8)RMS=1ni=1nei 2

ei=fi cal-fi meas         Where

Results and Discussion:

In the present work, the twenty-three methods in table (1) and the new model has programmed to predict the interfacial friction factor and tested with actual magnitude by using the experimental data shown in tables (2) and (3). The results presented by using the Root Mean Square (RMS) which is given by equation (8) and displayed in tables (4) through  table (8). The predicted and measured magnitudes of the interfacial friction factor are presented also graphically into figures (2) through (6).

The tables and figures are displaying the results for each data with single inclination angle except the table (8) which represent the results of the using of the whole data.

Table (4) represents the performance for the whole methods at horizontal data. It is clear that the new model has good results than the others while the correlation of Eck (1973) was the second best method. There methods designed to predict the interfacial roughness as Baker et al. (1988), Hamersma and Hart (1987) and Bendiksen et al. (1989). These methods depend on the equation of Colebrook and White to calculate the friction factor; therefore, they get the same results as shown in all tables of the results.

Figure (2) shows that the prediction of the present model is satisfied the measured interfacial friction factor.

Table (5) displayed the results of the whole methods where using the data with -5o inclination angle. The table reveals that the accuracy of the new model is the best while the correlation of the Ellis and Gay (1954) was the second best method, while figure (3) presents the distribution of the output results of the present model for data with angle of inclination is -5o.

Table (6) shows the results of the whole methods by the using of data with -20o inclination angle; this table shows that the best performance is by the new model while the correlation of Kowalski (1987) was the second best, also, figure (4) displays the excellent estimation of the present model.

Table (7) displays the results of the whole methods where using the data of -30o inclination angle only. It is appear that the performance of the Ellis and Gay (1954) is the best among the others except the new model which is give the best results absolutely; as well as figure (5) shows that the prediction by the present model is more reliable one.

Table (8) displays the results for the whole method and by using the whole data in the testing procedure. The table shows that the best performance is by the new model while the second one is the correlation of Ellis and Gay (1954). The worst results are still by the correlation of Andrussi and Persen (1987), while, figure (6) displays the behavior of the present model estimation, this grope used the whole data (95) and regardless the specialization of the inclination angle.

Conclusions

- All models gave overestimation results to predict of the friction factor except the new model which seems to be underestimation

- The possibility of using the models those developed to operate in stratified flow to operate in annular flow, because of the results of Taitel and Dukler (1976) and Kowalski (1987) which gave best results than the correlation of Laurnat et al (1985) in spite of the firsts had designed for the estimation in stratified flow while the last is designed for estimating in annular flow.

- It is clear that the correlation of Ellis and Gay (1954) is valid for annular flow in 0o, -5o and -30o inclination angles, while the correlation of Kowalski (1987) is valid only for annular flow      in -20o inclination angle.

- Due to the best accuracy of the new model, it is recommended to be valid for estimating in horizontal and downwardly inclined flow.

- By looking to all figures and tables, it is clear that the inclination angle has no significant affect on the estimation of interfacial friction factor, this conclusion supports the assumption of the present model.

References

Abdul-Majeed, G. H.: ”Liquid Holdup in Horizontal Two-Phase Flow”, JPSE, 15,PP.271-280, (1996).

Ahmed S. Naji and Al-Kayiem, H. H.: “Study of Two-Phase Flow in Horizontal and Inclined Pipes”, M. Sc. Dissertation, Mech. Eng. Dept., Coll. of Eng., Al-Mustansiria University, (2001).

Ahmed S. Naji " A Comparison of the Interfacial Friction Factor Methods In Horizontal and Downwardly Inclined Annular Flow in Pipes", Journal of Babylon University, Vol. 14 , No. 5 ,(2006).

Ahmed S. Naji, "A Comparison of the Interfacial Friction Factor Correlations in Horizontal and Downwardly Inclined Stratified Flow in Pipes", Journal of Babylon University, Vol. 10 , No. 5  (2004).

Ahmed S. Naji "A New Model to predict the Interfacial Friction Factor In Stratified two-phase Flow in Pipes", Journal of Babylon University, Vol. 17, No. 4 (2009).

Alves, I. N., Caetano, E. F., Minami, K. and Shoham, O.:” Modeling Annular Flow Behavior for Gas Wells”, ASME J, Chicago, No. 2, (1988).

Andritsos, N. and Hanratty, T. J.: “Interfacial Instability for Horizontal Gas-Liquid Flow in Pipes,” In. J. Multiphase Flow 13, No. 5,583-603, (1987).

Andrussi, P. and Persen, L. N.: ”Stratified Gas-Liquid Flow in Downwardly Inclined Pipes,” In. J. Multiphase Flow 13, No. 4,565-575, (1987).

Baker, A., Nielsen, K. and Gabb, A.:” Pressure Loss Liquid Holdup Calculations Developed,” Oil and Gas J, 55-59, (1988).

Brill, J. P. and Beggs, H. D.: “Study of Two-Phase Flow in Pipes”, fifth Edition, The University of Tulsa, USA., (1986).

Ben Asante, " Two Phase Flow: Accounting for the Presence of Liquids in Gas Pipeline Simulation", Enron Transportation Services, Houston, Texas, USA, (2000).

Bendiksen, K. H. “An Experimental Investigation of the Motion of Long Bubbles in Inclined Pipes,” Int. J. Multiphase Flow, 10, pp 1-12 (1984).

Cheremisinoff, N.P., and Davis, E. J.: ”Stratified Turbulent-Turbulent Gas-Liquid Flow,” AIChE. J.25, No.1, 48-56,(1979).

Crowley, C. J. and Rothe, P. H.:”State of Art Report on Multiphase Methods for Gas and Oil Pipelines”, AGA Pipelines Research Committee Project PR-172-609, Dec., (1986).

Hamersma, P. J. and Hart, J.:”A Pressure Drop Correlation for Gas/Liquid Pipe Flow with a Small Liquid Holdup,” Chem. Eng. Sci.42, No. 5, 1187-1196, (1987).

Hart, J., J. Hamersma, & J. M. Fortuin, J. M. " Correlations Predicting Frictional Pressure Drop and Liquid Holdup During Horizontal Gas-Liquid Pipe Flow With A Small Liquid Holdup", Int. J. Multiphase Flow 15, 947-964 (1989).

Eck, B. Technische Stromunglehre. Springer, New York (1973).

Ellis, S. R. M. & B. Gay, " The Parallel Flow of TwoFluid Streams: Interfacial Shear and Fluid-Fluid Interation", Trans. Inst. Chem Engrs, Vol 37 (1959).

Kim, H. J., Lee, S. C., and Bankoff, S. G.:” Heat Transfer and Interfacial Drag in Countercurrent Stream-Water Stratified Flow,” In. J. Multiphase Flow 11, No. 5,593-606, (1985).

Kowalski, J. E.:”Wall and Interfacial Shear Stresses in Separated Flow in a Horizontal Pipe”, AIChE J.33, No. 2, 274-281, (1987).

Laurinat, J. E., Hanratty, T. J. and Dallman, J. C.: ”Pressure Drop and Film Height Measurements for Annular Gas-Liquid Flow”, Int. J. Multiphase Flow 10, No. 3, 341-356, (1984).

Linehan, J. H.: ”The Interaction of Two-Dimensional, Stratified, Turbulent Air-Water and Stream-Water Flow,” Ph. D. Dissertation, Department of Mechanical Engineering, Wisconsin University, (1968).

Lee, S. C. and Bankoff, S. G.:” Stability of Steam-Water Countercurrent Flow in an Inclined Channel:Flooding”, J. of Heat Transfer 105, 713-718, (1983).

Mukherjee, H.: “An Experimental Study of Inclined Two-Phase Flow”, Ph. D. Dissertation, the University of Tulsa, (1979).

Paras, S. V., Vlachos,N. A. & Karabeles, A. J., " Liquid layer characteristics in stratified atomization flow", Int J Multiphase Flow, Vol. 20, No. 5, 1994.

Petolas, N. and Aziz, K.: " A mechanistic model for multiphase flow in pipes", paper no. 98-39, Annual Technical meeting of the petroleum society of the Canadian institute of mining, metallurgy and petroleum, Calgary, Alberta, Canada, (1998).

Oliemans, R. V. A., Pots, B. F. and Trope, N. :” Modeling of annular Dispersed Two-Phase Flow in Vertical Pipes”, Int. J. Multiphase Flow 12, No. 5, 711-732, (1986).

Smith, T. N. and Taitel, R. W.: “Interfacial Shear Stress and Momentum Transfer in Horizontal Gas-Liquid Flow”, Chem. Eng. Sci., 21, 63-75, (1966).

Spedding, P. L. & N. P. Hand" Prediction in Stratified Gas-Liquid Co-Current Flow in Horizontal Pipelines", Int Journal of Heat Mass Transfer, 40, No. 8, (1997).

Shoham, O. and Taitel, Y.:” Stratified Turbulent-Turbulent Gas-Liquid Flow in Horizontal and Inclined Pipes,” AIChE J.30, No.3, 377-385, (1985).

Taitel, Y. and Dukler, A. E.: “A Model for Predicting Flow Regimes Transitions in Horizontal and near Horizontal Gas-Liquid Flow”, AIChE J.22, No.1, 47-55, (1976).

Tsiklauri, G. V., Besfamiliny, P. V. and Baryshev, Y. V:” Experimental Study of Hydrodynamic Processes for Wavy Water Film in a Cocurrent Air Flow”, Two-Phase Momentum, Heat and Mass Transfer, Vol. 1, 357-372, Hemisphere, New York, USA, (1979).

Vlachos et al. :" A mechanistic model for stabilized multiphase flow in pipes", Technical report for members of the reservoir simulation institute affiliates program (SUPRI-B) and horizontal well industrial affiliates program (SUPRI-HW), Stanford University, CA, (1997).

Wallis, G.B.:“One Dimensional Two-Phase Flow”, McGraw Hill, (1969).

Xiao, J. J., Shoham, O. and Brill, J. P.: “ A Comprehensive Mechanistic Model for Two-Phase Flow in Pipes”, 65th Ann. Tech. Conf., Soc. Pet. Eng., New Orleans, LA, Pap. SPE 20631, PP.14, (1990).

Table (4):The results when using Horizontal Data Only (40 points)

| RMS 10-4 | AAE 10-4 | AE 10-4 | The model | R |
| --- | --- | --- | --- | --- |
| 1202 | 190 | 190 | Andritsos-Hanratty [1987] | 1 |
| 11.9 | 1.89 | 1.89 | Taitel-Dukler [1976] | 2 |
| 14513 | 2294 | 2294 | Andrussi-Persen [1987] | 3 |
| 52.4 | 8.29 | 8.29 | Baker et al [1988] | 4 |
| 240 | 38 | 38 | Cheremisinoff-Davis [1979] | 5 |
| 52.4 | 8.29 | 8.29 | Hamersma-Hart [1987] | 6 |
| 448 | 70.8 | 70.8 | Hart et al [1989] | 7 |
| 49.1 | 7.77 | 7.77 | Kim et al. [1985] | 8 |
| 2645 | 418 | 418 | Linehan [1968] | 9 |
| 22.4 | 3.54 | 3.54 | Shoham-Taitel (1984) | 10 |
| 18.0 | 2.86 | 2.86 | Kowalski(1987) | 11 |
| 210 | 33.2 | 33.2 | Laurinat et al. (1984) | 12 |
| 68.3 | 10.8 | 10.8 | Crowley- Rothe (1986) | 13 |
| 52.4 | 8.29 | 8.29 | Bendiksen et al. [1989] | 14 |
| 4075 | 644 | 644 | Lee and Bankoff (1983) | 15 |
| 305 | 48.2 | 48.2 | Tsiklauri et al. (1979) | 16 |
| 73.6 | 11.6 | 11.6 | Xiao et al. (1990) | 17 |
| 9.69 | 1.53 | 1.53 | Eck (1973) | 18 |
| 11.8 | 1.87 | 1.87 | Ellis and Gay [1954] | 19 |
| 114 | 18.1 | 18.1 | Ben Asante (2000) | 20 |
| 1169 | 184 | 184 | Wallis (1969) | 21 |
| 8470 | 1339 | 1339 | Petolas and Aziz (1998) | 22 |
| 13.9 | 2.2 | 2.2 | Vlachos et al (1997) | 23 |
| 9.7x10-4 | 1.5x10-4 | -1.5x10-4 | New Model (present) | 24 |

Figure (2): The Prediction of the Present Model for Horizontal Data Only

Fi meas   10-6

Fi cal   10-6

Table (5):The results when using Inclined Data with -5o Only (17 points)

| RMS 10-4 | AAE 10-4 | AE 10-4 | The model | R |
| --- | --- | --- | --- | --- |
| 859 | 113 | 113 | Andritsos-Hanratty [1987] | 1 |
| 9.17 | 1.21 | 1.21 | Taitel-Dukler [1976] | 2 |
| 8992 | 1191 | 1191 | Andrussi-Persen [1987] | 3 |
| 39.1 | 5.19 | 5.19 | Baker et al [1988] | 4 |
| 542 | 71.8 | 71.8 | Cheremisinoff-Davis [1979] | 5 |
| 39.1 | 5.19 | 5.19 | Hamersma-Hart [1987] | 6 |
| 638 | 84.5 | 84.5 | Hart et al [1989] | 7 |
| 65.0 | 8.61 | 8.61 | Kim et al. [1985] | 8 |
| 6137 | 812 | 812 | Linehan [1968] | 9 |
| 18.7 | 2.48 | 2.48 | Shoham-Taitel (1984) | 10 |
| 41.9 | 5.55 | 5.55 | Kowalski(1987) | 11 |
| 464 | 61.4 | 61.4 | Laurinat et al. (1984) | 12 |
| 58.7 | 7.78 | 7.78 | Crowley- Rothe (1986) | 13 |
| 39.1 | 5.19 | 5.19 | Bendiksen et al. [1989] | 14 |
| 981 | 130 | 130 | Lee and Bankoff (1983) | 15 |
| 699 | 92.5 | 92.5 | Tsiklauri et al. (1979) | 16 |
| 71.7 | 9.49 | 9.49 | Xiao et al. (1990) | 17 |
| 14.3 | 1.9 | 1.90 | Eck (1973) | 18 |
| 7.77 | 1.03 | 1.03 | Ellis and Gay [1954] | 19 |
| 131 | 17.4 | 17.4 | Ben Asante (2000) | 20 |
| 954 | 126 | 126 | Wallis (1969) | 21 |
| 9266 | 1227 | 1227 | Petolas and Aziz (1998) | 22 |
| 25.2 | 3.34 | 3.34 | Vlachos et al (1997) | 23 |
| 7.6x10-3 | 1.0x10-3 | -1.0x10-3 | New Model (present) | 24 |

Figure (3): The Prediction of the Present Model for Data with -5o.

Fi meas   10-6

Fi cal   10-6

Table (6):The results when using Inclined Data with -20o (25 points)

| RMS 10-4 | AAE 10-4 | AE 10-4 | The model | R |
| --- | --- | --- | --- | --- |
| 209 | 23.0 | 23.0 | Andritsos-Hanratty [1987] | 1 |
| 10.6 | 1.17 | 1.17 | Taitel-Dukler [1976] | 2 |
| 5841 | 645 | 645 | Andrussi-Persen [1987] | 3 |
| 52.9 | 5.85 | 5.85 | Baker et al [1988] | 4 |
| 67.4 | 7.45 | 7.45 | Cheremisinoff-Davis [1979] | 5 |
| 52.9 | 5.85 | 5.85 | Hamersma-Hart [1987] | 6 |
| 201 | 22.2 | 22.2 | Hart et al [1989] | 7 |
| 27.2 | 3.01 | 3.01 | Kim et al. [1985] | 8 |
| 689 | 76.0 | 76.0 | Linehan [1968] | 9 |
| 15.6 | 1.73 | 1.73 | Shoham-Taitel (1984) | 10 |
| 9.71 | 1.07 | 1.07 | Kowalski(1987) | 11 |
| 96.5 | 10.6 | 10.6 | Laurinat et al. (1984) | 12 |
| 60.6 | 6.69 | 6.69 | Crowley- Rothe (1986) | 13 |
| 52.9 | 5.85 | 5.85 | Bendiksen et al. [1989] | 14 |
| 254 | 28.0 | 28.0 | Lee and Bankoff (1983) | 15 |
| 82.3 | 9.09 | 9.09 | Tsiklauri et al. (1979) | 16 |
| 33.7 | 3.72 | 3.72 | Xiao et al. (1990) | 17 |
| 10.3 | 1.14 | 1.14 | Eck (1973) | 18 |
| 16.6 | 1.84 | 1.84 | Ellis and Gay [1954] | 19 |
| 83.0 | 9.16 | 9.16 | Ben Asante (2000) | 20 |
| 804 | 88.8 | 88.8 | Wallis (1969) | 21 |
| 5255 | 580 | 580 | Petolas and Aziz (1998) | 22 |
| 12.1 | 1.34 | 1.34 | Vlachos et al (1997) | 23 |
| 2.6x10-3 | 2.9x10-4 | -2.9x10-4 | New Model (present) | 24 |

Figure (4): The Prediction of the Present Model for Data with -20o

Table (7):The results when using Inclined Data with -30o (13 points)

| RMS 10-4 | AAE 10-4 | AE 10-4 | The model | R |
| --- | --- | --- | --- | --- |
| 554 | 56.8 | 56.8 | Andritsos-Hanratty [1987] | 1 |
| 7.26 | 0.74 | 0.74 | Taitel-Dukler [1976] | 2 |
| 6648 | 682 | 682 | Andrussi-Persen [1987] | 3 |
| 31.1 | 3.19 | 3.19 | Baker et al [1988] | 4 |
| 177 | 18.2 | 18.2 | Cheremisinoff-Davis [1979] | 5 |
| 31.1 | 3.19 | 3.19 | Hamersma-Hart [1987] | 6 |
| 319 | 32.7 | 32.7 | Hart et al [1989] | 7 |
| 33.4 | 3.42 | 3.42 | Kim et al. [1985] | 8 |
| 1964 | 201 | 201 | Linehan [1968] | 9 |
| 14.5 | 1.49 | 1.49 | Shoham-Taitel (1984) | 10 |
| 16.7 | 1.71 | 1.71 | Kowalski(1987) | 11 |
| 171 | 17.6 | 17.6 | Laurinat et al. (1984) | 12 |
| 46.5 | 4.77 | 4.77 | Crowley- Rothe (1986) | 13 |
| 31.1 | 3.19 | 3.19 | Bendiksen et al. [1989] | 14 |
| 546 | 56.0 | 56.0 | Lee and Bankoff (1983) | 15 |
| 226 | 23.2 | 23.2 | Tsiklauri et al. (1979) | 16 |
| 47.8 | 4.90 | 4.90 | Xiao et al. (1990) | 17 |
| 12.2 | 1.25 | 1.25 | Eck (1973) | 18 |
| 6.39 | 0.656 | 0.656 | Ellis and Gay [1954] | 19 |
| 70.7 | 7.26 | 7.26 | Ben Asante (2000) | 20 |
| 735 | 75.4 | 75.4 | Wallis (1969) | 21 |
| 5612 | 575 | 575 | Petolas and Aziz (1998) | 22 |
| 18.0 | 1.84 | 1.84 | Vlachos et al (1997) | 23 |
| 2.7x10-3 | 2.8x10-4 | -2.8x10-4 | New Model (present) | 24 |

Figure (5): The Prediction of the Present Model for Data with -30o

Fi meas   10-6

Fi cal   10-6

Table (8): The results when using The overall Data (95 points)

| RMS 10-4 | AAE 10-4 | AE 10-4 | The model | R |
| --- | --- | --- | --- | --- |
| 554 | 56.8 | 56.8 | Andritsos-Hanratty [1987] | 1 |
| 7.26 | 0.745 | 0.745 | Taitel-Dukler [1976] | 2 |
| 6648 | 682 | 682 | Andrussi-Persen [1987] | 3 |
| 31.1 | 3.19 | 3.19 | Baker et al [1988] | 4 |
| 177 | 18.2 | 18.2 | Cheremisinoff-Davis [1979] | 5 |
| 31.1 | 3.19 | 3.19 | Hamersma-Hart [1987] | 6 |
| 319 | 32.7 | 32.7 | Hart et al [1989] | 7 |
| 33.4 | 3.42 | 3.42 | Kim et al. [1985] | 8 |
| 1964 | 201 | 201 | Linehan [1968] | 9 |
| 14.5 | 1.49 | 1.49 | Shoham-Taitel (1984) | 10 |
| 16.7 | 1.71 | 1.71 | Kowalski(1987) | 11 |
| 171 | 17.6 | 17.6 | Laurinat et al. (1984) | 12 |
| 46.5 | 4.77 | 4.77 | Crowley- Rothe (1986) | 13 |
| 31.1 | 3.19 | 3.19 | Bendiksen et al. [1989] | 14 |
| 546 | 56.0 | 56.0 | Lee and Bankoff (1983) | 15 |
| 226 | 23.2 | 23.2 | Tsiklauri et al. (1979) | 16 |
| 47.8 | 4.90 | 4.90 | Xiao et al. (1990) | 17 |
| 12.2 | 1.25 | 1.25 | Eck (1973) | 18 |
| 6.39 | 0.656 | 0.656 | Ellis and Gay [1954] | 19 |
| 70.7 | 7.26 | 7.26 | Ben Asante (2000) | 20 |
| 735 | 75.4 | 75.4 | Wallis (1969) | 21 |
| 5612 | 575 | 575 | Petolas and Aziz (1998) | 22 |
| 18.0 | 1.84 | 1.84 | Vlachos et al (1997) | 23 |
| 2.7x10-3 | 2.8x10-4 | -2.8x10-4 | New Model (present) | 24 |

Figure (6): The distribution of the prediction interfacial friction factor

by using the overall data (horizontal and inclined tests)

Fi meas   10-6

Fi cal   10-6