Hydraulic Control Of Shatt Al - Hilla Within Hilla City

Abdul Hadi A. Al–Delewy		Salah Tawfeek Ali

Nariman Yahya Othman

Civil engineering Department, Engineering College, Babylon University

Abstract

The distance on Shatt Al–Hilla between (km 32.000) and (km 51.500), denoted hereafter as "the reach", is where it crosses Hilla City. The approved development plan of Shatt Al - Hilla involves increasing its design discharge from (230 m3/s) to (303 m3/s). This will cause raising its normal water level within the reach by about (0.73 m) which will endanger some important localities in the city.

The research aims at examining the extent of effectiveness of some practical physical and hydraulic measures to control the flow in the reach, so that the expected danger will be minimized, if not eliminated.

The examined measures were deepening the reach, increasing the side slope, increasing the bed width, increasing the bed slope, decreasing the equivalent Manning’s (n), a combination of increments in bed width, flow depth, and side slope, and taking use a proposed cross regulator at the upstream end of the reach and the operation of Dora cross – regulator at the downstream end as hydraulic control measures.

The research showed that the most appropriate practical solutions, from a hydraulic point of view, would be deepening the reach by (0.35 m) or decreasing Manning’s (n) to (0.0294) or constructing the proposed cross regulator at km (32.200) and operating  it such that the (303 m3/sec) discharge will produce a water level downstream the regulator of (29.00 m.a.s.l.).

الخلاصة

تضمنت منطقة الدراسة المسافة الواقعة على شط الحلة بين (كم32.000 وكم 51.500 ).أن الخطة التوسعية لشط الحلة تتضمن زيادة التصريف من (230 م3/ثا) ألى (303 م3/ثا). هذا التوسع سوف يؤدي الى رفع منسوب ماء الشط ضمن المنطقة المدروسة بمقدار(0.73 م) مما يهدد بعض المناطق المهمة في المدينة.

يهدف البحث الى أختبار فاعلية بعض الأجراءات الفيزيائية والهيدروليكية العملية للسيطرة على جريان الشط للتقليل من الضرر المتوقع أن لم يكن أنهاؤه.

أن الأجراءات المختبرة هي تعميق المنطقة المدروسة، زيادة الميل الجانبي للشط، زيادة عرض القعر،  زيادة ميل القعر، تقليل معامل الخشونة المكافيء، زيادة في مجموعة من العوامل معا (عرض القعر و عمق الجريان و الميل الجانبي) وأستخدام ناظم مقترح مقدم المنطقة المدروسة وأخيراً تشغيل ناظم دورة المنشأ أصلا.

لقد بين البحث بان أكثر الحلول عملية من وجهة نظر هايدروليكية هي إما تعميق المنطقة بمقدار(0.35)م أو تقليل معامل الخشونة إلى(0.0294) أو أنشاء ناظم قاطع عند كم (32,200) وتشغيله بحيث أنه عند تصريف (303م3\ثا) يكون منسوب الماء مؤخر الناظم(29.00).

1- Introduction

Shatt Al-Hilla is the main channel that branches from the left side of the Euphrates River just at the upstream of the New Hindiya Barrage. It starts from its head regulater, Km 0.000, crosses Hilla City, and ends at the Daghara head regulator, Km 101.000(BWRD, 1998).The part of Shatt Al -Hilla within Hilla City, from Km 32.000 to Km 51.500, will be denoted hereafter as " the reach".

The approved development plan of Shatt Al - Hilla involves increasing its design discharge from (230 m3/s) to (303 m3/s)(F.C, 1999). This will cause raising its normal water level within Hilla City by about (0.73 m) which will endanger some important localities in the city.

This paper aims at finding appropriate feasible and practical measures to control the flow in the reach so that the expected danger of flooding will be minimized, if not eliminated.

2- Mathematical Model

The flow in the reach is represented mathematically by the Saint-Venant equations (continuity and momentum).The full forms of these equations, respectively, are:

∂ Q∂ x+∂ (A+A∘)∂ t±q=0 [1]

∂ Q∂ t+∂ (Q2A)∂ x+gA ∂ h∂ x+Sf+Se−qVx+WfT=0 [2]

where: Q = discharge, (L3 / T); x = distance along the channel, (L); A = cross – sectional area of flow, (L2); Ao = off–channel cross–sectional area where inflow velocity is considaered negligible, (L2); t = time, (T); q = lateral inflow (positive) or outflow (negative), (L3 / T / L); g = gravity – acceleration constant, (L / T2); h = water surface elevation, (L), (h = y + z); y = water depth, (L), z = channel bed elevation, (L); Sf = friction slope as defined in Manning's equation, (dimensionless); Se = eddy loss slope; Vx = velocity of lateral inflow or outflow in the x-direction, (Vx = q / A); Wf: wind shear force, (F); and T = width of the water surface, (L).

The full Saint – Venant equations are too complicated to be solved analytically. Therefore, numerical solutions are always recommended. Of the several numerical methods of solution in use, the four point implicit finite – difference scheme has been adopted(Othman, 2006).

On using the area of flow, (A), neglecting the off – channel cross – sectional area, (Ao), and replacing, (δ Aδ t) by (Tδ yδ t), where T = δ Aδ y, in Eq. (1); moreover, by neglecting eddy losses and wind shear effect in Eq. (2); then the finite – difference equations are used to replace the derivatives and other variables in Eqs. (1) and (2), and after rearranging, the following weighted, four – point implicit, finite – difference equations are obtained:

Ci:Δ tTij+1 Δxθ (Qi+1j+1−Qij+1)+ (1−θ) (Qi+1j−Qij)  +12 Ti+1j+1Tij+1(yi+1j+1−yi+1j)+(yij+1−yij) ±qΔ tTij+1=0                  [3]

Mi:0.5 Δ xΔ t g Qi+1j+1Ai+1j+1 + Qij+1Aij+1 − Qi+1jAi+1j − QijAij +12 gθ  (Qi+1j+1Ai+1j+1)2−(Qij+1Aij+1)2 +(1−θ) (Qi+1jAi+1j)2−( QijAij)2  +qΔ x2 g θ  Qi+1j+1(Ai+1j+1)2−Qij+1(Aij+1)2 +(1−θ) Qi+1j(Ai+1j)2− Qij(Aij)2 +Δ x θ (yi+1j+1+zi+1j+1 −yij+1−zij+1 +Sfj+1)+               [4]

where: : weighting factor for two time positions of an x-location. Ci and Mi denote the continuity and the momentum equations, respectively. {More details for solving these equations can be found in (Othman, 2006)}.

3-The Case Study

The aforementioned "reach" has been taken as the case study for this research. It constitutes a proposed cross regulator at km (32.200), four branch canals, an intake by pumping, and the existing Dora cross regulator at km (51.100); the basic information concerning these constituents are given in Table (1).

In the model, the river system is divided into a number of subreaches or segments which are joined by nodes. The unsteady flow equations (continuity and momentum) are written for different sections of each subreach. These equations are then solved simultaneously, subject to the boundary and initial conditions and the special conditions imposed by nodes. Accordingly, the reach has been divided into (65) cross – section as shown in Fig. (1).

The upstream boundary condition used in the model is an observed stage hydrograph, Fig. (2). The values of this stage hydrograph are measured at daily intervals for a period of fifteen days. However, when model requirement demanded stage at shorter time intervals, a linear interpolation was used. All the field data are taken from (BWRD, 2004).

The downstream boundary condition used is a rating curve at gaging station (No. 3). The rating curve is shown in Fig. (3), based on data from (BWRD, 2004). Data were fitted by computer and the respective best fit polynomial was:

QN=C∘+C1 yN+C2 yN2+C3 yN3+C4 yN4 		                    [5]

where: QN = discharge at the N- th cross – section, (L3 / T); yN = water depth at the N- th cross – section, (L); Co, C1, C2, C3, C4 = regression coefficients.

The respective values of the constants (with a determination coefficient: R2= 0.9917) were: Co = – 3.379, C1 = 11.486, C2 =8.8145, C3 = 0.0225, C4= – 0.0044. The equation is valid only for values of depth (y) in the range (0.85 - 6.00 m). The rating curve equation is based on the assumption that Dora regulator is fully opened.

For the mathematical model, the derivatives of the downstream boundary equation are needed; therefore, Eq. (5) may be written as:

DB (yNj+1 , QNj+1)=QN−[C∘+C1 yN+C2 yN2+C3 yN3 +C4 yN4]  		[6] where:DB (yNj+1 , QNj+1)=the residual from downstream boundary equation.

The partial derivatives become:

δ DBδ yN=− C1 − 2 C2 yN − 3 C3 yN2 − 4 C4 yN3                           		 [7]

δ DBδ QN=1                                     		                         		[8]

However, in this research the initial condition is considered as a steady flow condition. Then [hi (or yi) and Qi] at each (ith) cross section at time (j = 0), are calculated by using equations (9) and (10). Equation (9) is used to calculate the discharge (Qi) for each cross – section in the reach, whereas Eq. (10) is used to obtain the value of (hi+1):

Qi+1=Qi−qi Δxi                                                              [9]

hi+1=(Q2A)i−(Q2A)i+1g×Ai+hi+Δxi×Sf                          [10]

Moreover, Eq. (10) is used to calculate the initial water elevations considering the two cross regulators at the extremities of the reach are not existing because this equation is not applicable when there is a hydraulic structure on the river.

The unsteady flow equations are solved for several time steps using the initial conditions together with the relevant boundary conditions which are held constant. This allows the results to become error free when the actual simulation commences and transient boundary conditions are used (Fread & Lewis, 1998). Accordingly, the model has been run for (15 day) to eliminate the effects of inaccuracies of the initial conditions. The running ends when the values of the computed stages and discharges at the end of two consecutive  time steps are the same. Figures (4) and (5) represent the initial conditions calculated by the model.

4- Calibration of the mathematical model

The calibration means the right choice of the model parameters (Δt,θ and n) which affect the stability and convergence of the solution.

A set of values (that sound practically feasible) for each of the three aforementioned parameters has been chosen. Then, the set mathematical model was run, using the various possible combinations of the chosen values. Accordingly, the set of values of (n) which are used in the calibration were (0.026 – 0.035) step (0.02) with values of (Δt) (0.5 – 48 hr) and values of (θ) (0.5 – 1.0) step (0,05).The total number of tested combinations was (714).

The Calibration process showed that the appropriate values of (Δt),(θ), and (n) are (24 hr), (0.96), and (0.032), respectively; these values gave good agreement between the computed and the observed hydrographs as shown in Tables (2, 3, and 4).

5- Applications

The following control measures have been tested:

- Deepening the reach while keeping the existing bed width (B) and the side slope (Z).

- Increasing side slope (Z) while keeping the existing depth (Yo) and the bed width (B).

- Increasing bed width (B) while keeping the existing depth (Yo) and the side slope (Z).

- Increasing the bed slope while keeping the existing depth (Yo), bed width (B), and side slope (Z).

- Changing Manning’s (n) through an appropriate modification of the bed surface (such as some suitable under – water lining) while keeping all the other existing parameters unchanged.

- Some appropriate practical combinations of the aforementioned controls.

- Using a cross – regulator at (km 32.200) as an upstream control device on the reach.

- Testing the effect of Dora regulator as a downstream control device on the considered reach.

The tests were performed while considering the maximum discharge is (303 m3/sec) and all the values of the other parameters are fixed [Δt= 24 hr, θ = 0.96, n = 0.032, and bed slope (So) 7.5 cm/km] for each case.

6- Results

6-1 Deepening the reach

To make control on the reach considered in this research, different values of bed levels for each cross section were taken and used in the program. These values were less than the values of the existing bed levels by (0.05 to 0.40 m, step 0.05m). Table (5) shows the results for each bed level at km (39.000), (city center), and km (51.100), (just upstream of Dora regulator).

It is found that lowering the bed level by (0.35 m) will give water level at km (39.000) of (28.41 m. a. s. l.) and a water level downstream the reach, (km 51.100), of (27.47 m. a. s. l.), which means that the lowering in the water level is about (0.73 m).

6-2 The side slope (Zi) ( H:V )

Using different values of side slope (Zi)  as listed in Table (6), the results computed from the model for changing the side slope (Zi) are listed in Table (7).

It is found that non of these values lowered the water levels for (0.73 m); at km (39.000) the lowering is  (0.15 m) and at downstream, km(51.100), is (0.39 m)  where (Zi=6) for (ΔZi=3)and (Zi=2) for (ΔZi=2).The cause of chosing the changing in the side slpoe (ΔZi=2) is that the width of the water surface will increase about (10 m) inside  Hilla City and this is the maximum that can be used because of the bulidings between km(36.000 - 42.000) while  (ΔZi=3) for other parts of the reach may be used because the surroundings are agricultural lands.

6-3 The bed width (Bi)

The bed width (Bi) is changed while keeping the other parameters in the model at the standardized values. Table (8) shows the values of bed width used in the model.

The increasing in the bed width (∆Bi) was [(5, 10, 15, 20, and 25) m].  It is found that increasing the bed width by (20 m) will decrease the water level at km (39.000) by (0.24 m) and at km(51.100), by (0.74 m). Such a result is efficient at (km 51.100) but not at (km 39.000) unless it is accompanied by raising the embankment around (km 39.000) by (0.50 m). Figure (6) shows the effect of increasing the bed width on the water level at the downstream end of the reach. The water levels computed at km(39.000) and km(51.100) are shown in Table (9).

6-4 The bed slope (So):

The model is run using different values of bed slope (So) while fixing the values of the other parameters in the model, namely, (Bi, Zi, Δt, θ, n).

The values which are input to the model for the bed slope are [(7.5, 8, 9, 10, 11) cm/km]. The results of changing the bed slope are listed in Table (10).

It is found that increasing the bed slope by (3.50 cm/km) will decrease the water level at km (39.000) by (0.23 m) and at km(51.100) by (0.68 m); such a result is efficient at (km 51.100) but not at (km 39.000) unless it is  accompanied by raising the embankment around (km 39.000) by (0.50 m). Howover, adopting this proposal will require the bed level of Dora regulator to be (22.42 m. a. s. l.) instead of the existing bed level which is (23.04 m. a. s. l.).

6-5 Manning’s (n):

For the research, a stage hydrograph was used as an upstream boundary condition in the model because all the available data for the considered reach were measures of water levels. However, as a special case in this research, a discharge hydrograph was fabricated by personal connection with (BWRD) to use it as an upstream boundary condition to study the effect of changing Manning's (n) as one of the proposed control measures while fixing all the other parameters in the model, namely, [Bi, Zi, Δt, θ and So] for (Q = 303 m3 / sec.) at the head regulator. The results are listed in Table (11). The results showed that (n = 0.0294) will attain the goal by lowering the water surface profile throughout the reach by (0.72 m).

It is worth mentioning that the (n) referred to herein is not the theoretical Manning’s (n) but an equivalent (n) that incorporates all in – situ factors causing the channel roughness.

6-6 Appropriate practical combinations of the aforementioned controls

Of the many combinations that may be selected for the considered  control parameters, the following three, reasonably- practical, combinations have been considered as an indicator guideline, with the design values of bed width, bed level, and side slope being the basis for the proposed changes:

- Increasing the bed width by (5 m),lowering the bed level by (0.05           m), and increasing the side slope by (0.25).

- Increasing the bed width by (10 m),lowering the bed level by (0.10 m), and increasing the side slope by (0.50).

- Increasing the bed width by (15 m),lowering the bed level by (0.15 m), and increasing the side slope by (1.00).

The results of the aforementioned combinations are summarized in Table (12).

6-7 The proposed cross – regulator:

As an upstream control device, a cross – regulator is proposed at km (32.200) with a sill level equal to (24.10 m.a.s.l.) and (6) gates with a width equal to (9 m) for each gate(Al-Msaudi, 2001).

The program is run for different values of water level downstream the cross – regulator and as follows:

Case(1) : 29.60 m.a.s.l.

Case(2) : 29.30 m.a.s.l.

Case(3) : 29.00 m.a.s.l.

Table (13) shows the water levels for each case at km (39.000) and at km (51.100). It is found that when the water level downstream the cross regulator is (29.00 m.a.s.l.) the water level will be lower by (0.67 m) at km (39.000) and by (0.71 m) at km (51.100). Therefore, the proposed cross regulator at (km 32.200) shuold be operated such that when the flow discharge is (303 m3/sec.) the water level just downstream the regulator will not be higher than (29.00 m.a.s.l.).

6-8 Dora Cross – Regulator

The designed parameters of the existing Dora cross – regulator are mentioned in (Table 1). By running the model for (Q = 303 m3 /sec.) for (15 day) and with fixing all the values of the other parameters, it is found that the water level would be (28.24 m.a.s.l.) at km (51.100) when all the gates of Dora cross – regulator are fully opened. When the model is run for the mentioned case with different values of gates openning of Dora regulator [ and, consequently, producing water level downstream of the regulator lower than (28.24 m.a.s.l.), e.g., (28.00 m.a.s.l.)], it is found that the water levels along the reach will be raised where the upstream water level will be (29.70 m.a.s.l.). Therefore, when Dora cross – regulator is operated for the case that the design flow in Shatt Al –Hilla at the intake is (303 m3 /sec.), its upstream water level should not be higher than (28.24 m.a.s.l.); otherwise, it will create back water effect that may endanger the surroundings of the reach.

7- Anlysis of the results

The research showed that:

1.  Deepening the reach by (0.35 m) will lower the water level at km (39.000) by (0.67 m) and at km(51.100) by (0.73 m), which is acceptable.

- Increasing the side slope (Zi) ( H:V )from (0) to (2) for (km36.000 - km42.000) and from (3) to (6) for the rest of the reach  will lower the water level at km (39.000) by (0.15 m) and at km (51.100) by (0.39 m), which is not acceptable in this research.

- Increasing the bed width by (20 m)will lower the water level at km (39.000) by (0.24 m) and at km(51.100) by (0.74 m), which is an acceptable result, provided that the embankment level around (km 39.000) is raised by (0.50 m).

- Increasing the bed slope from (7.5 cm/km) to (11 cm/km)  will lower the water level at km (39.000) by (0.23 m) and at km (51.100) by (0.68 m), which is an acceptable result; however, this will necessate raising the embankment level around (km 39.000) by (0.50 m) too.

- Decreasing the equivalent Manning’s (n) from (0.032) to a value within (0.030 - 0.029) will lower the water level allover the reach by (0.72 m), which is a very acceptable result.

- A combination of increments in bed width, flow depth, and side slope of (15.00 m), (0.15 m), and (1.0), respectively, will result in water levels of (28.76 m.a.s.l.) and (27.26 m.a.s.l.) at km (39.000) and km (51.100), respectively. Such levels are safe ones.

- Using the proposed cross regulator at km (32.200) with a downstream water level of (29.00 m. a. s. l.) will lower the water level at km (39.000) by (0.67 m) and at km(51.100) by (0.71 m), which is an acceptable result.

- Gates of Dora cross – regulator must be so operated that its upstream water level would be not higher than (28.24 m.a.s.l.) to prevent backwater effect and overflowing in the reach.

8- Conclusions

The most appropriate practical solutions, from a hydraulic point of view, would be deepening the reach by (0.35 m) or decreasing Manning’s (n) to (0.0294) or constructing the proposed cross regulator at km (32.200) and operating  it such that the (303 m3/sec) discharge will produce a water level downstream the regulator of (29.00 m.a.s.l.).

Table (1): Key points on the reach {After:( F.C., 1999 ; BWRB, 2004)}

| No. | Designation | Location on Shatt Al-Hilla (km) |  | Design discharge (m3 / sec) | Design water level upstream the head regulator (m.a.s.l.) | Design sill level (m.a.s.l.) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Left side | Right side |  |  |  |
| 1 | Proposed cross regulator | 32.200 |  | 250 | 29.00 | 24.10 |
| 2 | Al-Wardiya canal | 36.960 |  | 1.300 | 28.40 | 27.90 |
| 3 | Al-Tajiya canal |  | 45.750 | 0.650 | 27.93 | 26.98 |
| 4 | Al-Ameer canal |  | 46.550 | 8.121 | 27.90 | 26.20 |
| 5 | Pump station |  | 47.510 | 1.847 | --------- | --------- |
| 6 | Dora canal |  | 50.950 | 3.617 | 27.62 | 26.35 |
| 7 | Dora cross regulator | 51.100 |  | 150 | 28.24 | 22.92 |

C2

C3

C4

C5

C6

C7

C8

C9

Al-Wardiya

C10

C13

C15

C16

C17

C19

C11

C12

C18

C31

C23

C20

C32

C33

C34

C22

C24

C28

C26

C35

C29

C30

C21

C25

C27

C38

C37

C36

C46

C47

C42

C39

C41

C50

C44

C45

C48

C51

C40

C43

C49

C53

C58

C52

C62

C61

C59

C14

Gaging station

(No. 2)

Gaging station

(No. 1)

Al-Tajiya

Al-Ameer

Pump station

C63

C57

C54

C55

C56

Dora

C60

Dora regulator

C64

Gaging station

(No. 3)

C1

C65

N

1:105

Fig. (1): Layout of the reach

3 / 3 / 2004

17 / 3 / 2004

Field data

Time (day)

Water level (m.a.s.l.)

Fig. (2): Actual stage hydrograph as an upstream boundary condition.

Discharge (m3 / sec.)

Field data

Fitted curve

Depth (m)

R2= 0.9917

Fig. (3): Rating curve as a downstream boundary condition

Location (km)

Gaging station

No.(1)

Dora regulator

Discharge (m3 / sec.)

146

148

150

152

154

156

158

160

162

32

34

36

38

40

42

44

46

48

50

Location (km)

Gaging station No.(1)

Dora regulator

Water level (m.a.s.l.)

w.s.p. by using

Eq. (4.10)

Final computed w.s.p. by

the program, for (15) day

Bed level (deepest points)

( )Branching canal sill level

[Figures refer to the numbering              in Table (1)]

(1)

(4)

(3)

(2)

(5)

Fig. (4): Computed initial discharge

Fig (5): Longitudinal water surface profile as initial water surface

Table (2): Observed and computed stage hydrographs (m.a.s.l.) for different values of (Δt) at km (39.000)

| Computed stage θ = 0.96 ; n=0.032 ; |  |  |  |  | Observed stage | Time (day) |
| --- | --- | --- | --- | --- | --- | --- |
| ∆t=24 hr | ∆t=12 hr | ∆t=6 hr | ∆t=3hr | ∆t=1hr |  |  |
| 27.96 | 27.97 | 27.97 | 27.97 | 27.97 | 27.95 | 0 |
| 28.10 | 28.11 | 28.11 | 28.11 | 28.11 | 28.10 | 1 |
| 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 2 |
| 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 3 |
| 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 4 |
| 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 5 |
| 28.15 | 28.16 | 28.17 | 28.17 | 28.17 | 28.14 | 6 |
| 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.14 | 7 |
| 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.14 | 8 |
| 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.14 | 9 |
| 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.14 | 10 |
| 28.39 | 28.45 | 28.47 | 28.47 | 28.47 | 28.38 | 11 |
| 28.31 | 28.31 | 28.31 | 28.31 | 28.31 | 28.30 | 12 |
| 28.31 | 28.31 | 28.31 | 28.31 | 28.31 | 28.30 | 13 |
| 28.31 | 28.31 | 28.31 | 28.31 | 28.31 | 28.30 | 14 |

Table (3): Computed stage hydrographs (m.a.s.l.) for different 
                        values of weighting factor (θ) at km (39.000)

| Time (day) | Computed stage n=0.032 ; Δt=24 hr |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | θ =0.5 | θ=0.55 | θ=0.6 | θ=0.65 | θ =0.7 | θ=0.75 | θ =0.8 |
| 0 | 28.02 | 27.85 | 27.95 | 27.98 | 27.93 | 27.90 | 27.97 |
| 1 | 28.25 | 28.20 | 28.15 | 28.13 | 28.13 | 28.13 | 28.11 |
| 2 | 28.12 | 28.06 | 28.10 | 28.10 | 28.10 | 28.10 | 28.11 |
| 3 | 28.10 | 28.13 | 28.10 | 28.11 | 28.11 | 28.11 | 28.11 |
| 4 | 28.12 | 28.10 | 28.13 | 28.12 | 28.11 | 28.11 | 28.11 |
| 5 | 28.10 | 28.13 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 |
| 6 | 28.18 | 28.14 | 28.16 | 28.16 | 28.16 | 28.16 | 28.16 |
| 7 | 28.12 | 28.16 | 28.14 | 28.14 | 28.14 | 28.14 | 28.14 |
| 8 | 28.18 | 28.13 | 28.14 | 28.14 | 28.14 | 28.14 | 28.14 |
| 9 | 28.12 | 28.16 | 28.14 | 28.14 | 28.14 | 28.14 | 28.14 |
| 10 | 28.18 | 28.12 | 28.14 | 28.14 | 28.14 | 28.14 | 28.14 |
| 11 | 28.41 | 28.41 | 28.39 | 28.38 | 28.36 | 28.38 | 28.36 |
| 12 | 28.38 | 28.28 | 28.30 | 28.30 | 28.30 | 28.30 | 28.30 |
| 13 | 28.32 | 28.32 | 28.30 | 28.30 | 28.30 | 28.30 | 28.30 |
| 14 | 28.28 | 28.28 | 28.30 | 28.30 | 28.30 | 28.30 | 28.30 |

[Table (3): continued]

| Time (day) | Computed stage n=0.032; Δt=24hr |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | θ=0.85 | θ=0.9 | θ=0.95 | θ=0.96 | θ=0.97 | θ=0.98 | θ=0.99 | θ=1.00 |
| 0 | 27.97 | 27.96 | 27.96 | 27.96 | 27.96 | 27.96 | 27.96 | 27.97 |
| 1 | 28.11 | 28.11 | 28.11 | 28.10 | 28.10 | 28.10 | 28.10 | 28.10 |
| 2 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 |
| 3 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 |
| 4 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 |
| 5 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 | 28.11 |
| 6 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.16 | 28.16 | 28.15 |
| 7 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 |
| 8 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 |
| 9 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 |
| 10 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 | 28.15 |
| 11 | 28.40 | 28.40 | 28.40 | 28.39 | 28.39 | 28.39 | 28.39 | 28.39 |
| 12 | 28.30 | 28.30 | 28.30 | 28.31 | 28.31 | 28.31 | 28.31 | 28.31 |
| 13 | 28.30 | 28.30 | 28.30 | 28.31 | 28.31 | 28.31 | 28.31 | 28.31 |
| 14 | 28.30 | 28.30 | 28.30 | 28.31 | 28.31 | 28.31 | 28.31 | 28.31 |

Table (4): Computed stage hydrographs (m.a.s.l.) for different values of Manning’s (n) at km (51.100)

| Time (day) | Computed stage θ =0.96 ; Δt= 24 hr |  |  |
| --- | --- | --- | --- |
|  | n = 0.030 | n = 0.032 | n = 0.034 |
| 0 | 26.94 | 26.92 | 26.89 |
| 1 | 27.03 | 27.01 | 26.98 |
| 2 | 27.03 | 27.01 | 26.98 |
| 3 | 27.03 | 27.01 | 26.98 |
| 4 | 27.03 | 27.01 | 26.98 |
| 5 | 27.03 | 27.01 | 26.98 |
| 6 | 27.06 | 27.04 | 27.01 |
| 7 | 27.06 | 27.04 | 27.01 |
| 8 | 27.06 | 27.04 | 27.01 |
| 9 | 27.06 | 27.04 | 27.01 |
| 10 | 27.06 | 27.04 | 27.01 |
| 11 | 27.19 | 27.17 | 27.14 |
| 12 | 27.17 | 27.15 | 27.12 |
| 13 | 27.17 | 27.15 | 27.12 |
| 14 | 27.17 | 27.15 | 27.12 |

Table (5): Water levels (for Q= 303 m3/sec.) for different values of bed levels

| The change of the bed levels (m) | Water levels at km(39.000) (m.a.s.l.) | Water levels at km(51.100) (m.a.s.l.) |
| --- | --- | --- |
| 0.05 | 28.90 | 27.96 |
| 0.10 | 28.82 | 27.88 |
| 0.15 | 28.73 | 27.79 |
| 0.20 | 28.64 | 27.70 |
| 0.25 | 28.55 | 27.61 |
| 0.30 | 28.49 | 27.55 |
| 0.35 | 28.41 | 27.47 |
| 0.40 | 28.32 | 27.38 |

Table (6): Values of side slope used in the model.

| Cross section | Zi original | Zi (1) | Zi (2) | Zi (3) | Zi (4) | Zi (5) |
| --- | --- | --- | --- | --- | --- | --- |
| C1-C9 | 3 | 3.25 | 3.5 | 4.5 | 5 | 6 |
| C10-C35 | 0 | 0.25 | 0.5 | 1 | 1.5 | 2 |
| C36-C65 | 3 | 3.25 | 3.5 | 4.5 | 5 | 6 |

Table (7): Water levels (m.a.s.l.)(for Q= 303 m3/sec.) for different values of side slope

| Modified side slope | Water level at km(39.000) (m.a.s.l.) | Water level at km(51.100) (m.a.s.l.) |
| --- | --- | --- |
| Zi1 | 29.06 | 28.10 |
| Zi2 | 29.03 | 28.00 |
| Zi3 | 29.00 | 27.94 |
| Zi4 | 28.98 | 27.88 |
| Zi5 | 28.96 | 27.81 |

Table (8): Values of bed width used in the model

| Cross section | Bi original | Bi (1) (m) | Bi (2) (m) | Bi (3) (m) | Bi (4) (m) | Bi (5) (m) |
| --- | --- | --- | --- | --- | --- | --- |
| C1-C9 | 56.75 | 61.75 | 66.75 | 71.75 | 76.75 | 81.75 |
| C10-C35 | 48.00 | 53.00 | 58.00 | 63.00 | 68.00 | 73.00 |
| C36-C61 | 46.25 | 51.25 | 56.25 | 61.25 | 66.25 | 71.25 |
| C61-C65 | 48.20 | 53.20 | 58.20 | 63.20 | 68.20 | 73.20 |

Water level (m.a.s.l.)

Line connecting the data points

Bed width increase (Bi) (m)

Fig. (6): The effect of changing the bed width on the water level at the downstream end of the reach

Table (9): Water levels (m.a.s.l.)at km(39.000) and km (51.100) for different values of bed width.

| Increase in bed width (m) | Water levels at km (39.000) | Water levels at km (51.100) |
| --- | --- | --- |
| 5 | 29.05 | 28.02 |
| 10 | 29.00 | 27.90 |
| 15 | 28.93 | 27.65 |
| 20 | 28.87 | 27.46 |
| 25 | 28.80 | 27.30 |

Table (10): Water levels (m.a.s.l.) (for Q= 303 m3/sec.) for different values of bed slope.

| The bed slope (cm/km) | Water levels at km(39.000) (m.a.s.l.) | Water levels at km(51.100) (m.a.s.l.) |
| --- | --- | --- |
| 7.5 | 29.08 | 28.15 |
| 8 | 29.04 | 28.00 |
| 9 | 28.97 | 27.79 |
| 10 | 28.90 | 27.65 |
| 11 | 28.88 | 27.52 |

Table (11): Water levels (m. a. s. l.) for different values of (n) for (Q = 303 m3 / sec.)

| The values of Manning's (n) | Water level at km (32000) | Water level at km (39000) | Water level at km (51500) |
| --- | --- | --- | --- |
| 0.032 | 29.60 | 29.08 | 28.14 |
| 0.031 | 29.34 | 28.82 | 27.88 |
| 0.030 | 29.09 | 28.57 | 27.63 |
| 0.0295 | 28.96 | 28.44 | 27.50 |
| 0.0294 | 28.91 | 28.39 | 27.45 |
| 0.029 | 28.84 | 28.32 | 27.38 |
| 0.028 | 28.60 | 28.08 | 27.14 |

Table(12): Water levels (m.a.s.l.) (for Q= 303 m3/sec.) for combinations of the aforementioned controls

| Case No. | Water level at km (32000) | Water level at km (39000) | Water level at km (51500) |
| --- | --- | --- | --- |
| 1 | 29.55 | 29.01 | 27.95 |
| 2 | 29.50 | 28.93 | 27.65 |
| 3 | 29.45 | 28.76 | 27.26 |

Table(13): Water levels (m.a.s.l.) at km (39.000) and km(51.100) resulting from the considered cases of operating the proposed cross regulator

| Case No. | Water level at km (39.000) | Water level at km (51.100) |
| --- | --- | --- |
| 1 | 29.08 | 28.14 |
| 2 | 28.76 | 27.81 |
| 3 | 28.44 | 27.49 |

References

Al-Msaudi, R.K. (2001): “Hydraulic control of Shatt Al-Hilla  upstream Hilla City”. M.Sc. thesis, College of Engineering, University of Babylon, Iraq.

BWRD  (Babylon Water Resources Department), (2004): “ Personal connection”.

BWRD (Babylon Water Resources Department), (1998):    “Report about the basic design of Shatt Al-Hilla and Shatt Al-Daghara”. (in Arabic).

F.C. (Al-Furat Center for Studies and Designs of Irrigation Projects), (1999): “Report about the design basis for Shatt Al-Hilla and Shatt Al-Daghara”. (in Arabic).

Fread, D.L. and Lewis, J.M. (1998): “NWS FLDWAV model”.    Hydrologic Research Laboratory. Office of Hydrology, National Weather Service (NWS), NOAA,Internet.http://www.nws.noaa.gov/oh/hrl.

Othman, N.Y. (2006): "Hydraulic control of Shatt Al-Hilla within Hilla City". M.Sc. thesis, College of Engineering, University of Babylon, Iraq.