Simulation of Beamforming Techniques for Phased Array Antenna Using Matlab

Saad Saffah Hassoon

University of Babylon , Collage of Engineering , Electrical Engineering Dept.

Abstract

This report describes the fundamentals of various time domain beamforming techniques with simulations in MATLAB. Details of MATLAB code required for simulation are described, and graphical results illustrated. Also, various sampling algorithms applicable to beamformers are described in detail.

Simulations permit the designer to easily determine the ability of each digital beamformer, before actual VLSI implementation.  Underlying problems and design constraints can also be easily pre-calculated using simulation.

Finally, it is possible to assess each type of beamformer, relative to each other, in terms of size and complexity.

Key words: simulation, beamforming, time domain, sampling

الخلاصة:

يَصِفُ هذا التقريرِ أساسيات تقنيات مختلفة لتوجيه حزم الإشارة (beamforming) في حيز الزمن وذلك بإجراء محاكاة باستخدام النظام البرمجي MATLAB. اذ تم شرح وتوضيح ايعازات البرنامج المطلوبةً للمحاكاة.

تسمح المحاكاةِ للمصممِ لتَقْرير قدرةِ كُلّ موجه من موجهات حزم الإشارة الرقمية بسهولة، قبل تطبيقه بشكل فعلي. كما ان القيود التصميمية والمشاكل الكامنة للمنفذين اثناء عملية التنفيذ يُمْكِنُ أيضاً حسابها بسهولة من خلال المحاكاةَ.

نتائج التحليل والمحاكاة المستخدمة تم عرضها وتحليلها. كما وقد وضحت المخططات والخوارزميات التي تم تطبيقها على موجهات حزمة الإشارة (beamformers).

أخيراً، من الممكن تقييم كُلّ نوع من موجهات حزم الإشارة نسبة إلى بعضها البعض، من ناحية الحجمِ والتعقيدِ.

1- Introduction to Beamforming Techniques

Beamforming is the process of performing spatial filtering, i.e., the response of an array of sensors is made sensitive to signals coming from a specific direction while signals from other directions are attenuated. There are various methods of implementation, including time and frequency domain, depending on the desired processing speed, and the type of signals to be processed.

Beamforming is typically referred to in SONAR (acoustic applications) [Curtis and Ward 1983] and RADAR (electro-magnetic applications) [Barton 1980, Edde 1993] however its applications extend into seismic, medical ultrasonic imaging, and various other applications [Donnell 1988, Saad 2007].

Time domain techniques, which aid in the design and implementation of a digital beamformers will be discussed and simulated in this paper. The efficiency and complexity for each beamformer type will be discussed and the resulting advantages/disadvantages can then be resolved.

1.1 Medium Constraints

In the simplest form, beamforming can be considered to be the combining of omni-directional sensor outputs, arranged in an arbitrary geometry, so as to spatially filter a required direction. This process can be implemented in a variety of ways, however it is important to consider the propagation medium of the operating environment [Dix 1978].

The extremes in which a beamformer must process signals are the environments in which SONAR and RADAR systems operate. Table 1 illustrates the governing properties for each medium [Curtis and Ward 1983].

The property of most significance is the velocity of propagation which affects the frequency range, pulse durations and repetition rates. This indirectly places different requirements on a SONAR beamformer when compared to a beamformer used in a RADAR application.

It should be noted that this table was compiled in 1980 when high power SONAR transducers had to operate at low frequencies, however today much higher frequencies ((50-200) kHz) transducers are in common use [Townsend et. al. 1991].

Table 1: Comparison of SONAR and RADAR properties.

| Property | SONAR | RADAR |
| --- | --- | --- |
| Velocity of Propagation | 1.5 × 103 m/s | 3 × 108 m/s |
| Frequency Range | (0.1 - 10) kHz | (300 - 30000) MHz |
| Typical Pulse Duration | (0.01 - 1.0) s | (0.1 - 10)s |
| Typical Pulse Repetition Rate | (0.2 - 0.01) s-1 | (2000 - 200) s-1 |
| Typical Wavelength | 0.3 m | 0.03 m |
| Doppler Shift | ±2% | ±0.0003% |

1.2 Sampling Methods

An implicit requirement for digital beamforming techniques is the availability of accurate, unaliased digital data stream. Various sampling methods can be employed to convert the analog signal from the transducer into a digital form via an A/D converter.

Firstly however, it is important to recognize the spectral frequencies commonly used in both RADAR and SONAR applications. Fig. 1 illustrates the spectral coverage of low-pass, band-pass and narrowband signals. A narrow band signal is special class of bandpass signals when the bandwidth (BW) is very narrow, i.e., a single frequency.

-wo              wo

-wo              wo

-wo              wo

Fig. 1: spectral definitions.

For each of the spectral domains certain possibilities exist for sampling the signal. The Nyquist theorem [Shannon 1949] states that a bandlimited signal must be sampled at a rate that is at least twice the highest frequency component to avoid aliasing. This is also known as low-pass sampling [Nasser and Namjin 2005].

Using time domain techniques, the sampling rate required for accurate beamforming is typically around (5-10) times the Nyquist rate, as fine time delays are required for accurate beamforming [Schafer and Rabiner 1973, Hampson and Paplinski 1993].

1.3 Time Domain Beamforming

Beamforming or spatial filtering is a method of observing signals from a desired direction while attenuating the response of the array to signals from other directions. Beamforming can permit a two or three-dimensional image to be formatted from an array of sensors, depending on the array geometry.

Fundamentals will be considered firstly using the simple case of beamforming in the time domain using one-dimensional array or linear array. This theory can then be extended to include two-dimensional or planar array imaging.

1.3.1 A three-dimensional input space

To begin with a three-dimensional coordinate system [Nielsen 1991] as shown in Fig. 2 is used where M sensors are located at rm, (m∈0,M−1), incoming wave direction u and a beam steering direction uo.



o

o



Incoming waves

u

Steering direction

uo

M sensors

rm

X

Z

Y

Fig. 2: Coordinate systems for three-dimensional input space.

When considering a linear array of transducers, this is simplified to two-dimensions by reducing the azimuth and elevation angles ( and   respectively) into a conic angle  [Pridham and Mucci 1978], such that (sin =cos cos ).

1.3.2 Beamforming using the linear array

A linear array of transducers is sufficient to map the two-dimensional input space as shown in Fig. 3. A plane incoming wave with direction u makes contact with the evenly spaced linear array (spacing d) of transducer elements rm.

A signal xm(t) arriving at the mth sensor is, in general, a sinusoidal pulse modulated signal with spatial information inscribed by reflections from objects to be detected. The phase of the incoming wave gives information about the spatial direction of the source.

To obtain spatial information from a direction ub, a beam is formed in the related conic direction b by performing a weighted summation of appropriately delayed signals xm(t).

0               d               2d            3d           4d

u

b

Z

rm on X

Plane wave

Fig. 3:  Projection of a plane wave on the linear array.

To form a beam in the direction b, each sensor signal xm(t) is delayed by:

tmb=rm⋅ubc=mdcsinψb								(1)

and the bth beam output is formed by the general beamforming equation:

yb(t)=m=0M−1amxm(ωt−ωtmb)								(2)

where am is a spatial weighting function usually defined as a window function [Pridham and Mucci 1978] to control main-lobe width and side-lobe magnitude. Fig.4 illustrates a beam steered 30° to the right using unity weighting.

Beam Pattern

Incoming angle pci

Beam Pattern

Magnitude

Beam Power

Incoming angle pci

Fig. 4: A simple Time domain-Delay sum beamformer pattern with M=8

A discrete version [Hampson and Paplinski 1993] of this equation is obtained by assuming a sampling frequency fs=1ts and Ω=ωtsSubstituting t=nts  then Eq. (2) becomes:

yb(n)=m=0M−1amxm(Ω(n−qmb))							(3)

where qmb= tmb/ts represents an integer delay (number of delay units) for a given sensor input, m, and beam direction, b. To form a beam yb in a steering direction b, the output of each sensor is delayed by an integer multiple of the sample spacing qmb, where qmb represents

qmb=mδssinψb     where    δs=dfsc						(4)

Since qmb is integer delay, it is imperative that δssinψb is an integer also. This obviously quantizes the steering directions which can be realized.

A simple time domain beamforming structure is the Delay-Sum beamformer which implements Eq. (3) and is shown in Fig. 5. The structure is repeated B times, where B is the number of beams desired.

x1(t)

xm(t)

xM(t)

ADC

a:1

amb

a:B

y1(t)

yb(t)

yB(t)

x(t)

ts

x(n)

M

M

M

M

y(n)

B

Linear Array

of Sensors

Fig. 5: A simple Time domain-Delay sum beamformer.

The sensor signals from the linear array are converted to a digital signal at a frequency greater than the Nyquist rate to achieve acceptable beam pattern performance. The signals are then delayed appropriately and the weighting function applied and effectively summed. If same set of weights are to be used for all steering directions, as is often the case, the weights can be applied directly at the sensor outputs, before the delay sum operation.

1.3.3 Beam Steering Directions

To form a beam in direction b, the delay required to bring the output of the M sensors into time alignment is equal to qmb, which are integer multiplies of the sample spacing ts, if the beam index b, is an integer  ( b=0, ±1, ±2, ±3, … )  then the beam directions that we can realize are those for which

δssinψb=b										(5)

and effectively quantizes the beam steering directions b, such that

ψb=sin−1(b⋅1δs)									(6)

Consider an average SONAR beamformer with sensor separation of d=0.003 and fs = 600kHz, then the discrete beamforming angles would be 0, ±56.4°. It can be shown [Pridham and Mucci 1979] that the beam separation is given by Eq. (7), and showing in Fig. 6 for  fs =2.5f.

Beam Separation=2ffs  in  sinψ  space 						(7)

-1   -0.8                   0                   0.8   1

Fig. 6: Directions of synchronous beams for fs =2.5f.

This is a major limitation of the simple beamformer, and it is only possible to get more beams by increasing the sensor spacing or increasing the sampling frequency. Over sampling can be achieved by using high speed A/D converters, or by interpolating the sensor signals in the digital domain.

Simulation in MATLAB [The MathWorks Inc] is achieved by examining the response of the beamformer for all input directions, for a particular steering direction. Firstly, create the input waves, next, apply each input direction, and calculate the power of the beam. The beam direction is set by b, the beam index.

Results for a beam formed at broadside (b = 0, 1, -1, b = 0° 28.7°, -28.7°, M = 8) are shown in Fig. 7. The plot illustrates that incoming wave angles other that the steering direction are attenuated.

1.3.4  Interpolation Beamforming

Interpolation is the process of increasing the effective sampling frequency by an integer factor, thereby decreasing the sampling period. This decreased sampling period allows finer time delays to be implemented in the beamformer, and hence more steering directions Refer to Appendix A for more information regarding interpolation.

An implementation of such an interpolation beamformer [Pridham and Mucci 1978, Pierre Da Sylva and Roy 1986] is shown in Fig. 8. Here the sensor signals are interpolated by a factor of D. A set of desired delays now is approximated more accurately by specifying integer multiples of the smaller sampling time ts/D. Once again a different set of integers qmb is specified for each beam direction.

qmb=mD⋅δssinψb									(8)

However, the effective sampling frequency D · fs is significantly larger enabling more closely spaced beams to be formed.

Wave direction (degrees)

Wave direction (degrees)

Wave direction (degrees)

Beam power

(dB)

Beam power

(dB)

Beam power

(dB)

Fig. 7: flowchart of the simulation program of the simple Beamformer with results which were: (A) zero phase shift, 22 dB beam power. (B) 28.7o phase shift, 18.3 dB beam power, and (C) -28.7o phase shift, with beam power = 16.2 dB

(

ψb=sin−1b⋅1Dδs									(9)

Start

Enter all parameters

Creating the input wave

Compute the integer delay for each sensor

Determine the delayed signal  Xqmb

Put a rectangular window

Determine the beam power of the sum of all the resultant delayed signals

End

If

I<phase range*2

+1

Yes

No

Figure 8 is referred to as the interpolation filter before beamforming structure. Here the linear array of sensors produces an analog signal, x(t), which is digitized (as in the simple beamformer) and padded with a vector of (D - 1) zeros. The zero padded signals are then interpolated using M FIR low-pass filters (one for each sensor). The delayed signals are then decimated by D and summed to produced a beam output yb(n).

Linear Array of Sensors

x1(t)

xm(t)

xM(t)

ADC

pad with D-1 zeros

M×FIR

Low-Pass Filter

Fc=fc/fs=1/2D

x(t)

ts

x(n)

M

M

M

a:b

z-q:b

M

Effective sampling frequency is Dfs and corresponding period is ts/D

Weighting is performed

at fs

Decimate by D

fs=1/ts

yb(n)

(A)

Start

Enter all parameters

Creating the input wave

Compute the integer delay for each sensor

Determine the delayed signal  Xqmb

Determine the beam power of the sum of all the resultant delayed signals

End

If

phase range×2

+1

Yes

No

Prepare the input signal to the interpolation process by putting the input signal ,X, into Xd with zeros

Interpolate datafor each sensor

Decimate and form beam

(B)

Fig. 8: (A) Interpolation filter before beamforming for one steering direction yb(n) (B)  flowchart of the simulated program.

Results for the interpolation beamformer indicate that the beam pattern degradation [Hampson and Paplinski 1993] increases as the interpolation factor D increases as shown by Fig. 9.

Beam Power Spectrum, M=8, f=200kHz, d=3mm

Beam Power Spectrum, M=8, f=200kHz, d=3mm

-100  -80   -60  -40   -20   0     20    40    60   80  100

-100  -80   -60  -40   -20   0     20    40    60   80  100

0

-5

-10

-15

-20

-25

0

-5

-10

-15

-20

-25

Beam Power (dB)

Beam Power (dB)

Incoming wave direction (degrees)

Incoming wave direction (degrees)

Fig. 9: Power spectrums for the interpolation beamformer D=2 and D=10. Note that the steering direction magnitude changes with D.

For a large number of sensors this structure becomes computationally expensive since each sensor requires a low-pass filter.

In this structure, the order in which the summation and filtering has been changed to optimize the number of FIR filters. Filtering and summation are both linear operations and can be interchanged. As shown in fig. 10, the figure illustrates that the sensor signals are zero padded, delayed, weighted and then summed at the increased sampling frequency Dfs. The interpolation filter follows the summation and is calculated at the increased sampling frequency, and then decimated.

Linear Array of Sensors

x1(t)

xm(t)

xM(t)

ADC

pad with D-1 zeros

B× FIR

Low-Pass Filter

Fc=fc/fs=1/2D

x(t)

ts

x(n)

M

M

M

a:b

z-q:b

Weighting is performed

at Dfs

Decimate output yb(n) by D

Effective sampling frequency is Dfs and corresponding period is ts/D

yb(n)

fs=1/ts

Fig. 10: Interpolation filter after beamforming.

2. Conclusions

1- It was shown that beamformers could be expected to operate on signals in a wide frequency range, and it is therefore important to consider the nature of the signals to be processed.

2- Lowpass sampling is sufficient for low-frequency signals, however for high frequency band-pass and narrow-band signals, bandpass sampling techniques must be adopted. It was also shown that interpolation could be used to increase the effective sampling frequency.

3- Beamforming was introduced using the simple time domain beamformer and later extended using interpolation. MATLAB simulations were given for each beamformer to supplement the understanding of the operations required in the beamformer. The simulations also give an insight into design considerations and specifications of a real implementation.

4- It is now possible to assess each type of beamformer, relative to each other, in terms of size and complexity.

3. References

Barton P., "Digital beamforming for RADAR", IEE Proceedings, vol. 127, August 1980

Curtis T. and Ward R., "Digital beamforming for SONAR systems", IEE Proceedings, vol. 127, August 1980.

Dix J., "Initial SONAR system considerations", IEE Colloquium Digest, November 1978.

Donnell M. O., "Applications of VLSI circuits to medical imaging", Proceedings of the IEEE, vol. 76 September 1988.

Edde B., "RADAR – Principles, Technology, Applications" Prentice Hall, 1993.

Hampson G. and Paplinski A., "Beamforming by interpolation", Tech. Rep. 93-102, Monash University,1993.

Knight S. M. and Pridham R. G., "Digital signal processing for sonar", Proceedings of the IEEE, vol. 69, November 1981.

Mikhail C., “An Introduction to Parametric Digital Filters and Oscillators”, John Wiley & Sons Ltd, 2003.

Nasser K. and Namjin K., “Digital Signal Processing System-Level Design Using LabVIEW”, Elsevier Inc. All rights reserved, 2005

Nielsen R. O., "Sonar Signal Processing", Artech House, 1991.

O'Donnell M., "Real-time phased array imaging using digital beam forming and autonomous channel control", Ultrasonics Symposium, pp. 1499-1502, 1990.

Pierre Da Sylva P. M. and Roy D., "A Reconfigurable Real-Time Interpolation Beamformer", IEEE Journal of Oceanic Engineering, vol. OE-11, January 1986.

Pridham G. and Mucci R. A., "Shifted Sideband Beamformer", IEEE Transactions on Acoustics, Speech and Signal Processing, vol. ASSP-27, December 1979.

Pridham R. G. and Mucci R. A., "A novel approach to digital beamforming", Journal on Acoustical Society of America, vol. 63, pp. 425-434, February 1978.

Pridham R. G. and Mucci R. A., "Digital interpolation beamforming for low-pass and bandpass signals", Proceedings of the IEEE, vol. 67, June 1979.

Saad S. H., “Optical Beamforming Networks for Linear Phased Array Antennas”, Ph.D. Thesis, University of Basrah, 2007.

Schafer R. W. and Rabiner L. R., "A digital signal processing approach to Interpolation", Proceedings of the IEEE, vol. 61, June 1973.

Shannon C., "Communication in the presence of noise", Proceedings of the IRE, vol. 37, pp. 10-21, January 1949.

Sotelo J., "Matrix Circuit Descriptions and Test Information", Interphase Technologies, 1201 Shaffer Road, Santa Cruz, California, 95060, October 1991.

The MathWorks Inc., PRO-MATLAB. Available at (http://www.mathworks.com ).

Appendix A

Sampling Procedures

An implicit requirement for digital beamforming techniques is the availability of accurate, un-aliased digital data stream. Various sampling methods can be employed to convert the analog signal from the transducer into a digital form via an A/D converter.

This appendix contains information regarding the sampling procedures relevant to beamforming, which include Interpolation.

Interpolation

Interpolation is the process of increasing the initial sampling frequency fs to an increased sampling frequency of fs by an integer factor D, i.e., the new sampling frequency is fs=D⋅fs and the time separation of samples is ts=tsD where  ts = 1/fs . Thus a sampling frequency of fs  satisfies the Nyquist criterion for the data, whereas  fs is required for accurate beamforming.

To interpolate a sequence x(n), where the samples are separated by ts seconds, zero padding is the first process:

x(k)x(kD),          if k=0, ±D, ±2D, ...0                        otherwise      					(A.1)

Note that D-1 zeros are inserted between each pair of terms in the original sequence x(n). An operation is then performed to interpolate between the nonzero samples in the sequence x(k).

An intuitive approach to interpolating between pairs of samples is simply to use linear interpolation. It can be shown that linear interpolation is equivalent to filtering with a symmetric FIR filter [Mikhail 2003] and exactly two input samples are used for each calculation.

It is expected that linear interpolation would perform poorly in the presence of high frequency signals and therefore the highest frequency in the original signal x(n) must be considerably smaller than fs/2 for linear interpolation to provide adequate performance. Fig. A.1 illustrates the process of interpolation to increase the effective sampling frequency.

Simulation using MATLAB is achieved by a few simple MATLAB commands:

A/D

Zero Pad

Interpolation filter (low-pass)

x(t)

x(nts)

x(nts)

x(nts/D)

Fig. A.1: The interpolation process.

x = sin(n*2*pi*f/fs);                     % input waveform

x_d = x(1:D:length(x));               % decimate the input waveform by D

x_i = kron(x_d, eye(I, 1));           % use the tensor product to zero pad

fc = ((f/D)*I)/fs;                           % normalized cut-off frequency

b = fir1(32,fc);                             % filter coefficients

x_r = filter(b, 1, x_i);                   % filter the zero-padded waveform

Figure A.2 illustrates an example where an original wave form (sampled at fs) is decimated such that there are samples a period. This is then interpolated and filtered by a factor of 9, i.e., 9 times the sample rate of the decimated waveform. Note that the interpolated waveform has smaller amplitude due to the averaging nature of the filter. i.e., the higher the interpolation factor, the smaller the amplitude.

Fig. A.2: MATLAB simulation of Interpolation.