## Liquid Entrainment in Annular Two-phase Flow

## in Vertical Pipes

## Ahmed Saib Naji

## Electrochemical Department, College of Engineering, Babylon University

## Abstract

In the present work, a semi-mechanistic model to predict the liquid entrainment in annular two-phase flow in vertical pipe. This model is originally a modified version to the model of Martin et. al. (1996). The experimental tests have conducted by the literature for air-water system  flowing through vertical pipe of 9.5 mm in diameter [Ryan 2004]. The comparison achieved by calculating the average and absolute percent error between the measured entrainment and the predicted entrainment by eight methods available in the literature. The comparison showed the best results of the present model among the others.

Keywords: two-phase, vertical pipe, annular flow pattern, liquid entrainment

الخلاصة

في العمل الحالي موديل شبه- ميكانيكي يستخدم لحساب مقدار تداخل السائل في الجريان الاسطواني ،الثنائي الطور في أنبوب شاقولي. هذا الموديل هو بالأصل نسخة مطورة لموديل مارتن وجماعته (1996). الفحوصات العملية منشورة سابقا" لنظام هواء- ماء  يجري خلال أنبوب شاقولي بقطر 9.5 ملم ]راين 2004[. المقارنة  تمت بحساب السائل المتداخل باستخدام معدل الخطأ النسبي ومعدل مطلق الخطأ النسبي بين القيم المقاسة والقيم المحسوبة بواسطة ثمانية طرق منشورة سابقا". بينت المقارنة أن نتائج الموديل الحالي هي الأفضل بين جميع الطرق الأخرى.

### 1. Introduction:

Annular flow is one of the most common flow patterns encountered in a wide range of industrial applications such as: nuclear power plants, evaporators, condensers, and petroleum transport pipelines [Zheng et. al. 2006].

When two fluids (such as: gas and liquid) flowing together in the same pipe or stream, and when the input liquid flow rate is little compared to the gas input flow rate, hence, the annular flow pattern will happen. It is identified by three fluids streams: liquid film on the wall, the vapor core and droplets entrained in the vapor stream.[Vladimir et, al. 2008],  the continuous vapor core down the center of the tube and may or may not include the entrained droplets [Qaisrai 2008]. The configuration of this pattern shown in figure (1). The thin layer (film) of the liquid undergoes three factors: one by the wall roughness friction, the second is

Figure (1): Typical Annular Flow Pattern [Ghiaasiaan 2008]

given by the gravity force which its direction is the downwardly and the third is the interfacial friction with the high flow rate of the gas core who generates an interfacial waves. The impact of the last fact leads to detach some droplets from the liquid film layer and fly through the space of the gas phase [Ghiaasiaan 2008]. The fraction of which liquid content in the gas core called the liquid entrainment. This factor is usually bounded between zero when there is an ideal annular flow and unity in case of fully mist flow pattern [Rame 2006]. This entrainment of liquid can cause a momentum transfer that can affect the velocity profile, pressure drop, film thickness and lastly heat transfer [Ariyadasa 2002]. Therefore, it represents one of the important needs of the modeling in annular flow pattern [Hasan-Kabir 2007].

2. Literature Review:

In the past, a few models treated this flow pattern assuming zero slip between the two phases in the gas core. For instance, the models of Duns and Ros (1963), and Aziz et al. (1972), who essentially adopted the Duns and Ros approach, fall into this category [Hasan-Kabir 2007].

Steen and Wallis (1964) stated that the liquid entrainment depends on the critical gas velocity factor which was defined by:

E = 0.0055 Vcrit2.86                   if    Vcrit < 4             -----------------(1)

E = 0.857 log10( Vcrit)−0.2       if    Vcrit > 4             -----------------(2)

Where: Vcrit=104 Vsg μgσ ρgρL

It clears that the entrainment will lastly depend on the ratio ρg/ρL. Wallis (1969) used the deduction of the available outlines of Steen and Wallis (1964) to proposed his correlation as a function to the critical gas velocity factor:

E = 1−e[−0.125 (Vcrit−1.5)]                                  -----------------(3)

In (1973), Hughmark proposed an iterative technique to calculate the entrainment. This method modified to be in the following form:

E = 1.3× 10−6 μLμgρgρL VsgVsL ρL VsL dμL0.81       -----------------(4)

Govier and Fogarasi (1975) proposed the entrainment as ranges related to the superficial liquid velocity. Their correlation was:

E =0                         if                VsL < 0.03 m/s      -----------------(5)

E = 1−0.00421VsL1.5          if    0.03 < VsL < 0.92 m/s      -----------------(6)

E =0.995                  if               VsL ≥ 0.92 m/s        -----------------(7)

Asali et, al. (1985) stated that the knowledge of entrainment fraction and the amount of the liquid flowing as a liquid film was needed to develop better design procedures for vertical annular gas-liquid flows. Azzopardi (1986) observed the gas-to-liquid density ρg/ρLwas affect the size of the entrained droplets.

In (1986), Oliemans proposed that the entrainment related to the whole geometrical and operation specifications. The correlation was a curve fitting to the available data used. The last version of this method was:

E = CC+1                                                                  -----------------(8)

Where   C=Cd × Cg× CL and

Cd = 10−2.52 × d1.72× g0.46× σ−1

Cg = ρg0.18 × μg0.25× Vsg1.44

CL = ρL1.08 × μL0.27× VsL0.7

Jepson et. al, (1989) showed that the reduction in gas density caused by a lower shear on the liquid film, which caused a decrease in the amount of the entrainment. In (1989), Ishii and Mishima suggested a detailed correlation on the basis of entrainment inception criteria and a force balance at the wavy interface. Their correlation was:

E=tanh7.25×10−7 Weg1.25 Ref0.25                    -----------------(9)

Where: Weg=ρg Vsg2 dσ Δρρg1/3    and Ref=ρL VsL dμL

Hewitt and Govan (1990) proposed their correlation based on large experimental data. They included the whole parameters of two-phase, annular flow. Their method was:

E =5.75 × 10−5 ρg VsgρL VsLρL3ρg2 σ VsL−VsLf20.316 -----------------(10)

Where: VsLf = μLρL de5.8504+0.4299μgμLρLρg

Martin et. al. (1996) developed semi-mechanistic entrainment correlation for fully developed annular flow conditions based on the droplet continuity equation and the entrainment rate model. This model then introduced a Weber number that included the droplet concentration Weg. This Weber number was shown to scale the available high and low pressure air-water data better than the other definitions. This correlation was:

E = ab+cWec                                                           -----------------(11)

where: Weg=ρgc Vsg2 dσ    and      ρgc= Wg+WLEWgρg

They proposed the constants as: a = 0.9642, b = 1 and c = 3836

Abdul-Majeed (1997), proposed simple correlation to calculate the liquid entrainment in annular two-phase flow with water content.

Petalas and Aziz (2000), proposed a new model to predict the entrainment, this method used by several investigators. This method was:

E = QQ+1

Where: Q = 0.735 NB0.074 VsgVsL0.2 and NB = μL2 Vsg2 ρgσ2 ρL

Bertodano et al (2001) concluded that the entrainment rate was dependent on the liquid-to-gas density ratio to a power of 0.5, and Vassallo et al (2001) stated that the entrainment rate was enhanced at higher pressures due to the higher gas density. In (2002), Pan and Hanratty stated that the relation was based on the density to a power of 0.25.

3. Measured Entrainment:

The actual entrainment obtained by using an experimental apparatus constructed and published in the literature. This test section consists of vertical tube of 9.5 mm in diameter carries a system of co-current two-phase flow represented by air-water system used to conduct (196) tests [Ryan 2004]. The measured data displayed in the following table:

|  | The properties | The Ranges |
| --- | --- | --- |
| 1 | VsL (m/s) | 0.081 to 0.288 |
| 2 | Vsg (m/s) | 18.1 to 29.4 |
| 3 | Pav (KPa) | 92.3 to 136.8 |
| 4 | δ (mm) | 0.125 to 0.359 |

Barnea (1987) suggested the following equation as the transition line to the annular flow pattern :

Y = 2−1.5 αLαL2 (1−1.5αL) X2                  ------------------------------------(12)

To include the entrainment impact in equation (12), Brill and Mukherjee (1999) suggested other versions of the Lockhart-Martinelli parameter (X) and Taitel-Dukler inclination angles parameter (Y ) to be in form of[Alves 1991], [Abdul-Mjeed 1997]:

Xm2=(1−E)2 fffsc (dP/dL)sL(dP/dL)sc     and.Ym=g (ρL−ρg) sinθ(dP/dL)sc    ----(13)

Where ff and fsc are fanning friction factors in film and gas core zones. Abdul-Majeed (2009) suggested using the equations (12) and (13) to obtain the actual entrainment.

4. Present Method

In the present work, two steps are achieved to develop the present method as in the following:

Step 1: By using the available tests, a comparison between eight methods published in the literature has done by using the statistical tools which will displayed in next paragraph. It is seen that the empirical correlation of Hughmark (1973) give the best results but it had developed originally based on very limited tests [Abdul-Majeed 1997] therefore, this method will excluded and went to the second best results which are given by semi-mechanistic method of Martin et al (1996).

Step 2: The semi-mechanistic method of Martin et al (1996) is modified in the present work by using the outlines of the previous studies and using the simple analysis of the actual results. Really, the modification concentrated on the constants of that method.  The new version of this method is:

E = ab+cWec                                      ----------------------------(14)

where:  a = 0.9 Vsg−0.218       b = 0.1008 VsL−1.5

c = 0.009 VsL × n      also       n=(VsgVsL) (μLμg) (ρLρg)

5. Statistical Tools

In this paper, average and absolute average percent errors are used in the comparison among the whole methods. The percent error achieved between the measured entrainment and the predicted entrainment by the available methods. These tools are:

1. Average percent error:     APE = 1n i=1nPEi                      ------(15)

2. Absolute average percent error:  AAPE = 1n i=1nPEi       -------(16)

Where: PEi =Ecal. − Emeas.Emeas. ×100 %

6. Results and Discussion:

The behavior of the predicted liquid entrainment against the measured entrainment have graphed in this study, the graphs of worst methods in the prediction the entrainment were excluded, the rest methods represented by the figure (2) to figure(7),

Figure (2): The Estimation of Steen

and Wallis Method

Figure (3): The Estimation of Wallis

Method

Figure (4): The Estimation of Hughmark

et al.  Method

Figure (5): The Estimation of Govier

and  Fogarasi Method

Figure (6): Estimation of Martin

et al Method

Figure (7): Estimation of The

Present Method

on the other hand, the statistical tool results have represented in tabular form as shown in table (1).

Table (1): The Statistical Results of the Published and The Present Methods

|  | The Used Methods | Average Percent Error (APE) | Average Absolute Percent Error (AAPE) |
| --- | --- | --- | --- |
| 1 | Steen and Wallis (1964) | -88.52 | 88.52 |
| 2 | Wallis (1967) | -86.75 | 86.75 |
| 3 | Hughmark et. al. (1973) | -41.65 | 67.75 |
| 4 | Govier and Fogarasi (1975) | 227.3 | 227.3 |
| 5 | Oliemans (1986) | -100 | 100 |
| 6 | Hewitt and Govan (1990) | -96.64 | 96.64 |
| 7 | Martin et. al. (1996) | -44.68 | 65.08 |
| 8 | Petalas and Aziz (2000) | 90.12 | 95.69 |
| 9 | Present Method | -8.740 | 62.17 |

By reviewing the whole figures and table, it is clear that the correlation of Hughmark et al (1973) gave good  results but because of it is an empirical correlation developed by using very limited tests [Abdul-Majeed 1997], its results will be ignored. The results of the semi-mechanistic method of Martin et al (1996) will be considered as a method of good results. The best results are given by the present method because its basis of derive is the semi-mechanistic and the constants (a, b, and c) are functions to most system properties. Most of the used methods are seem to be under-predicting includes the present method while the correlation of Govier and Fogarasi (1975), and Petalas and Aziz (2000) seem to be over-predicting. These observations are clear in the figures.

7. Conclusion

The comparison between the measured and predicting magnitudes of the liquid entrainment among the whole methods, it is important to display the following outlines:

- Generally, none of the used methods predicted the real values of the entrainment of liquid because of these methods are either fully empirical correlation or semi-empirical.

- According to the results of table (1), the correlation of Govier and Foarasi (1975) concludes non-right assumptions such as the ideal annular flow will happen where the superficial liquid velocity less than 0.03 m/s or as the semi-mist flow pattern existed when the superficial liquid velocity greater than 0.92 m/s.

- According to the whole figures, it's clear that all methods are under-predicting the liquid entrainment including Martin et al (1996) method except the method of Govier and Fogarasai (1973) which is over-predicting method. The distribution of the present method is good comparing with other methods as shown in figure (7).

- According to the result of the present method, the entrainment of liquid maybe depends on the ratio ρL/ρgbut to power of unity.

8. Nomenclature

AAPE Absolute Average Percent Error                       (dimensionless)

APE   Average Percent Error                                       (dimensionless)

d         Diameter                                                             (m)

E         Liquid entrainment                                            (dimensionless)

f         Friction Factor                                                    (dimensionless)

G       Gravitational acceleration                                  (m/s2)

L        Length                                                                (m)

P        Pressure                                                              (Pa)

PEi     Percent Error of the test of ranking (i)              (dimensionless)

Re      Reynolds number                                               (dimensionless)

V        Velocity                                                             (m/s)

We     Weber number                                                   (dimensionless)

X       Lockhart-Martinelli Parameter                           (dimensionless)

Y       Taitel-Dukler Inclination Angle Parameter        (dimensionless)

Subscripts:

crit         Critical                              g          Gas                  av        Average

sc           Superficial gas core           m         modified

sg           Superficial gas                  L          Liquid

sL          Superficial liquid               f          Film  zone

Greek Symbols:

α              Holdup                  (dimensionless)    ρ    Density      (kg/m3)

δ               Film thickness     (m)                        σ    Surface tension  (N/m)

Δ              Difference

θ              Inclination angle   (deg.)

μ              Viscosity               (Pa.s)

# 9. References:

Abdul-Majeed, G. H. ;" A comprehensive mechanistic model for vertical and inclined two-phase flow"; Ph. D. Dissertation, Pet. Eng. Dept., Coll. Of Eng., Baghdad university, Iraq, (1997).

Abdul-Majeed, G. H.;" Private Communication"; (2009)

Alves, I. N., Caetano, E. F., Minami, K. and Shoham, O.;" Modeling annular flow behavior for gas wells"; SPE Production Engineering, pp. 435-440,(1991).

Ariyadasa, U.;"An Investigation of Film Thickness and Pressure in Upward and Downward Annular Two-Phase Flow"; . M.Sc. Thesis, Department of Mechanical Eng., University of Saskatchewan, (2002).

Asali, J. C., Hanratty, T. J. and Andreussi, P.;" Interfacial Drag and Film height for vertical Annular Flow"; AIChE. J., vol. 31, pp. 895, (1985).

Aziz, K., Govier, G. W. and Fogarasi, M.;" Pressure drop in wells producting oil and gas"; J. Can. Pet., pp. 11-38, (1972).

Azzopardi, B. J. ;"Disturbance Wave Frequencies, Velocities and Spacing in Vertical Annular Two-Phase Flow", Nuclear Engineering and Design, Vol. 92, pp. 121-133, (1986).

Barnea, D.;" A unified model for predicting flow pattern transition for the whole range of pipe inclinations"; Int. J. Multiphase Flow, vol. 13, No. 1, pp. 1-12, (1987).

Brill, J. P. and Mukherjee, H.;" Multiphase Flow in Wells"; 1st printing, Henry L. Doherty Memorial Fund of AIME, Society of Petroleum Engineers Inc., Richardson Texas, (1999).

Dallman, J. C., Jones, B. G., and Hanratty, T. J.,;" Interpretation of Entrainment Measurements in Annular Gas-Liquid Flows.” Two-Phase Flow, heat and mass Transfer, Vol2, 681-693, Hemisphere, Washington, D.C., (1979).

Duns, H. Jr. and Ros, N. C. J.:" Vertical Flow of gas and liquid mixtures in wells"; Proc., 6th world Pet. Cong. Tokyo, pp. 451, (1963).

Fan Pu QIU Sui-Zheng JIA Dou-Nan,:" An investigation of flow characteristics and critical heat flux in vertical upward round tube", Nuclear Science and Techniques, vol. 17, No. 3, pp. 170-176, (2006).

Ghiaasiaan, S. Mostafa:" TWO-PHASE FLOW, BOILING AND CONDENSATION IN CONVENTIONAL AND MINIATURE SYSTEMS" Cambridge university press, (2008).

Govier, G. W. and Fogarasi, M.: “Pressure Drop in Wells Producing Gas and Condensate”, J. Can. Pet. Tech., 28-41, Oct – Dec (1975).

Hasan, a. r. AND Kabir, C. S.:" Asimple model for annular two-phase flow in wellbores"; Society of petroleum engineers, SPE Production and Operation, vol. 22, No. 2, 168-175, (2007).

Hewitt,G. F., and Govan, A. H. ;"Phenomena and prediction in annular two-phase flow", In ASME Advances in Gas-Liquid Flows, ASME, New York, FED- 99, 41–56, (1990).

Hughmark, G. A. ;" Film thickness, entrainment and pressure drop in upward annular and dispersed flow"; AIChE J. vol. 19, pp. 1062-1065, (1973).

Ishii, M. and K. Mishima. (1989). Droplet entrainment correlation in annular two-phase flow. International Journal of Heat and Mass Transfer, Vol. 32. No. 10, pp. 1835-1846.

Jepson, D. M., B. J. Assopardi and P. B. Whalley.;" The effect of gas properties

Martin A. Lopez de Betodano, Cheng-Shiun Jan and Stephen G. Beus,:" Droplet Entrainment Correlation for High Pressure Annular Two-Phase Flow"', U.S. Department of Energy, DE-AC1 1 -93PN38195, USA, (1996).

Oliemans, R.V., Pots, B.F.M. and Trompe, N.,;“Modeling of Annular Dispersed Two-Phase Flow in Vertical Pipes,” Int. J. Multiphase Flow, vol. 12, No. 5, pp 711-732 (1986).

on drops in annular flow". Int. J. of Multiphase Flow, Vol. 15, No. 3, pp. 327-339, (1989).

Pan, L. and T. J. Hanratty ;" Correlation of entrainment for annular flow in vertical pipes". International Journal of Multiphase Flow, Vol. 28, No. 3, pp. 363-384, . (2002).

Petalas, N., Aziz, K.;" A mechanistic model for multiphase flow in pipes". J. Can. Petrol. Technol. 39, 43–55, (2000)..

Rame, E., Balasubramaniam, R., Kizito, J. and Kassemi, M.;"Two Phase Flow Modeling: Summary of flow Regimes and Pressure Drop Correlations in Reduced and Partial gravity", NASA/CR, (2006)-214085.

Ryan M. MacGillivray,:" Gravity and Gas Density Effects on Annular Flow Average Film Thickness and Frictional Pressure Drop", M. Sc. Thesis, the Department of Mechanical Engineering, University of Saskatchewan, Canada, (2004).

Steen, D. A and Wallis, G. B. : AEC report, No. NYO-31142-2, (1964).

Tahir M. Qaisrani;" An introduction to air-liquid, two-phase flow, hydrodynamics and power consumption"; Institute of process engineering, Johannes kepler university, Linz, Austria, (2008).

Taitel, Y. and Dukler, A. E.;" A model for predicting flow regime transitions in horizontal and near horizontal flow";, AIChE J., vol. 22, pp. 47-55, (1976).

Vassallo, P. F., T. A. Trabold, R. Kumar, and D. M. Considine. ;"Slug-to-annular regime transitions in R-134a flowing through a vertical duct". International Journal of Multiphase Flow, Vol. 27, No. 1, pp. 119-145, (2001).

Vladimir Stevanovic, Sanja Prica and Blazenka Maslovaric:" Multi-fluid model predictions of gas-liquid Two-Phase Flows in vertical tubes"; university of Belgrade, Faculty of mechanical engineering, (2008).

Wallis, G. B. ;" One-dimensional Two-Phase Flow"; McGraw-Hill, New York, USA, (1969).