Designing Program for Economic  of Operation Power System

Ekbal Abd Al-Majeed

University of Technology, Baghdad

Abstract

In this paper, the economic dispatch algorithm is used to reduce the losses in the power system. The method of LaGrange was built to solve the problem of losses reduction case to reduce the total  generation cost.

The some capacitors in the power system are installed them on the buses of the system to minimize the power losses and get the best total generation cost.

To achieve that, the present work is developed to determine the optimal location and size of capacitors in transmission power system where, the Participation Factor Algorithm and the Steady State Genetic Algorithm are proposed to select the best locations for the capacitors (by the first algorithm PF) and determine the optimal size for them (by the second algorithm ssGA), taking into consideration the cost of power losses, cost of fuel, and cost of the capacitors.

الخلاصـــة

تمّ في هذا البحث استخدام خوارزمية الإرسالية الاقتصادي (Economic operation)؛ وذلك لتقليل الخسائر في أي نظام قدرة كهربائية، إذ تمّ استخدام طريقة لاكرانج لحل مشاكل تقليل الطاقة المفقودة التي تساعد في النهاية على تقليل كلفة النظام.

وقد تمّ حقن عدة متسعات في النظام من خلال تنصيبها على مسارات الطاقة التي تؤدي إلى الحفاظ على الطاقة وعدم ضياعها.

ولتحقيق ذلك فقد قام هذا البحث بتحديد أفضل مواقع وانسب عدد للمتسعات، إذ تمّ بناء خوارزمية معامل المشاركة (Participation Factor) والخوارزمية الجينية (Steady State Genetic Algorithm) ، استخدمت الخوارزمية الأولى لاختيار أفضل المواقع لحقن المتسعات  في حين استخدمت الخوارزمية الثانية  لتحديد حجم هذه المتسعات وعددها. آخذين بنظر الاعتبار كلفة القدرة المفقودة وكلفة الوقود وكلفة المتسعات.

1- Introduction

The Energy Management System or( EMS )as we know it today had its origin in the need for electric utility companies to operate their generators as economically as possible . To operate the system as economically as possible requires that the characteristics of all generating units be available so that the most efficient units could be dispatched properly along with the less efficient. In addition, there is a requirement that the on/off scheduling of generators units be done in an efficient manner as well . The scheduling of generators with limited fuel or water supplies is incorporated in energy management systems . This allows operators to further reduce the cost of operation by taking advantage of cheaper fuels or hydropower .

2- Inequality Constraints in Optimization

Practical optimization problems contain inequality as well as equality constraints. The optimization problem can be stated as:

- Minimize the cost function

F(x1, x2… xn)               	                                              (1)

- Subject to the equality constraints

gi(x1,x2,…xn)=0              i=1,2,…,m                                       (2)

- And the inequality constraints

hj(x1,x2,…xn)≤0              j=1,2,…,p                              	          (3)

The Lagrange multiplier is extended to include the inequality constraints by introducing the m-dimensional vector μ of undetermined quantities.

3-  Economic Dispatch Including Losses

The active power transmission losses may amount to 20 to 30% of the total load demand, ideally, the exact power flow equations should be used to obtain the active power transmission losses in the system, however, and the electric power system engineer may OPF for expressing the losses in terms of power generations only.

One common practice for including the effect of transmission losses is to express the total transmission loss as a quadratic function of the generator power outputs in one of the following forms, (Daniel S. Kirschen,2004)

- Simple form:

PL=i=1Nj=1NpiBijpj                                                           (4)

- Kron’s loss formula:

PL=i=1Nj=1NpiBijpj+j=1NB0jpj+B00                                     (5)

Bij are called the loss coefficients, which are assumed to be constant for a base range of loads, and reasonable accuracy is expected when actual operating conditions are close to the base case conditions used to compute the coefficients. The economic dispatch problem is to minimize the overall generation cost, C, which is a function of plant output, constrained by:

- The generation equals the total load demand plus transmission

Losses,

- Each plant output is within the upper and lower generation limits inequality constraints.

Mathematically:

F: Ctotal=i=1NCi=i=1Nai+bipi+cipi2                                 (6)

g : i=1Npi=PD+PL                                            	                  (7)

h : pi(min) ≤ pi ≤pi(max) i=1,..., N                                 (8)

The resulting optimization equation becomes:

L=Ctotal+λPD+PL−i=1Npi+i=1Nμi(max)(pi(max)−pi)+ i=1Nμi(min)(pi−pi(min)) pi<pi(max): μi(max) =0pi>pi(min): μi(min)=0

(9)

The minimum of the unconstrained function is found when:

∂L∂pi=0                                                                                       (10)

∂L∂λ=0                                                                                    (11)

∂L∂μi(max)=pimax−pi=0   				                         (12)

∂L∂μi(min)=pi−pi(min)=0				             (14)

When generator limits are not violated:

∂L∂pi=0=∂Ctotal∂pi+λ0+∂PL∂pi−1                               	            (15)

∂Ctotal∂pi=∂∂piC1+C2+...+CN=dCidpi		                        (16)

∴ λ=dCidpi+λ∂PL∂pi=11−∂PL∂pidCidpi=PFidCidpi                           (17)

∂L∂λ=0=PD+PL−i=1Npi∴i=1Npi=PD+PL                                    (18)

where PFiis known as the penalty factor of plant i and is given  by:

PFi=11−∂PL∂pi	                                                (19)

The effect of transmission losses introduces a penalty factor that depends on the location of the plant.

- The minimum cost is obtained when the incremental cost of each plant multiplied by its penalty factor is the same for all plants.

The incremental transmission loss is obtained from Kron’s loss formula as,

∂PL∂pi=2j=1NBijpj+Boi                             		              (20)

By setting the fuel cost equal to 1 $/MBTU, can be rewritten as:

dCidpi=bi+2cipi

(21 )

Substituting eq (20) and eq (21) in eq (17), yields :

λ=dCidpi+λ∂PL∂pi=bi+2cipi+2λj=1NBijpj+λBoi                         (22)

Rearranging the eq as:

j≠iN                                           (23)

Extending eq (23) for all plants results in the following linear equations (in matrix form),

c1λ+B11B12⋯B1NB21c2λ+B22⋯B2N⋮⋮⋱⋮BN1BN2…cNλ+BNNp1p2⋮pN=121−Bo1−b1λ1−B02−b2λ⋮1−BoN−bNλ              (24)

To find the optimal dispatch,

- The simultaneous linear equations in eq (24) are solved for an estimated value of , the solution will be pi(1), i=1,N .

- Then the iterative process is continued using the gradient method for the (N+1) system equations formed by eq (7) and eq (24)].( Saadat, H. 1999)

Or in the short form

E * P = D

E= c1λ+B11B12⋯B1NB21c2λ+B22⋯B2N⋮⋮⋱⋮BN1BN2…cNλ+BNN,P=p1p2⋮pN,D=121−Bo1−b1λ1−B02−b2λ⋮1−BoN−bNλ        (25)

To find the optimal dispatch for in estimated value of λ(1) ,the simultaneous linear equation given by (25) is solved. In MATLAB use the command    P=E \ D.

Then the iterative process is continued using the gradient method. To do this, from (23), Pi at the Kth iteration is expressed as:

Pi(K)= λ(K)(1−Boi)−bi−2λ(K)i≠jBijPj(K)2(ci+λ(K)Bii)             ( 26)

Substituting for Pi from (26) in (7) results in

i=1ngλ(K)(1−Boi)−bi−2λ(K)j≠iBijPi(K)2(ci+λ(K)Bii)=PD+PL(K)                         (27)

Or

f(λ(K))=PD+PL(K)	                                                    (28)

Expanding the left –hand side of the above equation in the Taylor series about an operation point λ(K)and neglecting the higher – order terms result in

f(λ)(K)+(df(λ)dλ)(K)Δλ(K)=PD+PL(K)                                    (29)

Or

Δλ(K)=ΔP(K)(df(λ)dλ)(K)=ΔP(K)(dPdλ)(K)                        (30)

where

i=1ng(∂Pi∂λ)(K)=ci(1−Boi)−Biibi−2cij≠iBijPi(K)2(ci+λ(K)Bii)2                       (31)

And therefore,

λ(K+1)=λ(K)+Δλ(K)                                         (32)

where

ΔP(K)=PD+PL(K)−i=1ngPi(K)                                (33)

The process is continued until ΔP(K) is less then a specified accuracy, and a specified accuracy =0.0001. If an approximate loss formula expressed by

PL=i=1ngBiiPi2

(34)

is used, Bij=0, B00=0, and solution of the simultaneous equation given by (26) reduced to the following simple expression

Pi(K)=λ(K)−bi2(ci+λ(K)Bii)                                         (35)

In addition, (31) is reduced to

i=1ng(∂Pi∂λ)(K)=ci+Biibi2(ci+λ(K)Bii)2                                             (36)

4- Outline of the Basic steady state Genetic Algorithm (Witwit,2006)

- Start : Generate random population of n chromosomes (suitable solutions for the problem).

- Fitness : Evaluate the fitness f(x) of each chromosome x in the population

- New population : Create a new population by repeating following steps until the new population is complete:

      - Selection : Select two parent chromosomes from a population according to their fitness (the better fitness, the bigger chance to be selected).

      - Crossover : With a crossover probability cross over the parents to form a new offspring (children). If no crossover is performed, an offspring is an exact copy of parents.

      - Mutation : With a mutation probability mutates a new offspring at each locus (position in chromosome).

      - Accepting : Place a new offspring in a new population .

- Replace: Use new generated population for a further run of algorithm

- Test : If the end condition is satisfied, stop, and return the best solution in current population.

- Loop : Go to step 2.

5- The Capacitor Placement

The capacitor placement problem comprises two terms; first term represents the cost of capacitor placement, which has two components:

- Fixed installation cost.

- Purchase cost.

The second term represents the total cost of energy loss. The energy loss is obtained by summing up the power losses for each load level multiplied by the duration of the load level. In practice, capacitors banks of standard discrete capacitance are ground. Hence, capacitor size is the discrete variables. The cost of capacitor placement at location                                                                                         k with sizing UK0 is:

CK(UK0)=KC.UK0US+CKfUK0                                         (37)

where KC is the cost of one bank of capacitor or is a fixed capacitor to be installed.CKfUK0.CKrepresents the cost associated with the capacitor installation at location k.]. For each load level, let the real power loss in the system bePloss,iXi,Ui, then the total cost of energy loss can be written as:

Kei=1NtTi.Ploss,iXi,Ui, where Ti is the duration for load level i and constant Keis the energy cost per unit. Let NC possible location to place capacitors and Nt different loads levels. Let C represent the set of fixed capacitors,

Let nt=[1,2,......Nt] and nc=[1,2,.....NC], then the general capacitor placement problem is formulated as follows:

MinimizeU0,UkK=1NCCKUK0+Kei=1NtTi.PLoss,iXi,Ui                      (38)

subject to:

CKUK0=KC.UK0Us+CKfUK010

where CKUK0 is the annual cost of capacitor at location K with sizeUK.

In this work we have developed the equation formulated of the general capacitor placement to find the annual cost by dividing the cost of capacitor by 10 by assuming that the capacitor lasts 10 years at least. These assumptions assist in minimizing the energy cost and the total cost of the system by the ability of an addition of the many of capacitors to the system.

U0 is the sizing vector whose components are multiples of the standards size of one bank.Uiis the control setting vector at load level i.( Mansour Y., 2005)

6- Computing of Eigenvalues and Eigenvectors

The modal analysis mainly depends on the power-flow Jacobian matrix. An algorithm for the computing of participation factor as fig (3).

ΔPΔQ=J11J12J21J22⋅ΔθΔ|V|                            (39)

By letting ΔP = 0  in Equation (31):

ΔP=0=J11Δθ+J12Δ|V|,Δθ=−J11−1J12Δ|V|                 (40)

And

ΔQ=J21Δθ+J22Δ|V|                                       (41)

Substituting Equation (41) in Equation (42): yields

ΔQ=JRΔ|V|                                                    (42)

where

JR=J22−J21J11−1J12                                         (43)

JR is the reduced Jacobian matrix of the system.

Equation (34) can be written as:

Δ|V|=JR−1ΔQ                                                 (44)

The matrix JRrepresents the linearzed relationship between the incremental changes in bus voltage (ΔV) and bus reactive power injection (ΔQ). It’s well known that, the system voltage is affected by both real and reactive power variations. In order to focus the study of the reactive demand and supply problem of the system as well as minimize computational effort by reducing dimensions of the Jacobian matrix J the real power (ΔP = 0) and angle part from the system in Equation (39 ) are eliminated. The eigenvalues and eigenvectors of the reduced order Jacobian matrix JRare used for the voltage stability characteristics analysis. Voltage instability can be detected by identifying modes of the eigenvalues matrixJR. The magnitude of the eigenvalues provides a relative measure of proximity to instability. The eigenvectors on the other hand present information related to the mechanism of loss of voltage stability. Eigenvalue analysis of JRresults in the following:

JR=ΦΛΓ                           (45)

where

Φ= right eigenvector matrix of JR

Γ= left eigenvector matrix of JR

Λ= diagonal eigenvalue matrix of JR

Equation (45) can be written as:

JR−1=ΦΛ−1Γ                                                                (46)

where      ΦΓ=1

Substituting Equation (46) in Equation (44) gives

Δ|V|=ΦΛ−1ΓΔQ

Or

Δ|V|=iΦiΓiλiΔQ                                                          (47)

where λi is the ith eigenvalue, Φi is the of ith column right eigenvector and Γi is the ith row left eigenvector of matrix JR  Each eigenvalueλi and corresponding right and left eigenvectorsΦi andΓi, define the ithmode of he system. The ithmodal reactive power variation is defined as:

ΔQmi=KiΦi                                                             (48)

whereKi is a scale factor to normalize vector ΔQiso that

Ki2jΦji=1                                                          (49)

where Φjithe jthelement of Φi

The corresponding ithmodal voltage variation is:

Δ|V|mi=1λiΔQmi                                                   (50)

Equation (50) can be summarized as follows:

1. Ifλi=0, the ithmodal voltage will collapse because any change in that modal reactive power will cause infinite modal voltage variation.

2. Ifλi>0, theith modal voltage andith reactive power variation are along the same direction, indicating that the system is voltage stable.

3. Ifλi<0, theith modal voltage and theith reactive power variation are along the opposite directions, indicating that the system is voltage unstable.

If Φiand Γi represent the right- and left- hand eigenvectors, respectively, for the eigenvalue λiof the matrixJR, then the participation factor measuring the participation of the Kthbus in ithmode is defined as

Pki=ΦkiΓik                                                               (51)

Note that for all the small eigenvalues, bus participation factors determine the area close to voltage instability. Equation (51) implies that Pkishows the participationPki of theith eigenvalue to the sensitivity at bus k. The node or bus k with highest value is the most contributing factor in determining the sensitivity at ithmode. Therefore, the bus participation factor determines the area close to voltage instability provided by the smallest eigenvalue of JR( Lee and 2007)

.

Figure (3)  Algorithm for the computing of Participation Factor

Obtain the load flow solution for a base case of the      system and get the Jacobian matrix (J)

7- The Discussion of the Proposed System

We will test case in the proposed system :

- The case of 22 – bus bars with 10 generators (Iraq system).

It is aimed in this research to reduce the total generation cost of power system using optimum economic dispatch by placing some capacitors on the bus bars of power system to reduce the losses of power and this will lead to reduce the total cost.

To select the optimal locations and sizes of the capacitors, we will use the Participation Factor to get the best locations in the bus bars of power system. And we will use the Genetic Algorithm steady state to get the best sizes for these capacitors.

In this section we will present how we will get the best total cost and compare with the previous cost that will get it before the adding of capacitors. So we will present the procedures that will fellow them to get the cost of system:

- Compute the Newton Raphson method.

- Compute the losses of power system.

- Determine the economic dispatch before compensation using Lagrange multiplier method.

- Compute the Participation Factor to detect the optimal locations in the power system.

- Apply the Genetic Algorithm to select the best sizes for the capacitors that will place in the selected locations (in the previous procedure).

- Compute the optimum economic dispatch to get the total cost, and compare with the previous cost.

8-  Iraq System / 10 – Generators:

A 10- generator case (the system of Iraq) with 22 bus bars is taken to illustrate the proposed algorithm to solve the economic dispatch problem.

As in the previous section the same procedures will be computed to get the reduced total cost.

Figure (1) presents the schematic of 22 bus transmission system.

The Newton - Raphson, B– coefficients and the losses of the system, and the total cost for the power system before placing the capacitors are presented in Tables (1), (2) below:

Figure (1) The Schematic of Iraq System (22 Bus Bars)

Power Flow Solution by Newton-Raphson Method

Maximum Power Mismatch = 6.38099e-007

No. of Iterations = 10

Table (1) Power Flow Solution By Newton-Raphson before placing the capacitors

| Bus | Voltage | Angle | Load | Load | Generation | Generation | Injected |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Mag | Degree | MW | Mvar | MW | Mvar | Mvar |
| 1 | 1.020 | 0.000 | 0.000 | 0.000 | -1121.349 | 223.221 | 0.000 |
| 2 | 1.018 | 1.520 | 394.000 | 16.000 | 0.000 | 0.000 | 0.000 |
| 3 | 1.020 | 6.417 | 98.000 | 2.000 | 1320.000 | 88.710 | 0.000 |
| 4 | 1.070 | 11.837 | 0.000 | 0.000 | 318.000 | 86.934 | 0.000 |
| 5 | 1.058 | 13.511 | 79.000 | 25.000 | 260.000 | 89.215 | 0.000 |
| 6 | 1.022 | 11.161 | 299.000 | 68.000 | 0.000 | 0.000 | 0.000 |
| 7 | 1.040 | 11.631 | 75.000 | 2.000 | 660.000 | -53.439 | 0.000 |
| 8 | 1.025 | 8.112 | 67.000 | 3.000 | 0.000 | 0.000 | 0.000 |
| 9 | 0.971 | 12.379 | 282.000 | 75.000 | 0.000 | 0.000 | 0.000 |
| 10 | 0.977 | 13.271 | 149.000 | 5.000 | 0.000 | 0.000 | 0.000 |
| 11 | 0.974 | 11.998 | 245.000 | 34.000 | 0.000 | 0.000 | 0.000 |
| 12 | 0.990 | 16.783 | 0.000 | 0.000 | 492.000 | 52.635 | 0.000 |
| 13 | 0.993 | 12.011 | 65.000 | 54.000 | 0.000 | 0.000 | 0.000 |
| 14 | 1.000 | 15.155 | 170.000 | 10.000 | 1200.000 | 155.428 | 0.000 |
| 15 | 0.988 | 20.938 | 124.000 | 63.000 | 0.000 | 0.000 | 0.000 |
| 16 | 0.984 | 13.932 | 217.000 | 74.000 | 0.000 | 0.000 | 0.000 |
| 17 | 1.001 | 13.676 | 159.000 | 47.000 | 0.000 | 0.000 | 0.000 |
| 18 | 1.000 | 22.387 | 165.000 | 94.000 | 840.000 | 186.857 | 0.000 |
| 19 | 0.964 | 22.271 | 40.000 | 58.000 | 0.000 | 0.000 | 0.000 |
| 20 | 1.020 | 23.581 | 185.000 | 10.000 | 250.000 | 60.922 | 0.000 |
| 21 | 1.040 | 25.479 | 89.000 | 15.000 | 400.000 | 135.965 | 0.000 |
| 22 | 0.967 | 12.248 | 38.000 | 30.000 | 0.000 | 0.000 | 0.000 |
| Total |  |  | 2940.000 | 685.000 | 4618.651 | 1026.448 | 0.000 |

Table (2) The B-coefficients , The Power losses , and The total generation cost

| B= | 0.0048 | 0.0030 | 0.0027 | 0.0016 | 0.0002 | -0.0001 | -0.0011 | -0.0013 | -0.0018 | -0.0020 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.0030 | 0.0032 | 0.0030 | 0.0019 | 0.0003 | 0.0001 | -0.0010 | -0.0011 | -0.0017 | -0.0019 |
|  | 0.0027 | 0.0030 | 0.0030 | 0.0033 | 0.0003 | -0.0001 | -0.0010 | -0.0011 | -0.0017 | -0.0019 |
|  | 0.0016 | 0.0019 | 0.0019 | 0.0096 | -0.0000 | -0.0002 | -0.0011 | -0.0012 | -0.0018 | -0.0020 |
|  | 0.0002 | 0.0003 | 0.0003 | -0.0000 | 0.0014 | 0.0007 | -0.0003 | -0.0004 | -0.0010 | -0.0012 |
|  | -0.0001 | 0.0001 | 0.0001 | -0.0002 | 0.0007 | 0.0075 | -0.0004 | -0.0006 | -0.0012 | -0.0014 |
|  | -0.0011 | -0.0010 | -0.0010 | -0.0011 | -0.0003 | -0.0004 | 0.0024 | -0.0005 | -0.0001 | -0.0003 |
|  | -0.0013 | -0.0011 | -0.0011 | -0.0012 | -0.0004 | -0.0006 | 0.0005 | 0.0019 | 0.0012 | 0.0010 |
|  | -0.0018 | -0.0017 | -0.0017 | -0.0018 | -0.0010 | -0.0012 | -0.0001 | 0.0012 | 0.0071 | 0.0066 |
|  | -0.0020 | -0.0019 | -0.0019 | -0.0020 | -0.0012 | -0.0014 | -0.0003 | 0.0010 | 0.0066 | 0.0130 |

| B0= 1 .0e-003* | 0.0083 | -0.0380 | 0.0842 | 0.1427 | -0.0844 | 0.0149 | -0.0423 | -0.0201 | -0.0829 | -0.1093 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

B00 =    0.0087

Total system loss = 58.7616 MW

Total generation cost =   66290.72 $/h

To reduce the total cost , the method of Participation factor is performed by computing the Jacobian Matrix reduction JR to analyze the stability of the voltage. This can be made by computing the eigenvalues and eigenvectors for JR matrix, where the values of eigenvalues give a proximate for the voltage instability at the load level ,and from this we can determine the  Participation Factor for the buses of the system , and the buses that have the minimum eigenvalues are selected and large PF(participation factor) values are used to inject the capacitors in the buses.

Figure (2) below presents the load level participation factor. And Tables (3), (4) below present the results of eigenvalues for the selected buses and the Participation factor for these buses:

Figure (2) The Load Level Participation Factor for the Eigenvalues

Table (3) The Eigenvalues of The Selected Buses

| Location | Eigenvalue |
| --- | --- |
| 2 | 247.071 |
| 6 | 288.752 |
| 8 | 224.783 |
| 9 | 105.238 |
| 10 | 87.196 |
| 11 | 77.561 |
| 13 | 51.724 |
| 15 | 23.209 |
| 16 | 17.698 |
| 17 | 5.614 |
| 19 | 4.597 |
| 22 | 4.642 |

Table (4) The Participation Factor for The Selected Buses

| Bus No. | Participation Factor |
| --- | --- |
| 2 | 0.0025 |
| 6 | 0.1344 |
| 8 | 0.0025 |
| 9 | 0.1344 |
| 10 | 0.0836 |
| 11 | 0.0374 |
| 13 | 0.0836 |
| 15 | 0.1514 |
| 16 | 0.1623 |
| 17 | 0.1500 |
| 19 | 0.0374 |
| 22 | 0.3115 |

From the results of the participation factor, the 10 candidate locations in the system are selected for placing the capacitors. ( see table (5)).

After the best candidate buses are obtained, the next step is to get the optimal sizes for the capacitors to place them in these buses, this is done by using the Steady State Genetic Algorithm (ssGA). And to achieve this target, we need to building an initial population and getting the fitness function and determined the number of generations. At initial, random population are selected and the encoding of the chromosomes of the population use integer values ( because each chromosome will present the number of capacitors that will be injected to system). Each of chromosome is added to the buses data array to the field of Qsht  (which present the field of capacitors) and compute the power losses for the system.

The value of power losses will be multiplied by the cost of energy and  will be added to the price of capacitors and their maintenance to give the fitness function. So, to get the minimum total cost we must get the best chromosome that has the minimum fitness function.

FitFun=  the cost of capacitor+ the power losses*energy cost +

the maintenance(fixed installment cost)

Energy cost (Ke) = 60$/ MW.h.

The cost of capacitor

- Fixed installment cost = 1000$.

- Purchase = 3500$/bank.

Table (5) presents the location of candidate capacitors and their optimal sizes which are obtained from the ssGA, and Tables (6), (7), and (8) below present the final results to get the reduced total generation costs:

Table (5) The Fixed Capacitors Placement & Their Sizes

| Optimal locations | Optimal Size of Capacitors |
| --- | --- |
| (bus No.) | (MVAR) |
| 6 | 6 |
| 9 | 7 |
| 10 | 15 |
| 11 | 2 |
| 13 | 9 |
| 15 | 5 |
| 16 | 13 |
| 17 | 6 |
| 19 | 11 |
| 22 | 1 |

Absolute value of the slack bus real power mismatch, dpslack =   0.0023 pu

Power Flow Solution by Newton-Raphson Method Maximum Power

Mismatch =7.87172e-007

No. of Iterations = 10

| Bus | Voltage | Angle | Load | Load | Generation | Generation | Injected |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Mag | Degree | MW | Mvar | MW | Mvar | Mvar |
| 1 | 1.020 | 0.000 | 0.000 | 0.000 | 187.047 | 3.596 | 0.000 |
| 2 | 1.019 | -0.246 | 394.000 | 16.000 | 0.000 | 0.000 | 0.000 |
| 3 | 1.020 | 0.414 | 98.000 | 2.000 | 348.000 | -49.847 | 0.000 |
| 4 | 1.070 | 5.497 | 0.000 | 0.000 | 400.000 | 64.665 | 0.000 |
| 5 | 1.058 | 5.516 | 79.000 | 25.000 | 300.000 | 84.714 | 0.000 |
| 6 | 1.022 | -1.270 | 299.000 | 68.000 | 0.000 | 0.000 | 11.000 |
| 7 | 1.040 | -0.733 | 75.000 | 2.000 | 449.000 | -67.958 | 0.000 |
| 8 | 1.025 | -4.252 | 67.000 | 3.000 | 0.000 | 0.000 | 0.000 |
| 9 | 0.976 | -1.516 | 282.000 | 75.000 | 0.000 | 0.000 | 7.000 |
| 10 | 0.984 | -2.556 | 149.000 | 5.000 | 0.000 | 0.000 | 15.000 |
| 11 | 0.979 | -2.542 | 245.000 | 34.000 | 0.000 | 0.000 | 2.000 |
| 12 | 0.990 | 1.649 | 0.000 | 0.000 | 355.000 | 40.907 | 0.000 |
| 13 | 0.996 | -0.004 | 65.000 | 54.000 | 0.000 | 0.000 | 9.000 |
| 14 | 1.000 | -3.541 | 170.000 | 10.000 | 125.000 | 117.085 | 0.000 |
| 15 | 1.009 | 0.348 | 124.000 | 63.000 | 0.000 | 0.000 | 5.000 |
| 16 | 0.991 | -2.797 | 217.000 | 74.000 | 0.000 | 0.000 | 13.000 |
| 17 | 1.006 | -3.651 | 159.000 | 47.000 | 0.000 | 0.000 | 6.000 |
| 18 | 1.020 | 1.119 | 165.000 | 94.000 | 368.827 | 212.467 | 0.000 |
| 19 | 0.994 | 1.018 | 40.000 | 58.000 | 0.000 | 0.000 | 11.000 |
| 20 | 1.040 | 2.247 | 185.000 | 10.000 | 300.000 | 50.125 | 0.000 |
| 21 | 1.060 | 3.637 | 89.000 | 15.000 | 339.000 | 144.516 | 0.000 |
| 22 | 0.974 | -2.829 | 38.000 | 30.000 | 0.000 | 0.000 | 1.000 |
| Total |  |  | 2940.000 | 685.000 | 3173.154 | 600.269 | 80.000 |

Table (6) Power Flow Solution By Newton-Raphson after placing

Table (7) The B-coefficients , The Power losses , and The total generation cost

| B= 1.0e-004* | 0.2280 | 0.1529 | 0.1334 | 0.0756 | 0.0080 | -0.0078 | -0.0569 | -0.0640 | -0.0888 | -0.0997 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.1529 | 0.1429 | 0.1504 | 0.0940 | 0.0185 | 0.0032 | -0.0639 | -0.0650 | -0.0821 | -0.1011 |
|  | 0.1334 | 0.1504 | 0.2293 | 0.1480 | 0.0156 | -0.056 | -0.467 | -0.0550 | -0.0821 | -0.0915 |
|  | 0.0756 | 0.0940 | 0.1480 | 0.4286 | -0.0027 | -0.0140 | -0.0393 | -0.0512 | -0.0828 | -0.0888 |
|  | 0.0080 | 0.0185 | 0.0156 | -0.0027 | 0.0672 | 0.0356 | -0.0170 | -0.0243 | -0.0518 | -0.0639 |
|  | -0.0078 | 0.0032 | -0.0056 | -0.0140 | 0.0356 | 0.3700 | -0.0189 | -0.0276 | -0.0575 | -0.0657 |
|  | -0.0569 | -0.0639 | -0.0467 | -0.0393 | -0.0170 | -0.0189 | -0.0042 | 0.0166 | -0.0026 | -0.0060 |
|  | -0.0640 | -0.0656 | -0.0550 | 0.0512 | -0.0243 | -0.0276 | 0.0166 | 0.0669 | 0.0530 | 0.0389 |
|  | -0.0888 | -0.0821 | -0.0821 | -0.0828 | -0.0518 | 0.0575 | -0.0026 | 0.0630 | 0.3175 | 0.2857 |
|  | -0.0997 | -0.1011 | -0.0915 | -0.0888 | -0.0639 | -0.0657 | -0.0060 | 0.0389 | 0.2857 | 0.5132 |
|  | 0.2280 | 0.1529 | 0.1334 | 0.0756 | 0.0080 | -0.0078 | -0.0569 | -0.0640 | -0.0888 | -0.0997 |

| B0= | -0.0389 | -0.0655 | -0.1010 | -0.1187 | 0.0078 | 0.0088 | 0.0847 | 0.0468 | 0.0261 | 0.0871 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

B00 =    -1.7434

Total system loss = 16.4216 MW

Total generation cost =   35874.18 $/h

Incremental cost of delivered power (system lambda) = 16.024488  $/MWh

Table (8) The Total Losses , The Total Cost, The Lambda of The System, and The Optimal Dispatch of Generation

Optimal Dispatch of Generation

187.5000

139.0000

400.0000

279.1755

345.2646

268.2019

125.4000

279.0975

300.0000

268.1165

9- Conclusions

The following conclusions are derived from this work:

- The power flow method, together with the system losses and the economic dispatch methods can be used to obtain the optimal dispatch of generation.

To get this target will need reducing the losses of system by reducing the loss coefficients. The dispatch method produces a variable named dpslack. This is the difference (absolute value) between the scheduled slack generation determined from the coordination equation, and the slack generation, obtained from the power flow solution. A power flow solution obtained with new scheduling of generation results in a new loss coefficient, which can be used to solve the coordination equation again. The process continues until dpslack is within a specified tolerance, but it is seen that the dpslack value must not be very small because this will increase the iterative and this will lead to increase the cost (it is found that the best value for dpslack is at 0.1).

- The Participation Factor and Steady State Genetic algorithm are used to determine the candidate buses and the number of reactive power gives the accuracy and the enhancement to the work because the first stage (participation factor stage) simultaneously contributes to narrowing down the search domain for the second stage (GA optimization), as well as ensures those buses that are sensitive to voltage problems considered for reactive power compensation.

- The use of ssGA gives more accurate results than the simple genetic algorithm SGA because the SGA replaces the entire parent population with the children. And in elitism the fittest individuals pass unchanged from the parent population to the children while the ssGA replaces few individuals, and it provide a set of solutions rather than only one solution.

Finally, from the computational results, the following can be observed

- The total generation cost for the initial operation condition is reduced by using the total generation cost with optimal dispatch of generation and genetic algorithms.

2.   Mvar of generators is reduced by using injection capacitors.

3.  The solution methodology is based on an optimization technique chosen by genetic algorithms to minimize the objective function while the load constraints and operational constraints like the voltage profile at different load levels are satisfied.

4.   ssGA, which is based on the laws of natural selection and survival of the fittest, has been used successfully to reduce power loss considering balanced condition, because ssGA reaches quickly the region of optimal solutions and its accuracy for one reason: ssGA avoids local minima by searching in several regions.

References

Daniel S. Kirschen, " Fundamentals of Power System Economic"  March 2004  Hardback

Mansour, Y. W. Xu, F. Alvardo and C. Rinzin, "SVC Placement Using Critical Modes of Voltage Instability", Proc. 2005

Lee K.Y.  and Yang F.F., "Optimal reactive power planning using evolutionary algorithms: a comparative study for evolutionary programming, IEEE Trans. On power systems, Vol. 13, No. 1, Feb. 2007, pp. 101-108

Saadat H., "power system analysis", the McGraw-Hill companies, 1999

Witwit A.R., "A Hybrid Algorithm for Data compression Using Huffman & Genetic Algorithms", M.Sc. Thesis, Babylon University , Babylon, 2006.