Computer Program for Analysis and Design of Rectangular Edge-Supported Two-Way Slabs by Coefficient Method

Hayder Mohammed Kadhim Al-Mutairee

Department of Civil Engineering/ College of Engineering/ University of Babylon

Abstract

A computer program to analysis and design any rectangular edge-supported two-way slab under effects of uniformly distributed load is presented here. The program which called (POTWS – Program Of Two-Way Slabs) analyzes the two-way slab according to the coefficient method (or method three) and satisfies all design requirements of the (ACI 318M-2008). The deflection computes and compares with the permissible limit. QuickBASIC language used to construct the program. The analysis steps and design results concern middle strip of the slab. The main program with related external file briefly discussed. The program results were accurate, precise and coincide with hand calculations results. The present program facilitates the designer job and makes him gain the time as well as opens the way to study the effects of many design parameters easily.

الخلاصة

برنامج حاسوبي لتحليل وتصميم اي سقف مستطيل ثنائي العمل و ذو نهايات مسندة تحت تأثير أحمال منتشرة بأنتظام تم أعداده. البرنامج أطلق عليه (POTWS) (برنامج السقوف ثنائية العمل) يقوم بتحليل السقوف ثنائية العمل وفقاً لطريقة المعامل (أو طريقة ثلاثة) ويوفر كل متطلبات معهد الخرسانة الأمريكي (ACI 318M-2008). حسب الهطول وتم مقارنته مع الحدود المسموحة. أستخدامت لغة (QuickBASIC)  لأعداد البرنامج. نتائج خطوات التحليل والتصميم متعلقة بالشريط الوسطي للسقف. البرنامج الرئيسي مع الملف الخارجي المتعلق به تم شرحه بإختصار. نتائج البرنامج كانت مضبوطة، دقيقة ومتطابقة مع نتائج الحسابات اليدوية. البرنامج المستحضر يسهل وظيفة المصمم ويجعله رابحاً للوقت أضافة الى إنه يفتح الطريق لدراسة معاملات تصميم عدة بسهولة.

1- Introduction

This research is limited to analysis and design a two-way rectangular slabs supported on stiff beams or wall. Two-way slabs are rarely statically determinate in their internal moments and shears. Such slabs most highly indeterminate of all ordinary structures, thus require a three dimensional approach for analysis, even single panel simply supported [Ferguson, 1981].

An exact analysis of forces, moments and displacements in a two-way slab is complex due to the fact of highly indeterminate nature; this is true even when the effects of creep and nonlinear behavior of concrete are neglected [Hassoun, 1985]. Numerical methods such as finite elements can be used for analysis of the two-way slab but it is required mesh of finite elements which leads to a lot of input data which it is not suitable for everyday practical design. The application simplicity of ACI Code methods (coefficient methods: Method Two , and Method Three [Nilson and Winter, 1986] ) made these methods suitable for practical design as well as some designers believe that these methods are still valuable design methods [Ferguson, 1981] and [Syal and Goel, 2004].

2- Coefficient Methods

The complexity of the generalized approach for two-way slabs, particularly for systems that do not meet the requirements permitting analysis by the "direct design method" [Nilson et al., 2004], has led many engineers to continue using the coefficient methods [[Syal and Goel, 2004], [Nilson and Winter, 1986] and [Vazirani and Chandola, 2004]] for special cases of two-way slabs supported on four sides of each slab panel.

Though coefficient was not a part of the [ACI 318M-2008] or later ACI Codes, its use is permissible under the current code provision that a slab system may be designed by any procedure satisfying conditions of equilibrium and geometric compatibility, if it is shown that the design strength at every section is at least equal to the required strength and the serviceability requirements are met [[Ferguson, 1981], [Nilson and Winter, 1986] and [Amer et al., 2008]].

3- Deflection Control

The [ACI 318M-2008], section 9.5.3, specifies a minimum slab thickness in two-way slabs to control deflection, this specification as illustrated in references of [Nilson et al., 2004], depend on properties of (beams around the slab), therefore, eliminate here. Alternative to the use of minimum slab thickness equations, the deflection at the center of a slab panel can be calculated, and results compared against limitations of ACI-Code, Table 9.5 (b), as explained in Reference [Nilson and Winter, 1986], and as illustrated in steps of main program (680 to 780).

4- The Present Program

4.1 Details of the Main Program

This program had been written utilizing [QuickBASIC Help Manual] and the steps as follows:

Steps 10 to 60: clear window, dimension statements, define the roots and open files for input coefficients and output results.

Steps 70 to 120: read the matrix of coefficients and closed the input file (Coeffs.Dat).

Steps 130 to 320: input case of slab, dimensions of slab, loads, properties of materials, size of reinforcement, required thickness of slab and type of occupation.

Steps 330 to 470: computation of total dead load, ultimate load and coefficients.

Steps 480 to 570: computation the applied shear forces for long and short directions and nominal shear strength provided by concrete section.

Steps 580 to 610: compute the minimum area of reinforcement.

Steps 620 to 650: computation of external moments.

Steps 660 to 780: determination of deflection and comparison with permissible limits.

Steps 790 to 820: calculation of design parameters.

Steps 830 to 990: calculation of negative reinforcement for short direction.

Steps 1000 to 1160: calculation of positive reinforcement for short direction.

Steps 1170 to 1330: calculation of negative reinforcement for long direction.

Steps 1340 to 1500: calculation of positive reinforcement for long direction.

Steps 1510 to 1630: printing output results on screen.

Steps 1640 to 1740: printing output results in output file (Out.Dat).

Steps 1750: close output file.

Steps 1760: end program.

Note: when the thickness of the slab (t) does not check the shear or strength requirement, the program prints these errors and cancels the input data then starts from the beginning, while when serviceability requirement does not check the program offers two choices either continue or start from beginning, after printing the error.

4.2 List of the Main Program

REM ===========================================================

REM = Computer Program for Analysis and Design of Rectangular Two-Way Slabs By Coefficient Method =

REM ===========================================================

10    CLS

20    DIM COE(99, 9), NPO(9)

30    DRI$ = "D:\Coeffs\Coeffs.DAT"

40    DRO$ = "D:\Output\Out.DAT"

50    OPEN DRI$ FOR INPUT AS #1

60    OPEN DRO$ FOR OUTPUT AS #2

70    FOR I = 1 TO 99

80    FOR J = 1 TO 9

90    INPUT #1, COE(I, J)

100  NEXT J

110  NEXT I

120  CLOSE #1

REM  ***** INPUT INFORMATION *****

REM  -----------------------------------------------

130   INPUT "INPUT CASE OF SLAB ", C

140   INPUT "INPUT CLEAR LONGER SPAN (m) ", L

150   INPUT "INPUT CLEAR SHORTER SPAN (m) ", W

160   INPUT "INPUT SERVICE LIVE LOAD (kN/m^2) ", WL

170   INPUT "INPUT SERVICE SUPERIMPOSED DEAD LOAD (kN/m^2) ", Wds

180   INPUT "INPUT YIELD STRENTH OF STEEL (MPa) ", fy

190   INPUT "INPUT COMPRESSIVE STRENTH OF CONCRETE (Cylinder)(MPa) ", fc1

200   INPUT "INPUT DIMETER OF BARS (mm) ", db

210   INPUT "INPUT THICKNESS OF THE SLAB (m) ", t

220   PRINT " SELECT THE TYPE OF OCCUPATION OF SLAB (1) OR (2) OR (3): "

230   PRINT " (1)- WHEN SLAB NOT SUPPORT OR ATTACHED TO NONSTRUCTURAL "

240   PRINT "        ELEMENTS LIKELY TO BE DAMAGED BY LARGE DEFLECTION       "

250   PRINT " (2)- WHEN SLAB SUPPORT OR ATTACHED TO NONSTRUCTURAL           "

260   PRINT "        ELEMENTS LIKELY TO BE DAMAGED BY LARGE DEFLECTIONS    "

270   PRINT " (3)- WHEN SLAB SUPPORT OR ATTACHED TO NONSTRUCTURAL           "

280   PRINT "   ELEMENTS NOT LIKELY TO BE DAMAGED BY LARGE DEFLECTIONS "

290   INPUT ID

300   IF ID = 1 THEN FD = 360

310   IF ID = 2 THEN FD = 480

320   IF ID = 3 THEN FD = 240

REM  ***** COMPUTATIONS OF LOADS AND COEFFICIENTS *****

REM  ------------------------------------------------------------------------------------

330   WD = Wds + 24.5 * t

340   Wu = 1.6 * WL + 1.2 * WD

350   m = W / L

360   mm = INT(m * 20) / 20

370   IF m < .5 THEN PRINT "Error !! This Slab Is One-Way, Change Dimensions ": GOTO 130

380   IF m > 1 THEN PRINT "Error !! In Dimensions Of Slab ": GOTO 130

390   HHH = (C - 1) * 11 + (20 - mm * 20)

400   FOR K = 2 TO 9

410   NPO(K) = COE(HHH, K) + (COE(HHH + 1, K) - COE(HHH, K)) * (mm + .05 - m) / (.05)

420   NEXT K

430   NPO(1) = COE(HHH, 1)

440   CA = NPO(1): Cnw = NPO(2): Cnl = NPO(3): Cpwl = NPO(4): Cpll = NPO(5)

450   Cpwd = NPO(6): Cpld = NPO(7): SFw = NPO(8): SFl = NPO(9)

460   dw = (1000 * t) - 20 - db / 2

470   dl = (1000 * t) - 20 - 1.5 * db

REM  ***********  SHEAR FORCES  *********

REM  --------------------------------------------------------

480   RL = (Wu * L * SFl) / 2

490   Vul = (RL - Wu * SFl * (dl / 1000)) / .75

500   Vc = 170 * SQR(fc1) * (dl / 1000)

510   IF Vul > Vc THEN PRINT " Change The Thickness Of Slab, Vul/.75 > Vc"

520   IF Vul > Vc THEN GOTO 130

530   RW = (Wu * W * SFw) / 2

540   Vuw = (RW - Wu * SFw * (dw / 1000)) / .75

550   Vc = 170 * SQR(fc1) * (dw / 1000)

560   IF Vuw > Vc THEN PRINT " Change The Thickness Of Slab, Vus/.75 > Vc"

570   IF Vuw > Vc THEN GOTO 130

REM  ***** MINIMUM AREA OF REINFORCEMENT *****

REM  ------------------------------------------------------------------------

580   IF fy < 420 THEN ASmin = 2 * (t * 1000)

590   IF fy = 420 THEN ASmin = 1.8 * (t * 1000)

600   IF fy > 420 THEN FAS = (.756 / fy): IF FAS < .0014 THEN FAS = .0014

610   IF fy > 420 THEN ASmin = 1000 * FAS * (t * 1000)

REM  ***** CALCULATIONS OF MOMENTS *****

REM  -------------------------------------------------------------

620   Mnw = (Cnw * Wu * W ^ 2)

630   Mpw = (Cpwl * 1.6 * WL + Cpwd * 1.2 * WD) * W ^ 2

640   Mnl = (Cnl * Wu * L ^ 2)

650   Mpl = (Cpll * 1.6 * WL + Cpld * 1.2 * WD) * L ^ 2

660   Mnw = INT(Mnw * 100) / 100: Mpw = INT(Mpw * 100) / 100

670   Mnl = INT(Mnl * 100) / 100: Mpl = INT(Mpl * 100) / 100

REM  ***** CHECK DEFLECTION IN TWO WAY-SLAB *****

REM  ---------------------------------------------------------------------------

660  DefL = W / FD * 1000: DF = 1 / 16: LF = 3 / 32: IZ = 0

670  INPUT "The Edge-Supports Masonry Walls? (1 for Yes and 2 for No) ", IZ

680  Ec = 4700 * fc1 ^ .5: Ie = 1000 * (1000 * t) ^ 3 / 12

690  Mpwdd = Cpwd * WD * W ^ 2: Mpwld = Cpwl * WL * W ^ 2

700  Mpldd = Cpld * WD * L ^ 2: Mplld = Cpll * WL * L ^ 2

710  IF IZ = 1 THEN DF = 5 / 48: IF IZ = 1 THEN LF = 5 / 48

720  DLd = DF / (2 * Ec * Ie) * (Mpwdd * W ^ 2 + Mpldd * L ^ 2) * 10 ^ 12

730  DLl = LF / (2 * Ec * Ie) * (Mpwld * W ^ 2 + Mplld * L ^ 2) * 10 ^ 12

740  IF ID = 1 THEN DLT = DLl: GOTO 770

750  IF ID = 2 THEN DLT = 1.5 * DLd + DLl: GOTO 770

760  IF ID = 3 THEN DLT = 1.5 * DLd + DLl: GOTO 770

770  IF DefL < DLT THEN INPUT " CONTINUE 1-YES. 2-NO. ", IU

780  IF IU = 2 THEN PRINT " DEFLECTION NOT CHECKED ": GOTO 130

REM  ***** CALCULATIONS OF REINFORCEMENTS *****

REM  -------------------------------------------------------------------------

790   IF fc1 <= 28 THEN B1 = .85 ELSE B1 = (.85 - .05 * (fc1 - 28) / 7)

800   IF B1 < .65 THEN B1 = .65

810   OUmax = (382.5 * B1 / (fy + 600))

820   Rut = 2.55 * B1 * fc1 / (8 * fy)

830   FII = .9: IFII = 0

840   Rnw = (Mnw / (FII * fc1 * dw ^ 2 / 1000))

850   KK1 = .7183 - (1.695 * Rnw)

860   IF KK1 < 0 THEN PRINT "ASnw > ASmax, Increase Thickness": GOTO 130

870   OUnw = (.8475 - SQR(KK1))

880   IF OUnw > OUmax THEN PRINT "ASnw > ASmax, Increase Thickness": GOTO 130

890   ROnw = OUnw * fc1 / fy

900   ASnw = ROnw * 1000 * dw

910   IF IFII > 0 GOTO 1000

920   IF ROnw <= Rut GOTO 1000

930   a = ASnw * fy / (850 * fc1)

940   CC = a / B1

950   Et = .003 * (dw - CC) / CC

960   FII = .65 + (Et - .002) * 250 / 3

970   IF FII < .65 THEN FII = .65

980   IFII = 1

990   GOTO 840

1000   FII = .9: IFII = 0

1010   Rpw = (Mpw / (FII * fc1 * dw ^ 2 / 1000))

1020   KK1 = .7183 - (1.695 * Rpw)

1030   IF KK1 < 0 THEN PRINT "ASpw > ASmax, Increase Thickness": GOTO 130

1040   OUpw = (.8475 - SQR(KK1))

1050   IF OUpw > OUmax THEN PRINT "ASpw > ASmax, Increase Thickness": GOTO 130

1060   ROpw = OUpw * fc1 / fy

1070   ASpw = ROpw * 1000 * dw

1080   IF IFII > 0 GOTO 1170

1090   IF ROpw <= Rut GOTO 1170

1100   a = ASpw * fy / (850 * fc1)

1110   CC = a / B1

1120   Et = .003 * (dw - CC) / CC

1130   FII = .65 + (Et - .002) * 250 / 3

1140   IF FII < .65 THEN FII = .65

1150   IFII = 1

1160   GOTO 1010

1170  FII = .9: IFII = 0

1180  Rnl = (Mnl / (FII * fc1 * dl ^ 2 / 1000))

1190  KK1 = .7183 - (1.695 * Rnl)

1200  IF KK1 < 0 THEN PRINT "ASnl > ASmax, Increase Thickness": GOTO 130

1210  OUnl = (.8475 - SQR(KK1))

1220  IF OUnl > OUmax THEN PRINT "ASnl > ASmax, Increase Thickness": GOTO 130

1230  ROnl = OUnl * fc1 / fy

1240  ASnl = ROnl * 1000 * dl

1250  IF IFII > 0 GOTO 1340

1260  IF ROnl <= Rut GOTO 1340

1270  a = ASnl * fy / (850 * fc1)

1280  CC = a / B1

1290  Et = .003 * (dl - CC) / CC

1300  FII = .65 + (Et - .002) * 250 / 3

1310  IF FII < .65 THEN FII = .65

1320  IFII = 1

1330  GOTO 1180

1340  FII = .9: IFII = 0

1350  Rpl = (Mpl / (FII * fc1 * dl ^ 2 / 1000))

1360  KK1 = .7183 - (1.695 * Rpl)

1370  IF KK1 < 0 THEN PRINT "ASpl > ASmax, Increase Thickness": GOTO 130

1380  OUpl = (.8475 - SQR(KK1))

1390  IF OUpl > OUmax THEN PRINT "ASpl > ASmax, Increase Thickness": GOTO 130

1400  ROpl = OUpl * fc1 / fy

1410  ASpl = ROpl * 1000 * dl

1420  IF IFII > 0 GOTO 1510

1430  IF ROpl <= Rut GOTO 1510

1440  a = ASpl * fy / (850 * fc1)

1450  CC = a / B1

1460  Et = .003 * (dl - CC) / CC

1470  FII = .65 + (Et - .002) * 250 / 3

1480  IF FII < .65 THEN FII = .65

1490  IFII = 1

1500  GOTO 1350

REM  ***** OUTPUT RESULTS *****

REM  -------------------------------------------

1510  ASnw = INT(ASnw + .5): ASnl = INT(ASnl + .5): ASpw = INT(ASpw + .5)

1520  ASpl = INT(ASpl + .5): ASmin = INT(ASmin + .5)

1530  PRINT " Negative moment of short direction (kN.m/m) ="; Mnw

1540  PRINT " Positive moment of short direction (kN.m/m) ="; Mpw

1550  PRINT " Negative moment of long  direction (kN.m/m) ="; Mnl

1560  PRINT " Positive moment of long  direction (kN.m/m) ="; Mpl

1570  PRINT " Negative area of reinforcement  for short direction (mm^2/m) ="; ASnw

1580  PRINT " Positive area of reinforcement  for short direction (mm^2/m) ="; ASpw

1590  PRINT " Negative area of reinforcement  for long  direction (mm^2/m) ="; ASnl

1600  PRINT " Positive area of reinforcement  for long  direction (mm^2/m) ="; ASpl

1610  PRINT " Minimum  area of reinforcement  must  be  provided  (mm^2/m) ="; ASmin

1620  PRINT " Permissible deflection (mm) ="; DefL

1630  PRINT " Total deflection as computed (mm) ="; DLT

1640  PRINT #2, " Negative moment of short direction (kN.m/m) ="; Mnw

1650  PRINT #2, " Positive moment of short direction (kN.m/m) ="; Mpw

1660  PRINT #2, " Negative moment of long  direction (kN.m/m) ="; Mnl

1670  PRINT #2, " Positive moment of long  direction (kN.m/m) ="; Mpl

1680  PRINT #2, " Negative area of reinforcement  for short direction (mm^2/m) ="; ASnw

1690  PRINT #2, " Positive area of reinforcement  for short direction (mm^2/m) ="; ASpw

1700  PRINT #2, " Negative area of reinforcement  for long  direction (mm^2/m) ="; ASnl

1710  PRINT #2, " Positive area of reinforcement  for long  direction (mm^2/m) ="; ASpl

1720  PRINT #2, " Minimum  area of reinforcement  must  be  provided  (mm^2/m) ="; ASmin

1730  PRINT #2, " Permissible deflection (mm) ="; DefL

1740  PRINT #2, " Total deflection as computed (mm) ="; DLT

1750  CLOSE #2

1760  END

4.3 Related External File (Coefficients File)

1,0,0,.036,.036,.036,.036,.5,.5

1,0,0,.04,.033,.04,.033,.55,.45

1,0,0,.045,.029,.045,.029,.6,.4

1, 0, 0,.05,.026,.05,.026,.66,.34

1, 0, 0,.056,.023,.056,.023,.71,.29

1, 0, 0,.061,.019,.061,.019,.76,.24

1, 0, 0,.068,.016,.068,.016,.81,.19

1, 0, 0,.074,.013,.074,.013,.85,.15

1, 0, 0,.081,.01,.081,.01,.89,.11

1, 0, 0,.088,.008,.088,.008,.92,.08

1, 0, 0,.095,.006,.095,.006,.94,.06

2,.045,.045,.027,.027,.018,.018,.5,.5

2,.05,.041,.03,.025,.02,.016,.55,.45

2,.055,.037,.034,.022,.022,.014,.6,.4

2,.06,.031,.037,.019,.024,.012,.66,.34

2,.065,.027,.041,.017,.026,.011,.71,.29

2,.069,.022,.045,.014,.028,.009,.76,.24

2,.074,.017,.049,.012,.03,.007,.81,.19

2,.077,.014,.053,.01,.032,.006,.85,.15

2,.081,.01,.058,.007,.034,.004,.89,.11

2,.084,.007,.062,.006,.035,.003,.92,.08

2,.086,.006,.066,.004,.037,.002,.94,.06

3,0,.076,.027,.032,.018,.027,.17,.83

3,0,.072,.031,.029,.021,.025,.2,.8

3,0,.07,.035,.027,.025,.024,.23,.77

3,0,.065,.04,.024,.029,.022,.28,.72

3,0,.061,.045,.022,.034,.02,.33,.67

3,0,.056,.051,.019,.04,.018,.39,.61

3,0,.05,.057,.016,.046,.016,.45,.55

3,0,.043,.064,.014,.054,.014,.53,.47

3,0,.035,.071,.011,.062,.011,.61,.39

3,0,.028,.08,.009,.071,.009,.69,.31

3,0,.022,.088,.007,.08,.007,.76,.24

4,.05,.05,.032,.032,.027,.027,.5,.5

4,.055,.045,.035,.029,.03,.024,.55,.45

4,.06,.04,.039,.026,.033,.022,.6,.4

4,.066,.034,.043,.023,.036,.019,.66,.34

4,.071,.029,.048,.02,.039,.016,.71,.29

4,.076,.024,.052,.016,.043,.013,.76,.24

4,.081,.019,.057,.014,.046,.011,.81,.19

4,.085,.015,.062,.011,.05,.009,.85,.15

4,.089,.011,.067,.009,.053,.007,.89,.11

4,.092,.008,.072,.007,.056,.005,.92,.08

4,.094,.006,.077,.005,.059,.004,.94,.06

5,.075,0,.032,.027,.027,.018,.83,.17

5,.079,0,.034,.024,.028,.015,.86,.14

5,.08,0,.037,.021,.029,.013,.88,.12

5,.082,0,.041,.019,.031,.011,.9,.1

5,.083,0,.044,.016,.032,.009,.92,.08

5,.085,0,.047,.013,.033,.007,.94,.06

5,.086,0,.051,.011,.035,.005,.95,.05

5,.087,0,.055,.009,.036,.004,.96,.04

5,.088,0,.059,.007,.037,.003,.97,.03

5,.089,0,.063,.005,.038,.002,.98,.02

5,.09,0,.067,.004,.039,.001,.99,.01

6,.071,0,.035,.032,.033,.027,.71,.29

6,.075,0,.038,.029,.036,.024,.75,.25

6,.079,0,.042,.025,.039,.021,.79,.21

6,.083,0,.046,.022,.042,.017,.83,.17

6,.086,0,.051,.019,.045,.015,.86,.14

6,.088,0,.055,.016,.048,.012,.88,.12

6,.091,0,.06,.013,.051,.009,.91,.09

6,.093,0,.064,.01,.054,.007,.93,.07

6,.095,0,.068,.008,.056,.006,.95,.05

6,.096,0,.073,.006,.058,.004,.96,.04

6,.097,0,.078,.005,.061,.003,.97,.03

7,0,.071,.032,.035,.027,.033,.29,.71

7,0,.067,.036,.032,.031,.031,.33,.67

7,0,.062,.04,.029,.035,.028,.38,.62

7,0,.057,.045,.026,.04,.025,.43,.57

7,0,.051,.051,.023,.045,.022,.49,.51

7,0,.044,.056,.02,.051,.02,.56,.44

7,0,.038,.063,.017,.058,.017,.62,.38

7,0,.031,.07,.014,.065,.014,.69,.31

7,0,.024,.077,.011,.073,.012,.76,.24

7,0,.019,.085,.009,.081,.009,.81,.19

7,0,.014,.092,.007,.089,.007,.86,.14

8,.033,.061,.028,.03,.02,.023,.33,.67

8,.038,.056,.031,.027,.022,.021,.38,.62

8,.043,.052,.035,.024,.025,.019,.43,.57

8,.049,.046,.04,.022,.029,.017,.49,.51

8,.055,.041,.044,.019,.032,.015,.55,.45

8,.061,.036,.049,.016,.036,.013,.61,.39

8,.068,.029,.054,.014,.04,.011,.68,.32

8,.074,.024,.059,.011,.044,.009,.74,.26

8,.08,.018,.065,.009,.048,.007,.8,.2

8,.085,.014,.07,.007,.052,.005,.85,.15

8,.089,.01,.076,.005,.056,.004,.89,.11

9,.061,.033,.03,.028,.023,.02,.67,.33

9,.065,.029,.032,.025,.024,.017,.71,.29

9,.068,.025,.036,.022,.026,.015,.75,.25

9,.072,.021,.039,.02,.028,.013,.79,.21

9,.075,.017,.042,.017,.029,.01,.83,.17

9,.078,.014,.046,.013,.031,.007,.86,.14

9,.081,.011,.05,.011,.033,.006,.89,.11

9,.083,.008,.054,.009,.034,.005,.92,.08

9,.085,.006,.059,.007,.036,.004,.94,.06

9,.086,.005,.063,.006,.037,.003,.95,.05

9,.088,.003,.067,.004,.038,.002,.97,.03

Note: Each raw consists of nine terms as follows;

- case of the slab.

- coefficient of short direction for negative moment.

- coefficient of long direction for negative moment.

- coefficient of short direction for live load positive moment.

- coefficient of long direction for live load positive moment.

- coefficient of short direction for dead load positive moment.

- coefficient of long direction for dead load positive moment.

- ratio of load for short direction.

- ratio of load for long direction.

5- Illustration Example

Design a two-way slab has clear dimensions (8.2m length and 5m width), the superimposed dead load Wds = 2 kN/m2, and live load WL= 5 kN/m2. The slab is supporting to nonstructural elements likely to be damaged by large deflections. = 400MPa,  = 28MPa and diameter of bars (available in situ) db = 12mm. The slab location represents case nine.

5.1 Hand Calculations Solution

a- Compute the ultimate load and effective depths:

Wu = 1.2	(2 + 24.5 t) +1.6  5 thus; Wu = 7.545 + 8 = 15.545 kN/m2

ds = t – cover – db/2 = 175 – 20 – 12/2 = 149 mm

dl = ds – db = 149 – 12 = 137 mm.

b- Determine the coefficients: Case (9), m = 5/8.2= 0.6098 thus from tables 8.3 to 8.6 respectively of reference [Nilson and Winter, 1986], the coefficients are as follows:  = 0.084608,  = 0.006392,  = 0.035608,  = 0.004196, = 0.058020,  = 0.007392 and the ratio of load in short directions = 0.93608 (all these values from interpolation).

c- Compute the moments:

d- Calculate the ratios of reinforcements; by solving the equation of strength [ACI 318M-2008]:

, the solution of this equation for  will be:

, (assume, i.e. assume all ratios smaller than ) thus:

= 0.004267, = 0.002338,= 0.000997 and= 0.000911, now compute which equal to , hence  larger than all ratios, thus  always. The areas of reinforcements will be:

,

,,

Compute the minimum area of reinforcement must be provided for temperature and shrinkage, , hence the  and  must be modified to be equal to .

e- Check shear requirements; the ratio of load in short direction represents the critical and equals to 93.608% as computed above, thus the maximum reaction along the long direction is equal to 15.545 0.93608  5  0.5 = 36.378 kN, thus the shear at the critical section Vud = 36.378 – 15.545 0.93608  0.149 = 34.21 kN,

, which , hence the ACI Code requirements for shear are satisfied.

f- Check the deflection requirements; the deflection due to sustain dead load can be computed as (computed for long and short directions then take the average):

, where; = the positive dead load moment of long or short direction, = the length of long or short direction, Ec = modulus elasticity of concrete and  represent effective moment of inertia of the concrete cross section of unit width and taken here as the gross moment of inertia because the amount of cracking at service load is usually not extensive [Nilson and Winter, 1986],

,

,

= 24870 MPa, /12= 446614583.3, thus

The deflection due to live load can be computed as (computed for long and short directions then take the average):

, = the positive live load moment of long or short direction,

,,

Total deflection DLT= × T +  × 1.5 + 1.47039 = 2.56429mm

The limit of deflection DefL = , [ACI 318M-2008], thus , larger than the total deflection, hence ACI Code requirements for deflection are satisfied.

5.2 Program Calculations Input Data and Results

a- Input the following data:

9, 8.2, 5, 5, 2, 400, 28, 12, 0.175, 2, 2, the results appear as shown below.

Note: each comma meaning press enter key to continue.

b- Output results:

Negative moment of short direction (kN.m/m) = 32.88147

Positive moment of short direction (kN.m/m) = 18.32177

Negative moment of long direction (kN.m/m) = 6.679375

Positive moment of long direction (kN.m/m) = 6.103653

Negative area of reinforcement for short direction (mm^2/m) = 636

Positive area of reinforcement for short direction (mm^2/m) = 348

Negative area of reinforcement for long direction (mm^2/m) = 136

Positive area of reinforcement for long direction (mm^2/m) = 124

Minimum area of reinforcement must be provided (mm^2/m) = 350

Permissible deflection (mm) = 10.41667

Total deflection as computed (mm) = 2.564106

As shown above, there is agreement between the results of program and hand calculations (small differences expected because of the approximate nature of the calculations), and the simplicity of application of the present program (POTWS).

6- Conclusions

Based on the results obtained from several test examples done the following conclusions can be drawn. These may be summarized as follows:

- The results of the present program (POTWS) are accurate, precise and coincide with hand calculation results, therefore; its can be used safely to analysis and design of any rectangular edge-supported two-way slab under uniformly distributed load.

- The program (POTWS) facilitates the engineers and designers job as well as make them gain the time, this fact is very important. The required time to input data is about ten seconds and the running time less than one second.

- The constructed program opens the way to study the effects of material properties, loads and many other design parameters on the values of moments, shear forces and deflection of rectangular edge-supported two-way slabs easily.

- The application of program proved the facilities and efficiency of the QuickBASIC language to construct and running the engineering programs.

Appendix 1: References

ACI-Code (318M-08), 2008, “Building Code Requirement for Structural Concrete”, Provided by IHS under license with ACI.

Amer M.I., Mohammed S.M. and Qussay W.A., 2008,“Design of Reinforced Concrete Structures”, Diyala University, Iraq, First Edition.

Arthur H. Nilson and George Winter, 1986, “Design of Concrete Structures”, McGraw-Hill, New York.

Arthur H. Nilson, David Drawin and Charles W. Dolan, 2004, “Design of Concrete Structures”, McGraw-Hill, New York.

I.C. Syal and A.K. Goel, 2004, “ Reinforced Concrete Structures”, S. Chand and Company Ltd, India.

M. Nadim Hassoun, 1985, “Design of Reinforced Concrete Structures”, PWS Publishers, USA.

Phil M. Ferguson, 1981, “Reinforced Concrete Fundamentals” John Wiley & Sons, New York.

QuickBASIC Help Manual.

V.N. Vazirani and S.P. Chandola, 2004, “Concise Handbook of Civil Engineering”, S. Chand and Company Ltd, India.

Appendix 2- Notation

|  | = factor relating depth of equivalent rectangular compressive stress block to neutral axis depth. |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | = 24.5 kN/m3. |  |  |
|  |  | = net tensile strain in extreme layer of longitudinal tension steel at nominal strength. |  |  |
|  |  | = ratio of negative reinforcement for long direction, (in program ROnl). |  |  |
|  |  | = ratio of negative reinforcement for short direction, (in program ROnw). |  |  |
|  |  | = ratio of positive reinforcement for long direction, (in program ROpl). |  |  |
|  |  | = ratio of positive reinforcement for short direction, (in program ROpw). |  |  |
|  |  | = crushing strain of concrete = 0.003. |  |  |
| a |  | = depth of equivalent rectangular stress block, mm. |  |  |
| ASnl | = negative area of reinforcement required in long direction, mm2/m. |  |  |  |
| ASnw |  | = negative area of reinforcement required in short direction, mm2/m. |  |  |
| ASpl |  | = positive area of reinforcement required in long direction, mm2/m. |  |  |
| ASpw |  | = positive area of reinforcement required in shot direction, mm2/m. |  |  |
| b |  | = width of the section, usually taken (1000 mm) in slabs. |  |  |
| C | = case of the slab. |  |  |  |
| CC | = distance from extreme compression fiber to neutral axis, mm. |  |  |  |
| db |  | = diameter of bars used in situ, mm. |  |  |
| DefL |  | = permissible deflection limit [ACI 318M-2008] (Table 9.5.b), mm. |  |  |
| dl | = effective depth of slab in long direction, mm. |  |  |  |
| DLT | = computed deflection, mm. |  |  |  |
| ds | = effective depth of slab in short direction (mm). |  |  |  |
| Ec | = modulus elasticity of concrete, MPa. |  |  |  |
| fc' |  | = compressive strength of concrete (cylinder), MPa. |  |  |
| FII |  | = . |  |  |
| fy |  | = yield strength of steel, MPa. |  |  |
| ID |  | = factor specify the occupation of slab. |  |  |
| , |  | = effective moment of inertia of the concrete cross section of unit width, mm4. |  |  |
| L and | = length of the slab (clear), m. |  |  |  |
| m | = the resisting moment in the short direction, kN.m/m. |  |  |  |
| Mnl | = negative moment of long direction, N.mm. |  |  |  |
| Mnw |  | = negative moment of short direction, N.mm. |  |  |
| Mpl |  | = positive moment of long direction, N.mm. |  |  |
| Mpw |  | = positive moment of short direction, N.mm. |  |  |
| OUmax | = = . |  |  |  |
| RL |  | = means reaction of long direction (on short span). |  |  |
| Rut |  | = 2.55 * * / (8 *) [ACI 318M-2008]. |  |  |
| RW |  | = means reaction of short direction (on long span). |  |  |
| t |  | = thickness of slab, m. |  |  |
| Vc |  | = nominal shear strength provided by concrete section, kN. |  |  |
| Vul |  | = shear force at the critical section of long direction, kN. |  |  |
| Vuw |  | = shear force at the critical section of short direction, kN. |  |  |
| W or |  | = width of the slab (clear), m. |  |  |
| WD |  | = total dead load, kN/m2 (superimposed plus weight of slab). |  |  |
| Wds |  | = dead load (superimposed), kN/m2. |  |  |
|  |  | = total ultimate dead load, kN/m2. |  |  |
| WL |  | = service live load (superimposed), kN/m2. |  |  |
|  |  | = total ultimate live load, kN/m2. |  |  |
| Wu |  | = total ultimate load on slab, kN/m2. |  |  |

Appendix 3-Cases [[Nilson and Winter, 1986] and [Syal and Goel, 2004]]

1

2

3

4

5

6

7

8

9

Appendix 4-Steps of  Installation and Running the Program

- Open the appendix CD disc and copy the folder of QuickBASIC language, then paste this folder at any drive.

- Construct folder called “Coeffs” on drive “D”.

- Construct folder called “Output” on drive “D”.

- Open the appendix CD disc and copy the file Coeff.Dat, then paste this file in folder Coeffs which construct in step two above.

- Open the appendix CD disc and copy the file of program POTWS.BAS, then paste this folder at any drive.

- Press double on the file POTWS.BAS, to open with QuickBASIC language.

- After opening program, press Run order then press Start to run the program.

- Now input the data required, as printed in the screen, then press enter after each term, the results appear.

- Go to the output folder construct in step three mentioned above to show the results at Out.dat file.