Determination of clioquinol as its Cu(II) complex in pharmaceutical and environmental wastewater samples

Nief Rahman Ahmad

Department of Environmental Technology, College of Environmental University of Mosul

(NJC)

(Received on 18/3/2013)            (Accepted for publication 17/6/2013)

Abstract

To develop spectrophotometric method for the determination of clioquinol in commercial dosage forms and industrial wastewater samples. The method is based on the chelation of the drug with Cu(II) to form red colored metal chelate at room temperature which absorbs at 435 nm. Beers law is obeyed over the concentration range of 2-28 µg/mL with molar absorptivity and Sandell s sensitivity of 6.41x103 L/mol.cm and 0.047µg/cm2 respectively, relative standard deviation (RSD)is less than2.0 (n=10). The method is applied successfully for determination of clioquinol in some pharmaceutical formulations (creams) and industrial  wastewater samples.

Keywords: Clioquinol, Spectrophotometry,   Pharmaceutical    Preparations , Industrial  wastewater

الخلاصة

تم تَطوير طريقةِ طيفيةِ لتقديرِ كليوكوينول في مستحضراته الصيدلانية وعيناتِ من المياه الصناعيةِ المطروحة. تعتمد الطريقةَ على تكوين معقد كليتي بين الدواء وايون النحاس الثنائي لتَشكيل معقد احمر اللَوَّنِ في درجةِ حرارة الغرفةِ والذي له أقصى امتصا ص    عند  435  نانوميتر. حيث ان قانون بير ينطبق على مدى تركيزَ 2-28 مايكروغرام \مل ووجد بان قيمة معامل الامتصاص المولاري ودلالة ساندل للطريقة هما     6.41x 103 لتر\مول. سم و0.047 مايكروغرام\سم 2  على التوالي،ان الانحراف القياسي النسبي للطريقة أقل مِنْ 2.0 % n=10)). تم تطبيق الطريقةَ بنجاح لتقدير كليوكوينول في بَعْض المستحضراتِ الصيدلانيةِ (كريم) وعينات مياه صناعيةِ مطروحة.

Introduction

Clioquinol (5-chloro-7-iodo-8-hydroxyquinoline; CQ;(1) Fig.( 1)

C9H5ClINOıı  305.5

Fig[1]: Chemical Structure of Clioquinol

It was first prepared in Germany in the early part of the last century (2) and has been widely used as an antibiotic for the treatment of diarrhea and skin infection. The metal-binding properties of clioquinol led to its use in a mouse model of Alzheimer's disease in which it was shown to reduce or prevent the formation of amyloid plaques in the brain (3). It was also shown to have efficacy in an animal model of Parkinson's disease (4). . Biochemical analysis revealed that clioquinol induced cancer cell death through apoptotic pathways that require caspase activity. Although clioquinol induced modest inhibition of SOD1 activity in treated cells, comparable inhibition by a known SOD1 inhibitor, diethyldithiocarbamate, did not result in cytotoxicity. (5). Few reports have been described for the determination of clioquinol , these include titrimetric methods(6,7),spectrofluorimetric method(8),spectrophotometric method(9) extractive alkylation and gas-liquid chromatography(10), high-performance liquid chromatography(11-12) and gas chromatography(13) . These methods are not simple for routine analysis and required expensive  or sophisticated  instruments. The purpose of this work was to develop rapid accurate ,procedure for the determination of pure  clioquinol, commercial dosage forms and environmental water samples . The method is based on the reaction of drug with Cu+2ion at pH4 resulting in the formation of yellow  complex which absorbs maximally at 435 nm.

Experimental

Apparatus

A Shimadzu UV- 1700 pharmaspec double beam spectrophotometer  equipped (Japan) with 1.0 cm quartz cells was used for absorption measurements, and Jen way 3310 pH meter was used.

Reagents

All chemical used were of analytical or pharmaceutical grade and clioquinol standard material was provided from  state company of  drug industries and medical appliance (NDI) Nineveh - Iraq .

Clioquinol standard solution(100 ppm)

This solution was prepared by dissolve 0.01 gm of  clioquinol in 100 ml of ethanol(96%)in calibrated flask.

Copper sulfate penta hydrate : 1%

This solution was prepared by dissolve 1 gm of CuSO4.5 H2O in distilled water containing 1 ml of concentrated H2SO4 and diluted to 100ml in calibrated flask.

Buffer solution(pH 4)

This solution was  prepared by mixing 41ml of 0.2 M acetic acid with 9 ml of 0.2M sodium acetate, then the volume is completed to 100 ml with distilled water in a volumetric flask( 14).

General procedure :

Different aliquots of standard solution of  clioquinol equivalent 50-700 µg were transferred into a series of 25ml volumetric flasks, 1ml of buffer solution pH4 , and 3 ml of Copper sulfate solution were added. The content was mixed and let stand for 5min with occasional shaking. The volume was diluted to the mark with distilled water and mixed well. The absorbance of each solution was measured at 435 nm against a reagent blank.

Procedures for pharmaceutical preparations(creams)

The amount of cream equivalent to 10 mg of clioquinol was transferred quantitatively into a conical flask. The drug content of the cream extracted with three succesive30 ml of dimethyl formamide and collected in a 100 ml calibrated flask after filtering  through a whatman no.1 filter paper. The solution was made up to the mark with dimethyl  formamide. Treat  3ml of this solution as mentioned under general procedure.                                                                                               Procedure for wastewater samples

To demonstrate the practical applicability of the proposed method, real water samples were analyzed by this method. Industrial waste water from the state company for drug industries and medical appliances Mosul-Iraq, were fortified with the concentrations in the range of 4,10,16 µg/ml of clioquinol  .The fortified water samples  were analyzed as described above for general  procedure and the concentration  was calculated  by using the calibration curve of this method.

Results and Discussion

Clioquinol was found to react with Cu(II) at room temperature resulting in formation of red colored complex which absorbed at435nm  Fig [2].The various experimental affecting  the development and stability of the reaction product was optimized by changing each variable in turn while keeping all other variables constant.

Fig[2]:Absorption spectrum of  Clioquinol (12 µg/ml) – Cu(II)  complex against reagent blank.

Effect of  pH

The effect  of pH was investigated in the range 2-6.The results indicated that the product remained maximum and constant over the pH range 3.5-4.5 Fig[3]. There for  a 1 ml of pH4 was selected for further study

Fig[3]:Effect of pH

Effect of copper sulfate solution

The amount of copper sulfate solution (1%) for maximal color intensity  was examined the aximum constant intensity was reached at 2 ml of reagent solution  and remained constant up to 5ml.Fig[4] .However 3ml of the reagent solution was selected for the subsequent work.

Fig(4):Effect of the amount of ferric ammonium sulfate solution

Effect of temperature and time:

The results  obtained indicated that complete color formation occurred immediately  and not effected by temperature therefore, room temperature was selected as suitable temperature.  The absorbance remained constant for 6 hours at least, and 5 minutes was selected as a suitable time.

Effect of order of addition

To test the effect of order of the addition of the reagents on the absorbance of the product, different order were tested. The selected order was sample solution, buffer solution pH4 followed by copper sulfate solution which was gave high absorbance value.

Calibration graph

Employing the conditions described in the general procedure a linear calibration graph of clioquinol was obtained  Fig[5], which shows that Beer’s law was obeyed over the concentration range 2-28 µg/mL with correlation  coefficient  of 0.9980, intercept of -0.042 and slope of 0.021. The conditional molar absorptivity of the product formed and sandell s sensitivity were found to be 6.41×103 L/ mol .cm  and 0.047µg/cm2 respectively.

Fig[5]:Calibration curve of clioquinol.

Accuracy and precision

The accuracy and precision of  the method was established  by analyzing the pure drug solution at three different levels. each determination being repeated ten times. The average recovery which is a measure of  accuracy is 100 ± 0.75 revealing high accuracy of  the method. The relative standard deviation (RSD) , which is an indicator of precision is less than 2% . The results are complied in Table[1]

Table [1]: Optical characteristics and statistical data for regression equation of the proposed  method

| Parameters | Value |
| --- | --- |
| λ max (nm) Beer´s law limits (µg .mL-1) Molar absorpitivity (L.mol-1.cm-1) Sandell s Sensitivity (µg/cm2) Correlation coefficient (r2 ) Regression equation (Y= a × + b ) Slope (a) Intercept (b) Recovery % Relative standard deviation (%) | 435 2-28 6.41× 103 0.047 0.9980 0.021 -0.042 100 ± 0.75 < 2.0 |

Stoichiometry of reaction

The stoicheiometry of the reaction  between clioquinol and Cu(II) was investigated using job's method of continuous variation and mole ratio methods of equimolar solution (3.27x10-3M), the result obtained show that 1:2 Cu(II)  to drug Fig[6].

Fig[6] : Mole ratio and continuous variation plots for reaction of Cu(II) with

Clioquinol

The suggested reaction and structure of the product might be written as :

Effect of interferences

The interfering effect of foreign species often accompanied with  clioquinol in the pharmaceutical preparations were studied by adding different amounts of foreign species to 20μg\ mL of clioquinol in solution and the recommended procedure for the determination of clioquinol was followed. The species are considered to interfere seriously if they cause a change of more than 2% in the absorbance obtained for clioquinol a lone (15). It was observed that the Betamethazone 17-valerate, gentamycine sulphate and tolnaftate don't interfere with determination method at levels found in the dosage form cited in table[2] so that the selectivity of method is very good.

Table [2]: determination of clioquinol in presence of excipients.

| Excipients | Amount taken (μg) | Average recovery* % |
| --- | --- | --- |
| Betamethazon 17-valerate | 30 | 100.05 |
| Gentamycine sulphate | 30 | 100.0 |
| Tolnaftate | 30 | 100.08 |

* Average of seven replicate analyses.

Application of the proposed method

The proposed  method was successfully applied to the analysis of clioquinol in creams and industrial waste water samples. The result of analysis for pharmaceutical formulations revels that there is close agreement between the results obtained by the proposed method and the label claim Table[ 3]. The results were also compared statistically by student t-test

and by the variance ratio F-test with those obtained by official method (7)at 95% confidence level. The calculated t- and F- values did not exceed the theoretical values indicating that there was no significant differences between the precision of the proposed and official BP method as cited in Table[3]. And the results of water samples Table [4] show that the recovery values obtained were close to 100%.

Table[3]: Assay of clioquinol in pharmaceutical formulations.

| Pharmaceutical formulation supplied by NDI | Amount of clioquinol* Proposed method | Label claim | Literature method (7) | T value | F value |
| --- | --- | --- | --- | --- | --- |
| Quadrim Cream | 9.98 | 10 | 9.99 | 1.96 | 2.23 |

*Mean of ten determinations.

T values (n=10, at 95% confidence level tabulated value 2.262).

F values (n1-1 and n2-1 =9, at 95% confidence tabulated value 3.18).

Table[4]: Determination of clioquinol in spiked industrial wastewater sample.

| Recovery% | clioquinol (µg/ml) * | Water samples |
| --- | --- | --- |
| 100 99.5 99.37 | .044.0 10 9.95 16 15.9 | Industrial wastewater |

*Mean of ten determinations

Conclusion

In this work, a simple, rapid, precise and accurate spectrophotometric method was developed and validated for the determination of clioquinol in pharmaceutical preparations and industrial waste water samples. The method free from such experimental variables as heating or solvent extraction step. The method rely on the use of simple and cheap chemicals and techniques and can be used for rapid routine determination and quality control of clioquinol pure form, bulk sample ,pharmaceutical preparations and real industrial waste water sample.

Acknowledgments

The author wishes to express gratitude to his former company[ the state company of  drug industries and medical appliance (NDI)  Nineveh – Iraq  for providing gift samples of clioquinol standard  materials and pharmaceutical preparations(Quadrim cream).

References

1-The pharmaceutical codex, 11Edn,the pharmaceutical press.London.1979,p .200.

2-  Hollingshead R. The dihalogen and trihalogen derivatives. In: Oxine and its derivatives, vol. III. London: Butter worths Scientific Publications; 1956. p. 740.

3- Cherny RA, Atwood CS, Xilinas ME, et al. Treatment with a copper-zinc chelator markedly and rapidly inhibits β-amyloid accumulation in Alzheimer's disease transgenic mice. Neuron 2001, 30, 665–76.

4-Kaur D, Yantiri F, Rajagopalan S, et al. Genetic or pharmacological iron chelation prevents MPTP-induced neurotoxicity in vivo: a novel therapy for Parkinson's disease. Neuron ,2003, 37, 899–909.

5-Kenyon G Daniel, Di Chen, Shirley    Orlu, Qiuzhi Cindy Cui1, Fred R          Miller and Q Ping Dou,Clioquinol        and pyrrolidine dithiocarbamate           complex with copper to form                 proteasome inhibitors and apoptosis     iducers in human breast cancer cells,    Breast Cancer Research                       ,2005,7(6),897-908.

6-Sobhi.A"A simple method for the        determination of 5,7-                             Diiodoquinoline -8-ol and 5-chloro7-    iodoquinolin-8-ol(clioquinol)                "Analyst,1975,100,696-702.

7- British pharmacopoeia ,H.M .            Stationery office ,London ,UK,             (2011), 539.

8-Glmenez.J,Guiteras.J,Izqulerde.A         and Prat.D "Spectrofluorimetric           determination of clioquinol in              pharmaceutical preparations                 "Freseius J Anal Chem,1991 ,341,638-640.

9-Saha.U,San.A, and Das.T                     "Spectrophotometric determination       of clioquinol and diiodoquin in             pharmaceutical preparations using        uranyl acetate as a chromogenic           agent"Analyst,1988,113,1653-1655.

10-Degen.PH,Find all citations by this author (default).

Or filter your current search

Schneider .W ,  Find all citations by this author (default).

Or filter your current search

Vuillard. P, Find all citations by this author (default).

Or filter your current search

Geiger .UP, and Find all citations by this author (default).

Or filter your current search

Riess W"The determination of clioquinol in biological materials by extractive alkylation and gas-liquid chromatography," Journal of Chromatography , 1976, 117 (2) , 407-413.

11-G.P. Bondiolotti a, C. Pollera b, R. Pirola a, S.R. Bareggi" Determination of 5- chloro- 7- iodo- 8- quinolinol (clioquinol) in plasma and tissues of hamsters by high-performance liquid chromatography and electrochemical

detection,   Journal  of Chromatography  B, 2006, 837, 87–91.

## 12-Zhang,Xiru.G, and Jlang.J" Determination of Clioquinol and Its Related Substances by HPLC,"Chinese Journal of Modern Applied Pharmacy ,2009,10. 844-846.

13- The United State Pharmacopeia Convection,Inc,33-NF ,28, (2010) , 2555.

14-Perrin.D.D,and Demposey. B ;Buffers for pH and Metal ion control,;Champman and Hall Ltd, London,1974,pp.128-153.

15- Hung S. Ch., Qu C. L. and Wu S. Sh.,"Spectrophotometric Determination of Uranium with 2-(3,5) dibromo -2- pyridylazo -5- diethylaminophenol in the presence of a nionic surfactant.Talanta, 1982, 29, 629-631.