Experimental comparison of CO emissions emitted from single cylinder S. I. Engine fueled with different kinds of hydrocarbon fuels and hydrogen

Miqdqam Tariq Chaichan                Khalil Ibrahim Abaas

Mechanical Engineering Dep., University of Technology, Baghdad, Iraq

ABSTRACT:

Liquefied petroleum gas (LPG), Natural gas (NG) and hydrogen were used to operate spark ignition internal combustion engine Ricardo E6, and compare CO emissions emitted from the engine, with emissions emitted from engine fueled with gasoline as a fuel.

The study was done when engine operated at higher useful compression ratio (HUCR) for gasoline 8:1, compared with its operation at HUCR for each fuel. Compression ratio, equivalence ratio and spark timing at constant speed 1500 rpm were studied there effects on the emitted emissions.

CO concentrations were little at lean ratios; it appeared to be effected a little with equivalence ratio in this side, at rich side its values became higher, and it appeared to be effected by equivalence ratio highly. The results appeared that CO emissions resulted from gasoline engine higher than that resulted from using LPG and NG all the time, while hydrogen engine emitted extremely low CO concentrations.

KEY WORDS: LPG, NG, equivalence ratio, compression ratio, optimum spark timing, brake power, CO concentrations.

مقارنة عملية لملوثات CO الناتجة من محرك اشتعال بالشرارة أحادي الاسطوانة يعمل بأنواع مختلفة ن الوقود الهيدروكاربوني والهيدروجين

مقدام طارق جيجان         خليل ابراهيم عباس

العراق -  بغداد – الجامعة التكنولوجية – قسم هندسة المكائن والمعدات

الخلاصة

استخدم الغاز النفطي المسال، الغاز الطبيعي والهيدروجين لتشغيل محرك احتراق داخلي يعمل بالشرارة نوع    Ricardo E6 ، ومقارنة ملوثات CO المنبعثة من المحرك مع مثيلاتها عند عمل المحرك بالجازولين.

تمت دراسة الملوثات الناتجة عند عمل المحرك بنسبة الانضغاط النافعة العليا للجازولين 8:1، ومقارنتها بعمله بنسبة الانضغاط النافعة العليا لكل وقود على حدة، ودراسة تأثير نسبة الانضغاط والنسبة المكافئة وتوقيت الشرر عند سرعة ثابتة 1500 دورة/دقيقة على تراكيز الملوث الناتجة.

أظهرت النتائج أن تراكيز CO تكون قليلة في الجانب الضعيف وتتأثر قليلا بالنسبة المكافئة، وتزداد في الجانب الغني، وتتأثر بشكل كبير بالنسبة المكافئة، وأظهرت النتائج أن ملوثات CO الناتجة باستخدام الجازولين أكبر من تلك الناتجة عن استخدام LPG  و NG على الدوام، بينما ينتج من استخدام الهيدروجين كوقود تراكيز CO قليلة جدا.