EFFECT OF ADDITIONAL BOUNDARY CONDITIONS ON THE AXISYMMETRIC FREE VIBRATIONS OF OBLATE SPHEOIDAL SHELLS

By

- Ahmed A. Al – Rajihy  ;  Ala M. Hussein Al – Jessany

- Nawal H. Al – Raheimy

- College of Engineering, Babylon University

- ABSTRACT

This research deals with the effect of using  additional boundary conditions as well as the eccentricity ratio on the axisymmetric free vibrational characteristics of thin isotropic oblate spheroid shells. The formulation depends on the boundary matching method using                                   the non – shallow shell theory. The oblate shell was assumed to be constructed from two  spherical elements matched along the continuous boundaries. The additional boundary conditions under construction were clamped – clamped, clamped – free and pined – pined. It was found that the natural frequency of the shell with clamped – clamped additional boundaries was greater than other two types of the additional boundaries. On the other hand it was found that in the case of clamped – clamped boundary condition, the eccentricity ratio has an effect on the natural frequency greater than the other two cases, i.e, clamped – free and pined - pined cases,  respectively.

# Key words: Vibration, Oblate shells, Spheroidal Oblate, Axisymmetric Oblate

- تأثير الشروط الحدودية إلا ضافية على الترددات الطبيعية المتناضرة المحور  للقشريات  البيضوية المفلطحة

الخلاصة

تناول هذا البحث دراسة تأثير إضافة شروط حدودية إضافة إلى نسبة الحيود على الخصائص  الاهتزازية المتناظرة المحور  للقشريات البيضوية المفلطحة ذات الخصائص المتماثلة. لقد اعتمدت طريقة تماثل الحدود  في عملية التمثيل الرياضي للمسالة وذلك باستخدام   نظرية القشريات العميقة . وقد تم افتراض ان القشرية مكونة من جزئين كرويين مثبتين على طول الحدود بينهما. إن الشروط ألحدوديه الاضافية  هي: تثبيت محكم – تثبيت محكم , تثبيت محكم  –حر , تثبيت  مفصلي - تثبيت مفصلي . لقد وجد ان الشرط الحدي نوع تثبيت محكم _ تثبيت محكم يزيد من قيمة التردد الطبيعي وان تاثير هذا النوع من الشروط الحدية على التردد الطبيعي اكبر من تاثير باقي النوعين الاخرين من الشروط الحدية. اضافة الى ذلك فان تاثير نسبة الحيود على التردد الطبيعي في حالة استخدام الشروط الحدية نوع  تثبيت محكم – تثبيت محكم  اكثر من تاثير هذه النسبة في حالة استخدام النوعين الاخرين من الشروط الحدية.