| List of symbols |  |
| --- | --- |
| VBBB | Bias voltage |
| VBRefB | Reference voltage |
| PBoutB | Output power |
| PBinB | Input power |
| Vπ | Bias voltage |
| EBoutB | Output electric field |
| EBinB | Input electric field |
| λ | Free space wavelength of the laser |
| ABOB | Area of crystal arm |

- INTRODUCTION :-

Electro optic effect is an effect when an electric field is applied to a crystal, the ionic constituents move to new locations determined by the field strength and the restoring force. A field applied to an anisotropic electro optic material modifies its refractive indices and thereby its effect on polarized light as shown in Fig. (1). Anisotropy in the optical properties therefore can be due to the unequal restoring force along three mutually perpendicular axes in the crystal. These changes can be described in terms of the modification of the index ellipsoid. The linear electrooptic effect or pockel effect is the change in the indices of the ordinary and extraordinary rays is proportional to an applied electric field. This effect exists only in crystals that do not possess inversion symmetry [E.Foutekouv 2005,Jose Antonio Ibarra Fuste2013]. There is a very large variety of useful electro optic materials, covering a wide range values for the electro optic tensors, refractive indices, response time and etc. However, most commercial devices use crystals for example, Potassium Dihydrogen Phosphate (KH2PO4) also known as (KDP), Barium Tantalate (BaTiO3), Lithium Tantalate (LiTaO3), Lithium Niobate (LiNbO3) and some liquid crystals [L.Thylen 1998, M. Suzuki 2011]. .In this research electro optic effect of a particular crystal, Lithium Niobate will be studied.

The phase shift induced by the electro-optic modulator is analyzed by anoptical interferometer. The interferometer is an optical device which utilizes the effect of two beam interference; hence it is a sensitive tool to measure any phase shift between the two beams. By monitoring the intensity of the interference, the half wave voltage can be determined since the phase shift from 0 to π, the constructive interference undergoes to destructive interference or vice versa. The phase change, in the case, is due to the voltage applied to each arm [I. P. Kaminow 1966, J. Cartledge 2009] .The scheme is presented in Fig. (2a and2 b). In electro-opticmodulator(EOM), the incoming light is split into two waveguide under the influence of conducting electrodes. The electro – optic effect induces a change in the refractive index of eachinterferometer arm andphase– modulates the light propagating into that arm according to the electric voltage applied to each electrode . Mathematically [G. Pecere 2011]:

Where again, (α) is the interferometry splitting ratio, 1/2 in the ideal case.The photo detected power (PBdB) will be, with the substitution    and

This device has been reported in the literature as an electro-optic modulator(EOM) for high digital bit-rate and RF transmission over optical fiber communication systems. The main operation of electro-optic modulators (EOM) is based on the linear electro-optical effect (pockels effect) where the refractive index of a medium is modified on proportion to the strength of the applied electric field. The electro-opticmodulator divided into two types as follows [G. Pecere 2011,Jose Antonio Ibarra Fuste2013 ].

a. SymmetricConfiguration

In this section the modulating signal and the bias voltage are applied to only one of the interferometry branches, either to the same or to different branches.In this type using the above condition in Eq. (2), the transfer function (3)is as shown in Fig. (3).

b.Push-PullConfiguration

This configuration is obtained by anapplying data and bias voltage in one arm (VB1B) and inverted bias voltage in the other arm (VB2B), i.e.

This increases the relative phase shift in one arm and decreases it in the other arm .Since phase changes are equal in magnitude but opposite in sign in each arm a chirp free intensity modulation is obtained, as shown in Fig.(4).Following from Eq.(2), using α = 1 and the condition in the Eq.(4), the transfer function is shown in Fig.(4).

2.  CONDITION FOR HIGH VISIBILITY FACTOR INTERFERENCE

Superposition principle is a basic property of all kinds of waves and is a key element to understand the condition for interference. When two waves are superimposed, the resulting amplitude distribution is the addition of the instantaneous amplitude of these two waves.

Consider an incident light beam with an electric field (E) which oscillates at a frequency (ωBtB) and has travelled the path(r ) [N. Tien 2007, S.K. Saha, 2011].

Where (EBO)B is the amplitude and (k) is the wave number equals to 2π/λ. If the two beams from path A and B interfere with each other, the intensity is

Constructive interference corresponds to the cosine term equals to (1) whereas destructive interference is equals to (-1). Hence,

The visibility factor is defined as follows [1]:

A full visibility factor of (1) happens when Imin = 0. In general, the following conditions must be met for two waves to be interfered:

(a) Same polarization. Two waves with orthogonal polarization cannot interfere witheachother.

(b) Same frequency. The two waves must oscillate at the same frequency to cancel thefrequency dependent term in Eq. (6) as derived.

(c)Constant phase relationship. The phase difference must be constant at any given point in the superposition region. Otherwise, no stable interference pattern can be observed.

3. MATHMATICAL MODULE

If an electric field is applied during a certain length to one arm, a phase difference Df between the light propagation through the two waveguides will be introduced see Figs.(5aand5b).This result an intensity modulation at the electro-optic modulator output, which can be written as equation (8) [J. S. Wilkinson 1998]:

If a laser diode source is used, the visibility factor () is:

Where: = The intensity of the interfered light wave,  = The intensity of the initial light wave,  IB2 = BThe intensity  of the initial light wave , f = visibility factor , EB1 B= The electric field of  first arm for EOM (V), and EB2 B=  The electric field of  second arm for EOM (V)

Substituting Eq.(10), into Eq. (11) is result:

The mathematical expression of visibility factor for electro-optic modulator (EOM) is shown in Eq.(12).

Equation (12) explains the relationship between thevisibility factorand the phase difference.  The phase difference  is related by equation (13),  [D. D. Johnson 2010].

Where:,,  K= wave number, Free space wavelength of the laser in ( nm) L= The length of the crystal arm in the modulator in ( μm), n= Refractive index (unit less),  Temperature difference in (k), (unit less), 𝓔BrB= radial strain (unit less), and  = pocket coefficient.The final mathematical expression of phase difference with respect radial strainforelectro-optic modulator (EOM) is shown in Eq. (14).

=

Where: F = Force (Newton), G=Young's module (100GPa), d = thediameter of thecrystal arm(62.5 µm), and = the change in diameterof the arm.This change results from radial strain in (µm).Addition Eq. (14) ,into  for  the Eq. (12)  is  result

The equation (17) is the fundamental equation for measurement the efficiencyof visibility factor whichconsiders the necessary factor in work the optical modulatorin the optical communication systems .Also; this equation shows the relationship between thevisibilityfactor , and theradial strain.

4. SIMULTION AND RESULTS

If a force in the form tensile stress is applied on an arm of the EOM, the radial strain will be produce as shown in   Fig. (6), and table (1) .    It can be noticed from the table below that the radial strainand the change of the diameter (have gradually increased from        (-5.5 *10^-2) and (3.45µm) until it reaches its maximum value -1 and 62.5 µm respectively.

The mark of minus sign for the radial strain due to the tensile stress which itapplies on an  arm of crystal for EOM. Also, when reaches its maximum value 62.5 µm, then abranch of EOM will be an inactive.Meaning that, the interference signal of EOM is removedand this it results from adamaged the arm of EOM.From all of this,it can be suggested a mathematical model and an equation (16), as shown in a previous section, to analysis these results and the damaged can be remove by using this technique, as shown in these Figs (7, 8, and 9).

In theFig. (7), when the effect of radial strain is below 0.2 (i.e. The effect of radial strain neglects), theEOM operates with ahigh of efficiencyand this leads to constructive interference .Otherwise, if a radial strain increases until reaching to its value between 0.5 and 0.6, and above of these values (i.e., 0.8, 1, etc) the visibility factor (f) of the EOM is equal 0%, and this means an interference signal removesand this results from destructive interference.

Also, from Fig. (8), the performance of EOM can be improved by reducing the change of diameter (∆d< 10µm), which it results from effect of a radial strain. Therefore, the change of diameter (∆d) is limited by decreasing the radial strain.Thus, the Fig.(9), gives details of view on an effecting the load of force on the factor which measures a performance of EOM, and this means the radial strain is limited by decreasing the load of force (F < 200 N) . Finally, the performance of EOM can be enhanced by controlling on the effect a radial strain and this achieves by using the mathematical model.

5. CONCLUSIONS

In this research, we introduced a mathematical model is an effective way to improve the electro-optic modulator sensitivity when the visibility factor ( f ) is relatively high and it can improve EOM performance by reducing an effect of the radial strain. We discussed the impact of increasing radial strain effect on the performance of EOM therefore it can be concluded when the radial strain increasespending reaching to its value (-1), this leads to decreasing a performance of EOM and thus it results a damage the armof EOM.We concluded the performance of EOM can be improved by decreasing the change of the diameter (∆d), (i.e. ∆d ˂ 10µm), which results from increasing the radial strain therefore it is limited by controlling on the radial strain. Finally, this presented a mathematical model it has ability to suppress the effect of radial strain by evaluating its effects (i.e. F< 200 N).

Table 1

| Force (N) | tensile stress (σBaxB=F/ABOB)(Mpa) | axial strain (𝓔BZB = σBaxB / G) | radial strain(𝓔BrB) | (µm) |
| --- | --- | --- | --- | --- |
| 100 | 3.25 * 10^4 | 0.325 | - 5.52 * 10^-2 | 3.45 |
| 200 | 6.5189 * 10^4 | 0.65189 | - 1.108 * 10^-1 | 6.925 |
| 300 | 9.778 * 10^4 | 0.9778 | - 1.662 * 10^-1 | 10.38 |
| 400 | 1.3037 * 10^5 | 1.3037 | - 2.216 * 10^-1 | 13.85 |
| 500 | 1.6297 * 10^5 | 1.6297 | - 2.770 * 10^-1 | 17.3 |
| 600 | 1.9556 * 10^5 | 1.9556 | -3.323 * 10^-1 | 20.7 |
| 700 | 2.2816 * 10^5 | 2.2816 | - 3.877 * 10^-1 | 24.23 |
| 800 | 2.6075 * 10^5 | 2.6075 | - 4.432 * 10^-1 | 27.7 |
| 900 | 2.933 * 10^5 | 2.933 | - 4.981 * 10^-1 | 31.13 |
| 1000 | 3.259 * 10^5 | 3.259 | - 5.540 * 10^-1 | 34.6 |
| 1200 | 3.911 *10^5 | 3.911 | - 6.648 * 10^-1 | 41.5 |
| 1400 | 4.563 * 10^5 | 4.563 | - 7.757 * 10^-1 | 48.48 |
| 1600 | 5.215 * 10^5 | 5.215 | - 8.865 * 10^-1 | 55.4 |
| 1850 | 6.030 * 10^5 | 6.030 | - 1 | 62.5 |

Fig.1. an electric field applied to an electro opticmaterial modifies its refractive indices.

Fig .2b. The electrodes are placed in both ends of crystal

Light input

Fig .2a. Electro-optic modulator with two voltages on the deferent arms

VBRFB

Light output

VBBB

Light input

Light output

VBRFB

Fig .2a. Electro-optic modulator with two voltages on the deferent arms

Fig .2b. The electrodes are placed in both ends of crystal

PBout B/PBinB

Fig. 3. Transfer function of EOM in the symmetric

Configuration   [G. Pecere 2011]

VBBias

Vπ

Vπ / 2

Quadrature point

1

Fig. 4. Transfer function of EOM in thePush-Pull

Configuration[G. Pecere 2011]

VBBias

PBout /BPB in BPBinB PBin PBinB                                                                     B/PBinB

Vπ

Vπ / 2

Quadrature point

1

VBRFB

VBBB

Light output

Light input

Fig .5a. EOM with two voltages on the one arm

EOM

Radial StrainSensor

Fig.5b.Illustration visibility factor of EOM with radial strain sensor

Laserdiode

Optical fiber

Optical connector

Electrical signal

Force

Force

Radial strain

Fig.6.Illustration applying force on an arm of  the EOM

(µm)R R

Fig. (8). Illustration the effect ofchange of          diameter (∆d) on the performanceof EOM

Fig. 7. Illustration the effect of radial strain on the performanceof EOM

𝓔BrB (µ-Epsilon)

Fig. 9. Illustration the effect offorce on the performanceof EOM

REFERENCES :-

D.D. Johnson " MatSE 280: Introduction to Engineering Materials ",2010.

E. Foutekouv "Mach–Zehnder based optical modulator    "Intel Technology Journal, 2005.

G. Pecere "Spectral Amplitude and Phase Characterization of Optical Devices by RF scan''Master of Science in Telecommunication Engineering, Third School of Engineering: Information Technology, Politecnico diTorino, 2011.

I. P. Kaminow, E. H. Turner, Applied Optics 5, 1612 (1966).

J. Cartledge et al “Electronic Signal Processing for Fiber-Optic Communications” IEEE 2009.

J.S.Wilkinson "Integrated Optical Mach–ZehnderModulator",J.Light Wave technology,1998.

Jose Antonio Ibarra Fuste " Design Rules and Optimization of Electro-Optic Modulators based onCoplanar Waveguides"Doctor of Philosophy Dissertation, Universitat Politecnica De Catalunya Barcelona tech, Departament de Teoria del enyali Comunicacions, 2013.

L. Thylen, J. Light wave Technol., LT-6:847 (1988).

M. Suzuki, Tomokazu Takahashi, Seiji Aoyagi, Yoshiteru Amemiya, Masataka Fukuyama, and   Shin Yokoyama "  A Study of Mach–Zehnder Interferometer Type Optical Modulator Applicable to an Accelerometer "Japanese Journal of Applied Physics 50 (2011)

N.TienTjuen, "A Fast Polarization Independent Phase Shifter Of  Light''  (A thesis submitted to the Department of Physics in partial fulfillment of the requirements for the degree of Bachelor of Science (Honours), Semester I & II 2006/2007.

S.K. Saha, Aperture Synthesis, Astronomy and Astrophysics Library, Chapter 2 "Principles of Interference", DOI 10.1007/978-1-4419-5710-8 2, © SpringerScience Business media, LLC 2011.