Experimental study for staggered perforated array of pins like fins in a rectangular air cross flow

Ali Shakir Baqira, Ahmed Qasimb, Anmar Adnanb

a Najaf Technical College, Najaf, Iraq, baqireng@yahoo.com,  b Baghdad Technical College, Iraq, Dept. of Refrigeration and Air conditioning Eng.

ABSTRACT

This paper investigated experimentally the friction factor reduction and heat transfer enhancement processes over staggered perforated pin fins in a rectangular channel with air cross flow. The channel had a duct cross section width of 62 mm, cross section depth of 167 mm and duct length of 1200 mm. The experiment covers the following ranges: Reynolds number 28000-113000, and pin fin shapes (solid, horizontal/vertical (HV) perforation and horizontal/vertical/lateral (HLV) perforation). Results show that the Nusselt numbers of pins with horizontal/vertical (HV) perforations are about 11% higher than those for solid pins and with horizontal /vertical/lateral (HLV) perforations are about 21% higher than those for the solid pins.. In addition, experimental results show that pins with horizontal/vertical/lateral (HLV) perforations, have good enhancement of heat transfer in addition to a significant reduction in weight compared with solid pin case performance.

Keywords: Perforated pin fins, Heat sink, Heat transfer enhancement, Thermal efficiency, Pressure drop, fin performance and thermal resistance.

دراسة عملية لمجموعة متداخلة من قضبان مثقبة تشبه الزعانف في قناة مستطيلة المقطع يتدفق عبرها الهواء بشكل عمودي

الخلاصة:

يتقصى هذا البحث تجريبيا اجراءات تقليل معامل الاحتكاك وتعزيز انتقال الحرارة لمجموعة قضبان متداخلة مثقبة تشبه الزعانف في قناة مستطيلة المقطع يتدفق عبرها الهواء بشكل عمودي. عرض المقطع العرضي للقناة 62 ملم, عمق المقطع العرضي 167 ملم وطول المجرى الهوائي 1200 ملم. غطت التجربة النطاق التالي: عدد رينولدز 28000-113000 , , شكل القضبان التي تشبه الزعانف المستخدمة في الجانب العملي هي قضبان صلبة تشبه الزعانف , قضبان تشبه الزعانف ذات ثقوب أفقية/ وعمودية وقضبان تشبه الزعانف ذات ثقوب افقية / جانبية/ عمودية. خمنت النتائج على ان عدد نسلت للقضبان التي تشبه الزعانف ذات الثقوب الافقية/ العمودية بحوالي 11% أعلى منه في القضبان الصلبة التي تشبه الزعانف وللقضبان التي تشبه الزعانف ذات الثقوب الافقية/ الجانبية /العمودية بحولي 21% اعلى منه في القضبان الصلبة التي تشبه الزعانف. أظهرت النتائج التجريبية بأن القضبان التي تشبه الزعانف ذات ثقوب افقية / جانبية/ عمودية لديها تعزيز نقل الحرارة و كفاءة حرارية وفعالية للزعانف جيدة. علاوة على ذلك فهي تسبب انخفاض مؤثر في الوزن مقارنة بالقضبان الصلبة التي تشبه الزعانف.