PREPARATION & CHARACTERIZATION OF STEP-WISE

AL-AL2O3 FUNCTIONALLY GRADED MATERIALS

Dr. Eng. Alaa Abdulhasan Atiyah** , Dr. KADHIM. H. GHLAIM* & Eng. Noor S. Ghafil*

*Dijllah University College

**University of Technology

ABSTRACT :-

In this work, simple ceramic-metal functionally graded materials with three different layers has been prepared and tested. Ceramic and metal materials were represented by alumina and aluminum respectively. Powder technology practices were used extensively in preparing each of four set of FGMs samples. All samples have the same chemical composition. Two variables were studied, compacting load and sintering temperature. All samples were imparting graded hardness reading across the thickness. The values of hardness readings are increased according to the hardest constituent’s increment (i.e. Alumina) at constant sintering temperature and compacting load. Compaction load increment as well as sintering temperature imparts another reasons for improvement in hardness properties. Microscopic observations show an interesting features of the layers microstructure where composed from an aluminum and alumina particles and show good adhesive between alumina particles and Aluminum while Aluminum surrounded alumina particles from all studies due to good mixing practice during preparation stage.

الخلاصة :

تم في هذا العمل  تحضير واختبار مواد المتدرجه بالخواص مكونه من ثلاث طبقات من سيراميك – معدن. السيراميك والمعدن متمثل بالالومينا والالمنيوم على التوالي. طريقه تكنولوجيا المساحيق استخدمت خصيصاً في تحضير اربع مجاميع من عينات المواد المتدرجه بالخواص، كل العينات تمتلك نفس التركيب الكيمياوي. وتم دراسه متغيرين ، حمل الكبس و درجه حراره التلبيد. ازدادت قراءات قيم الصلاده وفقا لزياده المكون الاصلد (الالومينا) عند ثبوت درجه حراره تلبيد و حمل الكبس. تلعب زياده حمل الكبس ودرجه حراره التلبيد كسبب اخر في تحسين خواص الصلاده. اظهرت الملاحظات التركيبيه خواص مميزه للطبقات والمتكونه من الالمنيوم ودقائق الالومينا واظهرت التصاقا جيدا بين دقائق الالومينا ومعدن الالمنيوم بينما احاط الالمنيوم دقائق الالومينا في جميع الحالات نتيجة للخلط الجيد اثناء عملية التحضير.

Keywords: - Al /Al2O3, functionally graded materials, step-wise, powder technology, metal-ceramic.