THERMAL ANALYSIS OF METAL FOAM HEAT SINK SUBJECTED TO IMPINGEMENT JET COOLING

Prof. Karima E. Amori                          Deyhaa A. Khalaf

Mechanical Eng. Dept.

Baghdad University

drkarimaa@yahoo.com                 Daliqabi@yahoo.com

ABSTRACT :

Jet impingement cooling is critical in many industrial applications (such as manufacturing process, gas turbine blades, etc), since it yields high local or averaged heat transfer coefficient, so a large amount of heat is extracted from hot surface. In this work the thermal characteristic of confined impinging air jet on a hot plate is evaluated experimentally and computationally. The parameters studied were , jet Reynolds number (ranged from 14000 to 70000), angle of impingement jet (0o to 50o measured from vertical axis on the hot plate), and the heat flux subjected on the hot plate (ranged from 1500 to 6000 W/m2). The two – dimensional continuity, Navier-Stockes, energy equations with (k-ε) turbulence model have been solved using finite volume method. The effect of covering the hot plate by a layer of Aluminum metal foam have on the impinging jet cooling of this plate is investigated experimentally for the same studied parameters. The experimental and computational results show that increasing the jet angle decreases the Nussetl no. , while increasing Re no. increases Nusselt no. Utilizing a layer of metal foam on the hot plate enhances the heat transfer by jet cooling by 33 % compared with jet cooling of bare hot plate under the same conditions. A comparison of the present results with the available reported data shows good agreement.

Keywords: jet impingement, metal foam, porous heat sink

تحليل حراري لرغوة معدنية تعمل كمصب حراري معرض للتبريد بالنفث التصادمي

أ.د. كريمة اسماعيل عموري                                  ضياء عبد الرحيم خلف

قسم الهندسة الميكانيكية/جامعة بغداد

يعتبر التبريد بالنفث التصادمي مهما في التطبيقات الصناعية  (كعمليات التصنيع، تبريد ريش التوربين وغيرها) لما يمتلكه من معامل انتقال حرارة عالي لذا يمكن سحب كمية كبيرة من الفيض الحراري من السطوح الساخنة. الهدف من هذا العمل  هو تقييم عملي وعددي للمواصفات الحرارية لنفث تصادمي محجوز يضرب صفيحة معدنية ساخنة. المنغيرات التي تم دراستها هي عدد رينولدز  بمدى (من14000 الى 70000)، زاوية النفث الصادم(0 الى 50ْ مقاسة من المحور العمودي على الصفيحة الساخنة)، وكذلك الفيض الحراري المسلط على الصفيحة الساخنة ( امتد من 1500 ال 6000 واط/م2). حلت معادلة الاستمرارية ، نافير ستوكس ومعادلة الطاقة  مع نموذج الاضطراب (k-ε) لحالة ثنائية البعد عدديا باسنخدام طريقة الحجوم المحددة. تم دراسة تأثير تغطية الصفيحة بطبقة من الرغوة المعدنية من مادة الألمنيوم عمليا على التبريد بالتفث التصادمي ولنفس المتغيرات اعلاه. بينت النتائج العملية والنظرية ان زيادة زاوية النفث قللت قيمة (h)، بينما زيادة عدد رينولدز ادى الى زيادة معامل انتقال الحرارة (h). بينت النتائج العملية تحسن واضح ( بمقدار 33%) لعدد نسلت عند استخدام طبقة من الرغوة المعدنية على الصفيحة الساخنة  عن الحالة بدون استخدام لهذه الطبقة. تم مقارنة النتائج  العملية الحالية مع نتائج سابقة واظهرت توافقا جيدا.