FORCASTING ANALYSIS OF TOTAL DISSOLVED SOLIDS AND CHLORIDE CONCENTRATIONS IN EUPHRATES RIVER IN BABYLON PROVINCE-HILLA CITY

Dr.Nassrin J. AL-Mansori

Nassrin20052001@yahoo.com

Environmental Engineering Department/ Collage of Engineering/University of Babylon

ABSTRACT

The monthly time series of the Total Dissolved Solids (TDS) and Chloride(CL) concentrations in Euphrates River at Babylon were analyzed as a time series.

The data used for the analysis was the monthly series during (2000-2013). The series was tested for non-homogeneity and found to be nonhomogeneous. A significant positive jump was observed at 2002 for Total Dissolved Solids (TDS) and at 2006 for Chloride (CL). This non homogeneity was removed using a method suggested by Yevichevich (7). The homogeneous series was then normalized using Box and Cox (2) transformation. The periodic component of the series was fitted using harmonic analyses, and removed from the series to obtain the dependent stochastic component. This component was then modeled using first order autoregressive model (Markovian chain). The above analysis was conducted using the data for the period (2001-2011), the remaining 2-years (2012-2013) of the observed data was left for the verification of the model. The observed model was used to generate future series. Those series were compared with the observed series using t-test. The comparison indicates the capability of the model to produce acceptable future data.

Key words: Total dissolved solids, Chloride, nonhomogeneous, periodic component, and dependent stochastic

. التنبؤ بالبيانات الشهرية لتراكيز المواد المذابة الكلية وتراكيز الكلور لنهر الفرات في مدينة بابل- الحلة

د.م. نسرين جاسم حسين المنصوري

قسم هندسة البيئة /كلية الهندسة / جامعة بابل

الخلاصـــــــة:

تم في هذا البحث تحليل البيانات الشهرية  لتركيز المواد المذابة الكلية وتراكيز الكلور لنهر الفرات في محافظة بابل (مدينة  الحلة) , حيث تم اخذ المعدل الشهري للبيانات المتوفرة من سنة ( 2000-2013).(دائرة بيئة بابل، بيانات).

في البداية تم اختبار البيانات لمعرفة فيما اذا كانت متجانسة ام غير متجانسة ووجد انها غير متجانسة عند سنة 2002 للمواد المذابة الكلية وعند سنة 2006 لتركيز الكلوريد باستخدام طريقة Yevichevich  تم ازالة عدم التجانس في البيانات . ثم تم توزيع البيانات توزيعا طبيعيا باستخدام طريقة العالمين Cox,Box .